---------------------------------------------------------------------------

CyReL SUNRISE M16-1280 M16 CONFIGURATION ACCESSORY Documentation

Version: 1.0b                                         Date: 6/93

---------------------------------------------------------------------------

CONTENTS

A.   OVERVIEW

B.   MAIN MENU

C.   SUB MENU

D.   NOTES

---------------------------------------------------------------------------

[A] OVERVIEW

---------------------------------------------------------------------------

The CyReL M16_CONF.ACC or M16_CONF.PRG is a small utility that allows the

user to configure certain aspects of the CyReL VDI or parts of the

operating mode of the graphics card in general.

Use this utility to configure the CyReL VT52 Emulator, the CyReL XBIOS

Emulator, the TT COLOR Emulator and the CyReL LINE-A Emulator.

These emulators are all embedded in the M16VDIxx.PRG drivers in your

AUTO folder.

---------------------------------------------------------------------------

[B] MAIN MENU

---------------------------------------------------------------------------

The main menu of the M16_CONF.ACC is divided into two major sections:

1. Status Section

Here you will find some informations (read only) about the actual

size of the desktop, the current mode and the operating mode of

the VT52 emulator: 'original' means that the VT52 output will go

to the ATARI video display, '80x25' means that it will show up

on the card.

2. Options Section

LOCK M16 BOARD

Here you can lock/unlock the board. This is only important if

you are having difficulties running certain programs that need

to have exclusive access to the board. This is only the case

if you run certain CyReL programs (like VIEW_GIF) etc.

Note:

Most GEM based programs do not require any changes

to the lock/unlock status of the board.

VT52 EMULATOR

This feature allows you do redirect the output of the VT52 emulator

to either the CyReL SUNRISE M16-1280 or to the original ATARI video

display. You can switch the output at any time. A two monitor setup

can provide a more comfortable environment.

VIRTUAL SCREEN

Not yet implemented.

XBIOS/LINE-A PARAMETERS

Click into this box to get to the M16_CONF sub menus.

OK

If the accessory detects any change in the current settings,

it will ask you if you want to only 'USE' the new configuration

or if you want to permanently 'SAVE' them in the EEPROM on the

card. In case you do not want to do either, select 'CANCEL'

CANCEL

Select this button to diregard any changes. Even if you already

changed some of the switches, the program will restore them so

that the proper mode will be shown the next time you enter the

M16_CONF.ACC.

---------------------------------------------------------------------------

[C] SUB MENU

---------------------------------------------------------------------------

XBIOS EMULATION ENABLED

Enable this option to run some programs that may otherwise

ask you to switch to a certain mode (like:'program runs in

high resolution only').

Note: This is only a trick to fool programs that do not

correctly evaluate the VDI informations. It is therefore

not a cure for all of the mentioned problems.

TT EXTENDED COLOR EMULATION

Enable this switch to emulate the extended color functions

found in the TT XBIOS.

Note: to use this feature, you MUST enable the XBIOS

EMULATOR as well!

LINE-A EMULATION ENABLED

If the CyReL LINE-A Emulator should cause any problems

during the execution of a program, try to disable it with

this switch. Despite the fact that this may seem very

unlikely, the option has been provided so that the user

can enable/disable the emulator as required.

OK

Returns to the main menu of the M16_CONF.ACC and accepts

all changes made in the sub menu.

CANCEL

Disregard any changes in this sub menu and return to the

main menu of M16_CONF.ACC.

SCREEN MODE

Click on the down arrow or up arrow (or on the mode field

itself to activate the popup menu) and select the information

that the XBIOS GetRez() call should return.

Some programs use this information to decide whether or not

they can run in the present resolution. Since this is neither

100% accurate, reliable nor the proper way of doing it, you

can select what type of information should be returned.

For more information, refer to the M16XBIOS.DOC in the M16_HELP

folder.

NOTE: This trick does not cover all cases. Some programs search

through the LINE-A area (which has just been adopted to work

with the new CyReL modes), some really ask the AES (but the AES

HAS to work in the new mode, otherwise there will be no valid

output on the screen) others ask the XBIOS and may therefore be

fooled by the previously described trick. Experiment with the

Screen mode parameter, to find out the best setting.

---------------------------------------------------------------------------

[D] NOTES

---------------------------------------------------------------------------

IMPORTANT:

The M16_CONF accessory/program will only update the CyReL VDI and the

configuration of the card AFTER you pressed the 'OK' button in the

main menu and selected either 'USE' (for temporary changes) or 'SAVE'

(for permanent changes).

Recommended default settings:

VT52 = ON   (if you use only one monitor at a time, otherwise

you should disable this feature so that the VT52

Status reads: '80 x 25')

XBIOS= ON   unless you have some programs that do funny things

with the colors. Color mapping & conversion from the

original Atari values will be performed.

LINEA= ON   it helps to run a variety of 'not 100% VDI compatible

software' on the card.

Note: This is just a preliminary description of the M16_CONF accessory.

The next update will feature a in-depth coverage of all available switches.

---------------------------------------------------------------------------

<EOF>