XBIOS Video Calls

920804 kbad

Introduction

The XBIOS video calls in TOS 4.0 provide a more machine-independent

XBIOS interface to video hardware for Falcon030 and beyond.  Some

existing calls differ slightly from previous TOS versions.  New calls

to support video device drivers and other low-level video programming

have been added.

Video Mode Word

The core of these video calls use a "mode word", a 16 bit value which

describes a video mode.  A mode word looks like this:

bit  name  description

0000 000v sopm hbbb  ---  ----- -----------

^^^  2-0  VBPP  log2 bits per pixel

h---   3   VHORZ Horizontal resolution (1=normal 0=halved)

m ----   4   VMON  Monitor flag (1=VGA 0=TV/SC1224)

p- ----   5   1=PAL 0=NTSC

o-- ----   6   Overscan

s--- ----   7   ST compatbile mode

v ---- ----   8   Vertical resolution (depends on 'm' bit)

0000 000- ---- ----  9-f  reserved, should be 0

Bits Per Pixel

VsetMode	[88 / 0x58]	Set video mode

-------------------------------------------------------------------------

XBIOS function - Trap 14

Available in TOS 4.00

-------------------------------------------------------------------------

VgetMonitor	[89 / 0x59]	Return monitor type

-------------------------------------------------------------------------

XBIOS function - Trap 14

Available in TOS 4.00

-------------------------------------------------------------------------

VsetSync	[90 / 0x5a]	Set internal/external video clock/syncs

-------------------------------------------------------------------------

XBIOS function - Trap 14

Available in TOS 4.00

-------------------------------------------------------------------------

VgetSize	[91 / 0x5b]	Return the size of a screen in bytes

-------------------------------------------------------------------------

XBIOS function - Trap 14

Available in TOS 4.00

-------------------------------------------------------------------------