SY55&TG.DK Help File

Loading in the file SY55&TG.DK will setup Band-in-a-Box for the Yamaha

SY55 multi-timbral synthesizer and the TG55 multi-timbral module.

It will setup:

1. Drum #'s for the Yamaha SY55/TG55 synthesizer/module(Voice63 Drum Set 1).

2. Patch Map (assigned to factory settings of the SY55/TG55).

3. Startup Patches and Channels in the Midi Settings Window.

The SY/TG series of synths and synth modules are 8 part multi-timbral

(8 instruments at once). We have included a System Exclusive file for the

SY55 and TG55 to make your setup an easy one time procedure. These files can

be accessed from the MIDI pulldown menu within the Band-in-a-Box program

(Send Sys Ex File).

To setup your SY55/TG55 do the following:

Go to UTILITY mode on your synth. Find the display UT MIDI\Bulk In

Protect=. Using the [+1/YES] key select ON. This will enable your SY55/TG55

to receive System Exclusive information(i.e. setup information). Next ensure

that the MIDI OUT of your computer is going to the MIDI IN of your synth.

Select 'Send Sys Ex ' from the MIDI pulldown menu in Band-in-a-Box.Choose

the file SY55&TG.SYX and hit {enter}. Your synthesizer should now be

receiving the Sys Ex data. The SY55/TG55 will now be setup with the proper

MIDI channels on their respective parts in MULTI mode. You may now proceed

to loading SY55&TG.DK under the MIDI pulldown menu (Load Alternate Drum/Midi

Set). Select MULTI mode on your synth so it will receive all 8 of the

instruments being sent to it from Band-in-a-Box.

Once you've loaded the SYX and DK files, you are ready to start having

fun with Band-in-a-Box!

Have Fun!