SQ_ENSON.DK Help File

Loading in the file SQ_ENSON.DK will setup Band-in-a-Box for the Ensoniq

SQ1 and SQ2 multi-timbral synthesizers.

It will setup:

1. Drum #'s for the Ensoniq SQ series All Purpose Kit (Patch 89).

2. Patch Map for the Ensoniq SQ series synths.

3. Startup Patches and Channels in the Midi Settings Window.

Your SQ has to be setup to receive in a Multi-timbral fashion,meaning the

ability to play more than one sound at a time. The SQ has its own built-in

sequencer which has eight tracks. In making songs on the SQ,you first put

together seperate sequences that will later be compiled into a song format.

This is where setting up a Multi-timbral patch comes into play. Multi-

timbral setups are done in a sequence patch using the TRACK buttons (1-8) as

if you were about to sequence something from within the SQ.

Ensoniq SQ Setup Instructions:

-Choose SELECT to find an unused sequence preset.Name it Band in a Box.

-Choose TRACK PARAMETER to get to STATUS.Select MIDI.

-Select the BLUE EDIT button. Press TRACK 1 button.Select TRACK PARAMETER.

-Scroll through to select MIDI/SOUND/and Volume settings.

-Using TRACK buttons 1 through 8 and the above command do the following:

Bass   Piano   Drums    Guitar  Horns   Strings  Melody    Thru

Track-        1       2       3       4       5       6        7         8

Midi -        2       3      10       7       8       6        4         5

Volume-      75      60      99      99      99      99        75        99

Sound-Bank   6-0     1-1     8-9     3-5     4-7     1-7      3-5       3-5

All that needs to be done now is load the file SQ_ENSON.DK from the DRUMS

pulldown menu(Load Drumkit From Disk) and you're ready to have fun

with Band-in-a-Box!

If you have a sequencer,we recommend saving these settings as a System

Exclusive File which can later be transmitted back to your SQ via Band-in-a-

Box should your synth settings change.

Have Fun!