AHDFIX                                            December 17, 1986

------

Warning:  This document applies to the Atari SH204 only.  It does not apply

to any other hard drive.

Background:    pc-ditto version 3.0 does not properly recognize or access

an Atari SH204 Hard Disk:

1.  When started, the pc-ditto startup screen will identify

an attached hard disk, but the number of partitions is

often wrong.

2.  In addition, when accessing one of the partitions, numerous

problems may be encountered.  The problems are not

well-defined, and thus can not be specified.  It is

sufficient to point out that any problem with the hard disk

is not normal.  Should the problems continue, the user

should exercise caution and maintain backups of valuable

data.

Solutions:     1.  To correct the number of partitions, select the YES option

in pc_dmenu.prg for the Ignore non-GEM partitions,

located in the DISK submenu.

2.  To correct the partition access problems, run the program

AHDFIX.PRG after EACH use of the Atari hard disk utilities:

HDX.PRG

HINSTALL.PRG

NOTE:  Reread the above and note it says after EACH use.

BUT, it does NOT mean after each autoboot from the

hard disk.  Only after formatting, partitioning,

zeroing, or marking bad sectors (which is done in

the HDX.PRG program).  And, only after installing

the autoboot (which is done in the HINSTALL.PRG).

NO REFORMATTING REQUIRED!  That's right!  Just run HDXFIX

on your Atari SH204.  It will not bother any data

or change the format.

Listed below are instructions for several possible alternatives.

Be sure to use the example procedure for your situation. (and, be sure

to be familiar with the Atari instructions for formatting an Atari

hard drive.)

Instructions on correcting SH204 problems:

No reformatting or autoboot installation

----------------------------------------

Do these steps, if you just want to get pc-ditto to recognize your hard

disk, but don't want DOS installed on it.

Step 1.   Run the AHDFIX program.

Step 2.   Run the pc_dmenu.prg program.

Step 3.   Select the DISK submenu.

Step 4.   Select the YES option for Ignore non-GEM Hard disk partitions.

(If already YES, skip to Step 6).

Step 5.   Select Save Changes.

Step 6.   Select Save All Changes.

Step 7.   Install the pc-ditto hard disk driver (see your update instructions).

You're finished!  Now, pc_ditto will recognize your Atari SH204 properly.

Reformatting SH204

------------------

Do these steps when you want to erase the entire SH204.

Step 1.   Run the HDX program.  (You must have the proper Atari files on your

disk.  See the Atari hard disk documentation for more information).

Step 2.   Select the FORMAT option and follow the prompts as usual.

Step 3.   Select the PARTITION option and select the partitions you desire.

Step 4.   Select the Quit option.

Step 5.   Run the AHDFIX program.

Step 6.   Run the pc_dmenu.prg program.

Step 7.   Select the DISK submenu.

Step 8.   Select the YES option for Ignore non-GEM Hard disk partitions.

(If already YES, skip to Step 10).

Step 9.   Select Save Changes.

Step 10.  Select Save All Changes.

Step 11.  Install the pc-ditto hard disk driver (see your update instructions).

You're finished!  Now, pc_ditto will recognize your Atari SH204 properly.

Installing hard disk autoboot

-----------------------------

Do these steps when you wish to autoboot GEM from the hard disk without

a floppy.  (Your disk should be already formatted, and may already contain

data).

Step 1.   Run the HINSTALL program.  (You must have the proper Atari files

on your disk.  See your Atari documentation for more information).

Step 2.   Select the INSTALL option.

Step 3.   Select the Quit option.

Step 4.   Run the AHDFIX program.

Step 5.   Run the pc_dmenu.prg program.

Step 6.   Select the DISK submenu.

Step 7.   Select the YES option for Ignore non-GEM Hard disk partitions.

(If already YES, skip to Step 9).

Step 8.   Select Save Changes.

Step 9.   Select Save All Changes.

Step 10.  Install the pc-ditto hard disk driver (see your update instructions).

You're finished!  Now, pc_ditto will recognize your Atari SH204 properly.

Installing DOS on your hard disk and installing GEM autoboot

------------------------------------------------------------

Follow these steps to install DOS and GEM on your hard disk.

(Only IBM DOS may be installed.  See your pc-ditto disk file HARDDISK.DOC for

more information about the hard disk, before proceeding.)

Step 1.   If you are going to format the entire hard disk, follow the

steps in the section "Reformatting SH204", AND then continue with

Step 2 below.

Note:  Here we mean to format the entire hard disk with the

Atari format utility, HDX.PRG.  Not just formatting a

single partition for installing DOS.

Step 2.   If you do NOT want to save any data on partition C:,

skip to Step 3 below.

Otherwise, save the data files and folders to another drive

partition (say, another folder), or copy to floppy disks.

Step 2.   Start pc-ditto.

(We assume you have already set the HARD DISK ATTACHED option

to YES in pc_dmenu.prg).

Step 3.   Insert the DOS disk into drive A: and press RETURN key.

Step 4.   Format the hard disk partition C: using the DOS FORMAT utility:

At the  A>  prompt   type

FORMAT C:/S

You should press the RETURN to in response to the volume label

question (if you get one).

Answer Y to the "format will erase the hard disk continue (Y/N)"

message.  (Actually, the format will erase C: only.  If you have

data on D: or other partitions, it will not be affected).

Step 5.   Return to GEM.

Step 6.   Run the HINSTALL program.  (You must have the proper Atari files

on your disk.  See your Atari documentation for more information).

Step 7.   Select the INSTALL option.

Step 8.   Select the Quit option.

Step 9.   Run the AHDFIX program.

Step 10.  Run the pc_dmenu.prg program.

Step 11.  Select the DISK submenu.

Step 12.  Select the YES option for Ignore non-GEM Hard disk partitions.

(If already YES, skip to Step 9).

Step 13.  Select Save Changes.

Step 14.  Select Save All Changes.

Step 15.  Install the pc-ditto hard disk driver (see your update instructions).

You're finished!  Now, pc_ditto will recognize your Atari SH204 properly.

-------Read no further if all you want to do is correct your problems-----

For only the curious and the technically-minded:

Why this problem occurred

-------------------------

pc-ditto is very complex.  The number of hardware combinations it must

be tested on is large.  To enhance the product, fix old bugs, and NOT

introduce any new bugs is a difficult task.

We use test matrixes to ensure that the most common combinations of

hardware are exercised.

Our tests of the SH204 used very old utilities.  The problem is caused by

changes in the Atari utilities of later versions.  We did not have those

later versions in our libraries (nor did we know of them, until recently).

What is causing the problems

----------------------------

1.   The partition number is caused by Avant-Garde following a de facto

standard used by Supra, ICD and Astra.  The standard permits from 2 to 12

partitions on a drive.  Atari permits 2 to 4.

When the hard drive shows 4 partitions, pc-ditto checks for a 5th.

The Atari root boot sector has data or trash where this 5th partition

ID would start.  Often, pc-ditto sees this data, assumes a 5th and more

partition exist.

Setting the non-GEM option stops pc-ditto after the 4th partition for

the Atari, since all other partitions would not have "GEM".

2.   The partition boot record has been trashed and that causes the second

problem for pc-ditto (Actually, the DOS has the hard time making sense

of the partition).

It appears that the partition image Atari uses is correct, but somehow

the HDX and HINSTALL trash part of the image when updating it for other

information.

Being familiar with testing procedures, this is one mistake that Atari

is really not at fault.  The Atari hard disk device drive AHDI.PRG

is programmed with information about the hard drive characteristics

(such as drive storage capacity and sectors per track).  Also,

information from the root boot sector provides partition sizes and

other information.

TRASHED DATA FIELDS      Atari          DOS

-------------------      -----          ---

# OF FATS                 20            02

MEDIA ID                  73            F8

SECTORS PER TRACK       2E70            0011

HEADS IN MEDIA          2C73            0004

HIDDEN SECTORS          2076            0001

Therefore, the driver does not read the partition boot records for

information concerning the partition boundaries and so on.  So, the

partition tables (Bios Parameter Blocks or BPBs for short) were never

read after the format operations.  Consequently, this problem was

never noticed, until...

pc-ditto version 3.0, unlike version 2.0, DOES read these tables and

uses the information to determine media size and pass other information

on to DOS.  This allows drives other than 20Meg and 4 maximum partitions

to be used.

What does HDXFIX.PRG do?

------------------------

Simple.  It reads the partition boot records and corrects the trashed

items.  Then, it writes the partition boot records back in place.

End of document

##########################################