Beep

Copyright (C) 1990

By David Baggett

--------------------------------------------------------------------------------

User Contract

This program is FREE.  You may distribute it at will, provided you

1) do not charge any money for it,

2) distribute it WITH the documentation, and

3) do not change the program or documentation in ANY WAY.

--------------------------------------------------------------------------------

Introduction

------------

In 1989, Dan Wilga released a Public Domain program called "newbell"

that replaced the boring "ding" sound the ST makes with a digitized

sound.  Unfortunately, newbell only allowed certain playback speeds, all of

which were incompatible with sounds sampled on the Mac.

Here's a new program that does the same thing, but allows sample playback

speed to be specified in Hertz.

General Usage

-------------

Put beep.prg in your AUTO folder and create beep.inf with a standard

ASCII text editor.  In beep.inf, put the name of the sample you want to

replace the system beep with and the playback speed in Hertz for the sample.

For example,

d:\usr\sounds\chbell.snd        8000

will replace the system beep with a church bell sample, played at 8000 Hertz.

Enabling and Disabling Beep

---------------------------

When beep.prg is invoked with no parameters, it reads the beep.inf

file and loads all the sounds.  This typically occurs at bootup in the

AUTO folder.  However, you can run beep.prg from a command shell (or

from the desktop if you rename a copy beep.ttp) as well.  To turn

Beep off, type

beep off

from a command shell, or run beep.ttp from the desktop with parameter

"off".

Similary, type

beep on

to turn Beep back on.

Specifying an Alternate D/A Device

----------------------------------

Beep can be configured to play its samples through other D/A devices.

(Currently, only the Hippo/Navarone digitizer is supported, but other

digitizers will be supported if people send me info on them.)

To use an alternate D/A device, put one of the following keywords

in curly brackets at the beginning or end of your beep.inf file:

hippo

For example, a beep.inf containing

{hippo}

d:\usr\sounds\zoinks.snd        8000

will cause Beep to play the "zoinks" sound through the Hippo/Navarone

digitizer attached to your ST's cartidge port.

Note that the normal ST system beep will still be masked, even if

you don't use the speaker as the D/A device.

Technical Information

---------------------

Beep steals Bios trap #13 and looks for Bconout(2, 7) calls.  When it sees

one, it plays the sound.

The program uses about 30K when resident, not including the space

taken up by the sample data.  (Down from 38K in version 1.0)

In earlier versions, Beep used bit 6 of the CONTERM global to enable

and disable itself.  Unfortunately, this conflicted with at least one

person's hard disk driver.  (Don't ask me why.)

This version uses the bit 6 of memory location 768, chosen arbitrarily

from the list of user interrupt vectors.  If for some reason Beep will not

run with a particular application resident, try moving the on/off flag

somewhere else in memory.  To do this, put the desired flag address in angle

brackets at the beginning or end of your beep.inf file; e.g.,

<1020>

d:\usr\sounds\zoinks.snd        8000

will cause Beep to use 1020 (decimal) as the memory location for the beep

on/off flag.  To then enable and disable Beep, pass the address as a third

parameter to the beep [on | off] call; e.g.,

beep off 1020

will mask bit 6 of the byte at location 1020 decimal, thereby disabling Beep.

Memory locations 512 through 1020 are good candidates for flag locations

since they are currently not claimed by Atari.

Whenever Beep is on it masks the normal system beep by clearing the

appropriate bit in CONTERM.

Beep is well-behaved and should work with all versions of TOS.  It does

not use any undocumented techniques or memory locations,except of course for

the beep on/off flag, which can be moved elsewhere in memory if necessary.

Caveats

-------

It is a FEATURE that Beep does not turn off mouse interrupts when playing

the sound.  (Otherwise you wouldn't be able to move the mouse until the

sound finished playing.)  What this means, however, is that you can slow

down the playback by jiggling the mouse around while the sound is playing.

Now  I don't want to hear any complaints about this, cause that's the way

it's gotta be!

Whenever you move the mouse, the processor is interrupted -- lots of

times, in fact.  (Technically, once for each mouse pulse).  The playback

routine taxes the 68000 and so the slightest interruption will slow the

playback down.  Ain't nothin' I can do about it.  'Nuff said.

It's best to play samples at fairly low rates (e.g., 8000 or 11000 Hertz)

so that you don't completely bog down the system.  Using samples at rates

above 11000 will often prevent the mouse from tracking correctly while the

sound is playing, and may cause weird things to happen to very CPU-intensive

applications.  (To see this effect, select the "About..." box in TOS 1.4 and

click outside the dialog box with Beep on.)

The program will not know it has already been installed if you

run it twice (or ten times) in a row.  Doing so will use up more and

more memory.