From comp.sys.atari.st.tech Fri Aug  3 14:29:21 1990

Path: ruuinf!piet

From: piet@cs.ruu.nl (Piet van Oostrum)

Newsgroups: comp.sys.atari.st.tech

Subject: atari st 3 MB memory

Message-ID: <3652@ruuinf.cs.ruu.nl>

Date: 27 Jul 90 11:43:05 GMT

Sender: news@ruuinf.cs.ruu.nl

Reply-To: piet@cs.ruu.nl (Piet van Oostrum)

Organization: Dept of Computer Science, Utrecht University, The Netherlands

Lines: 563

Please note that I am not the author of the following article. The author

had some problems posting, so I did it for him. Replies should be made to:

jeroen@phil.ruu.nl (Jeroen Scheerder)

Due to manifold requests (since my original posting was badly formatted - I

usually post things from xrn, which apparently has a weird editor) this articlei

is reposted in an edited version.

My ST *DEFINITIVELY* has 3.0 Mb. I agree, not long ago I thought this

impossible too; however, in the magazine "ST Computer" 3/1990 (a German

magazine), the author K. D. Litteck describes how it is done  in an article

called "Aus zweieinhalb mach drei" (an error correction appeared in ST

Computer 4/1990). This magazine is published in Germany by Heim Fachverlag.

It is in German, and my German is not too good, but I'll try to describe

the article accurately.

How does it work? Well after determining memory configuration (see below)

TOS starts clearing the amount of memory it thinks is available. Say

you've got 3.0 Mb available (like I do); TOS at first thinks 2 banks of

2 Mb are available and sets out to clear 4 Mbytes. It then runs into

nonexistent memory when it reaches the end of the three available megabytes.

An exception of type buserror occurs, and TOS determines at which adress the

error occurred. This adress is then written in the TOS variable 'phystop',

thus setting the correct memory size. Now TOS boots on ...

This works in all TOS versions known to mankind.

This is part 2 of an article about ST memory upgrading (technical part).

Before getting into the gruesome details, first some general ST memory theory:

ST is equipped with Dynamic RAM, and for every memory location a single

transistor is used. This allows for high speed and compact chips. However,

the chips need to be refreshed about every 2 millisecs, of which the MMU

(among other things) takes care.

The DRAMS are accessed bitwise; therefore, the ST adressing in 16 bits, only

banks of 16 chips can be used. Now the adressing signal is multiplexed (at

one adress line two adresses are located, that are consecutively transferred).

In DRAM these adresses are then separated. This keeps the number of pins as

small as possible. The multiplexing and sending control signals is catered

for by the MMU too.

Now the multiplexing is the reason the DRAMS can only be increased in 4-folds;

only 16 kBit, 64 kBit, 256 kBit, 1 MBit and 4 MBit chips currently exist.

Two control lines (called RAS and CAS) are of interest (both low-active). RAS

transfers the low adress of the multiplexed adress line, CAS the high adress.

They work too as chip-selectors. The signal WE selects READ or WRITE mode.

Furthermore, the ST divides CAS into CASL and CASH. CASL connects to the chips

on adress lines 0..7 and CASH to 8..15, so that memory can be byte-accessed

too. On WORD-read or write both CASL and CASH are active.

The ST's MMU can manage two banks of memory. For bank 0 the control lines

RAS0, CAS0L and CAS0H are used, bank 1 uses RAS1, CAS1L and CAS1H. These

banks can be equipped with 64, 256 or 1024 kBit chips.

On coldstart TOS writes patterns in memory and reads them back (all at

well-chosen locations) to determine the memory configuration, and stores

in the system variable memconf (adress $424) and the MMU's configuration

register ($FF8001) a configuration byte:

Bank 0:		Bank 1:		Value:		Total memory:

512 k		-		$04		0.5 M

512 k		512 k		$05		1.0 M

2.0 M		-		$08		2.0 M

2.0 M		512 k		$09		2.5 M

512 k		2.0 M		$06		2.5 M

2.0 M		2.0 M		$0A		4.0 M

(I hope this is right - anyway, it's in all good ST literature)

On top of all this, MMU also manages to make memory contiguous: allthough

the adress space of each bank is 2.0 Mb, in case your bank 0 is 512 k and 1

is 2.0 M the MMU concatenates the total adressing space to a contiguous one

(as TOS expects).

Now, that's all very well: now we want to fool the MMU to handle up to 5 (!)

memory banks.

A scan of the electronics schematic in MacPaint format (you need it!) is

posted too (binhexed and stuffed with Stuffit). This schematic is

corrected for the error in the original printing.

We need adress line A9 (not used on 256 kBit chips). It is multiplexed and

contains the real adresses A18 and A19. Both NAND-gates serve as OR-gate

and should release the D-Latch 1 and 2 when CAS1L or CAS1H become active.

An IC 74LS375 demultiplexes and gets out of A9 the adresses A18 and A19.

With the D-Latch 3 A18 is obtained, with D-Latch 1 A19. A18 is also led

over D-latch 2, being kept until CAS goes high.  The IC 74LS139 (2-bit

binary decoder) produces with A18, A19, CAS1L and CAS1H eight new CAS

signals, of which we only need four.  These four signals are fed to the

memory banks (all formerly 0).

Now, some things to pay attention to:

- the 2.0 Mb should be bank 0;

- the RAS0-line must be interrupted;

- the assignment of RAS- and CAS-lines MUST be corresponding.

MMU pin configuration:

6:	CAS0H

7:	CAS0L

8:	RAS0

18:	RAS1

21:	CAS1L

22:	CAS1H

64:	A9

Components:

- 1 IC 74LS00

- 1 IC 74LS375

- 1 IC 74LS139

- 4 68 Ohm resistors

- 1 Block condensator

- 1 microFahrad wire or copper litze

I hope this is clear enough to you all (once you've seen the picture

accompanying this, I hope).

Wish y'all good luck,

Jeroen.