FOREWORD

TM

GEM  DOS    is  a  single-user operating system designed for the Motorola..

TM

MC68000   or  compatible  68000  microprocessor.    GEM  DOS  requires  the

following minimum system configuration:

*   68000 microprocessor

*   Bit-mapped  video  controller  card  and  a  display  with  320  by 200

resolution

*   128K bytes of Random Access Memory (RAM)

*   160K bytes of Read-Only Memory (ROM)

*   An optical or track mouse or other pointing device

The following configuration represents a system that can exercise the  full

capabilities of GEM DOS:

*   68010, 68020, or 68070 advanced microprocessor

*   High-resolution,  color  bit-mapped  display and controller (640 by 400

pixel resolution, barrel-shifter hardware for very  high  speed  screen

refresh)

*   256K bytes of RAM

*   192K bytes of ROM

*   Mouse

*   Disk drives and controllers

*   Optical drives and controllers

*   Printer and communication ports

This  guide describes the programming interface to GEM DOS and is part of a

documentation set that also includes these manuals:

*   The GEM DOS User's Guide

*   The C Language Programming Guide for  GEM  DOS  (supplement  to  the  C

Language Programming Guide for CP/M-68K..)

*   The Mince User's Guide, prepared by Mark of the Unicorn, Inc.

*   The  Kermit  User's  Guide  prepared  by Columbia University Center for

Computing Activities

*   The GEM DOS Supplement to the Kermit User's Guide

This guide assumes you are an  experienced  programmer  familiar  with  the

Motorola  68000  assembly  language and the C programming language.  If you

are not  familiar  with  GEM  DOS,  refer  to  the  manuals  listed  above,

especially  the  GEM  DOS  User's  Guide.  If you are not familiar with the

Motorola 68000 assembly language, refer to the following Motorola manuals:

*   16-Bit Microprocessor User's Manual, third edition MC68000UM(AD3)

*   MC68000 Resident Structured Assembler Reference Manual M68KMASM(D4)

The following is a summary of the contents of this guide:

*   Section  1  discusses   the   GEM   DOS   system   architecture,   file

specification, and terminology.

*   Section  2  describes  the  programming  loading  procedure,  transient

program environment, and how to exit a transient program.

*   Section 3 describes the GEM DOS comand file (transient program) format.

*   Section 4 lists the system function call categories and  describes  the

system functions and their entry parameters and return codes.

*   Section 5 describes the GEM DOS assembler.

*   Section 6 describes the GEM DOS linkers.

*   Section 7 describes the GEM DOS archive utility.

*   Section 8 describes the DUMP, SIZE68, and SENDC68 utilities.

TM

*   Section 9 describes the SID-68K   debugger.

*   Appendix A lists the BIOS functions.

*   Appendix B shows the format of the GEM DOS base page.

*   Appendix C contains a decimal-ASCII-hexadecimal conversion table.

*   Appendix D lists the error messages returned by the GEM DOS programming

utilities.

*   Appendix E describes the Motorola S-Record format.

INTRODUCTION TO GEM DOS

1.1  SYSTEM ARCHITECTURE

Table 1-1 describes the three modules that comprise GEM DOS.

Table 1-1.  GEM DOS System Modules

Module

Console Command Processor

(CCP)

Basic Disk Operating System

(BDOS)

Basic I/O System

(BIOS)

1.2  TRANSIENT PROGRAMS

The Transient Program Area (TPA) consists  of  all  the  contiguous  memory

space  not  occupied  by  the  operating system when GEM DOS is resident in

memory.  GEM DOS loads executable command files from  disk  into  the  TPA.

Command  files  are  also  called  transient  programs because they are not

memory resident nor configured within the  operating  system.    Section  3

describes the format of a GEM DOS command file.

Section 2, "The CCP and Transient Programs," describes how

Section  2, "The CCP and Transient Programs," describes how GEM DOS formats

the TPA for programs it loads into memory for execution.

1.3  PROGRAMMING TOOLS

Table 1-2 lists the assembly language programming tools provided  with  GEM

DOS.  These tools are described in Sections 5 through 9 of this guide.

Table 1-2.  GEM DOS Programming Tools

Program   Purpose

AR68      Creates and modifies object module libraries

AS68      Processes   assembly  language  source  modules  to  create

relocatable object module library files

DUMP      Displays files in hexadecimal and ASCII notation

TM

LINK68    A linker that combines assembled or compiled object modules

with other modules from a runtime library

LO68      Another linker

NM68      Prints the symbol table of an object or command file

RELOC     Converts  relocatable  object  modules  from  one format to

another

SENDC68   Converts a command file to the Motorola S-record format

TM

SID-68K   Tests and debugs programs

SIZE68    Displays the size of each program  segment  within  one  or

more command files

XREF      Creates  a  table  of the symbols defined and referenced by

object files

Use AS68 to assemble the file and either LINK68 or LO68 to link it.    AR68

creates  library  modules  that can be linked with LINK68 or LO68.  Both of

the linkers use and create the same file formats.  However, if you are  not

linking with runtime modules, LO68 provides better performance.

NM68 is described in Section 3.

GEM  DOS  also supports a C language compiler for the 68000 microprocessor.

See the C Language Programming Guide for GEM DOS.

1.4  GEM DOS FILE SPECIFICATION

The GEM DOS file specification consists of four fields:    a  one-character

drive  designation  delimited  by  a  colon (d:), a directory path in which

directory names are delimited by backslashes (\), a one- to eight-character

filename,  and  an optional one- to three-character filetype separated from

the filename by a period (.).  Directories and files have the  same  naming

conventions.  The format of a file specification is shown below.

Format:

d:\path\filename.typ

Example:

A:\LIBRARY\OBJS\FLOATER.OBJ

The  drive designation, path, and filetype fields are optional.  The colon,

backslash, and period delimiters are required only when you  include  these

fields in the file specification.

Drive   designation   values   range   from  A  through  P  when  the  BIOS

implementation supports 16 drives, the maximum  number  that  GEM  DOS  can

support.  See Section 4.5, "File Management Functions."

The   characters  in  the  filename  and  filetype  fields  cannot  contain

delimiters (the colon, backslash,  and  period).    The  CCP  automatically

converts  a  command line and its file specifications (if any) to uppercase

internally before parsing them.

Table 1-3 lists some additional characters you should avoid using  in  your

file specifications.  These characters are reserved because several GEM DOS

built-in commands and utilities have special uses for them.

Table 1-3.  Delimiter Characters

Character

[]

()

<>

=

*

&

,

!

|

?

/

\

$

.

:

;

+

-

1.5  WILDCARDS

GEM DOS supports two wildcards, the question mark (?) and the  asterisk(*).

Several  utilities  and  BDOS functions allow you to specify wildcards in a

file specification to perform the operation or  function  on  one  or  more

files.

The ? wildcard matches any one character in the character position occupied

by  this  wildcard.    For  example,  the  file  specification  WH?TMAN.OBJ

indicates  that  the  third  letter of the filename can be any alphanumeric

character if the remainder of the file specification matches.  Thus, the  ?

wildcard matches exactly one character position.

The  * wildcard matches one or more characters in the field or remainder of

a field that this wildcard occupies.  GEM DOS internally pads the field  or

remaining portion of the field occupied by the * wildcard with ?  wildcards

before searching for a match.  For  example,  GEM  DOS  converts  the  file

specification  G*.BAT  to G???????.BAT before searching for a matching file

specification.  Thus, any file that starts with the  letter  G  and  has  a

filetype of BAT matches this file specification.

For  details  on  wildcard support by specific BDOS functions, refer to the

description of the function in Section 4 of this  guide.    For  additional

details  on  these wildcards and support by GEM DOS utilities, refer to the

GEM DOS User's Guide and Sections 5 through 9 of this guide.

1.6  GEM DOS TERMINOLOGY

Table 1-4 list the terminology used throughout this guide to  describe  GEM

DOS values and program components.

Table 1-4.  GEM DOS Terminology

Term      Meaning

Nibble    4-bit value

Byte      8-bit value

Word      16-bit value

Longword  Signed 32-bit value

Address   32-bit value that specifies a storage location

Offset    A  fixed  displacement  that references a storage location,

other data source, or destination

Text Segment

The   section  of  a  program  that  contains  the  program

instructions

Data Segment

The section of a program that contains initialized data

bss       The  block  storage  segment is a section of a program that

contains uninitialized data

Table 1-5 describes conventions used in this guide.

Table 1-5.  GEM DOS Programmer's Guide Conventions

Convention

Meaning

[]        Square   brackets   in  a  command  line  enclose  optional

parameters.

nH        The capital  letter  H  follows  numeric  values  that  are

represented in hexadecimal notation.

(n)       BDOS function numbers are enclosed in parentheses when they

appear in text.

Ctrl-X    The mnemonic Ctrl-X indicates that you are to hold down the

key labeled Ctrl while you press the key represented by the

variable X.

End of Section 1

THE CCP AND TRANSIENT PROGRAMS

This section discusses the Console Command Processor  (CCP),  built-in  and

transient commands, loading and exiting transient programs, and the GEM DOS

program execution model.

2.1  CCP BUILT-IN AND EXTERNAL COMMANDS

After an initial cold start, GEM DOS displays  a  sign-on  message  at  the

console.   The drive containing the system disk is logged in automatically.

GEM DOS displays its "{a}" prompt  to  indicate  the  currently  logged  in

drive,  and  that  the  operating system is ready to receive a command line

from the console.

In response to the prompt, a user types the filename of a batch or  command

file  with  its  command  tail, if required.  GEM DOS supports two types of

commands, built-in and transient.  Built-in commands are part of  the  CCP.

Transient  commands  are  loaded  in  the  TPA  and do not reside in memory

allocated to GEM DOS.  The GEM DOS User's Guide describes the built-in  and

transient commands that GEM DOS supports.

A  transient  command is a machine-readable program file that GEM DOS loads

from disk into memory for execution.  Section 3  describes  the  format  of

transient command files.

When the user enters a command line, the CCP parses it and tries to execute

the file specified.  The CCP asssumes a file is a command file when it  has

the  filetype  BAT.    When  the  user specifies only the filename, the CCP

searches the current directory  and  drive  for  a  file  with  a  matching

filename and a filetype of BAT.  If the CCP does not locate a batch file to

match the filename specified, it searches for a command file whose filetype

is PRG.

2.2  LOADING A PROGRAM

Either  the  CCP or a transient program can load a program into memory with

the BDOS Load and Execute a Process Function (4B) described in Section 4.7.

After  the  program is loaded, the TPA contains the program segments (text,

data, and bss), a user stack, and a base page.  A base page exists for each

program  loaded  into  memory.   The base page is a 256-byte data structure

that  defines  a  program's  operating  environment.    The  base  page  is

initialized  by  the  Load and Execute a Process (4B) function described in

Section 4.7.  GEM DOS allocates 256 bytes (minimum) for the program's  user

stack.

2.2.1  The TPA and Base Page Initialization

The format of the Transient Program Area (TPA) that GEM DOS establishes for

an application program is shown in Figure 2-1.

__________ <-------- High TPA

|          |<---- Initial Stack Pointer

|  Appli-  |

|  cation  |

|   User   |

|   Area   |

|          |

------------

|    BSS   |

------------

|   Data   |

------------

|   Text   |

------------

| Base Page|

------------<-------- Low TPA

Figure 2-1.  GEM DOS TPA Format

The initial stack pointer is directed at the top end of the TPA.  The  base

page  contains  pointers  as  described  in Section 2.3.  To insure maximum

compatibility with future releases of  GEM  DOS,  it  is  recommended  that

memory  not  required  by your program be freed for use by other processes.

The top of the stack should be moved down to an acceptable  value  adjacent

to  or  within  the bss, and an M_SHRINK (4A) function should be performed.

If additional memory is needed later, perform an  M_FREE  (49)  or  M_ALLOC

(48) function to determine the amount of available memory (specify nbytes =

-1 for the M_ALLOC call); follow this by another call to M_ALLOC to have  a

portion  of  that  memory  allocated.  These operations reformat the TPA as

shown in Figure 2-2.

__________ <---------- High TPA

|          |

| Available|

| to other |

| Processes|

------------<----------- Stack Pointer

|  Appli-  |

|  cation  |

|  User    |

|  Area    |

____________

|   BSS    |

------------

|   DATA   |

------------

|   TEXT   |

------------

| BASE PAGE|

------------<----------- Low TPA

Figure 2-2.  Adjusted TPA

2.3  BASE PAGE FORMAT

Table 2-1 lists the byte offset, length, value, and briefly  describes  the

fields  in  the  base page that Load and Execute a Process (4B) initializes

for transient programs.

Table 2-1.  Base Page Format

Base Page

Offset    Length  Value        Description

00        4     p_lowtpa     Base address of the TPA

04        4     p_hitpa      High address of the TPA + 1

08        4     p_tbase      Base address of text (code)

0C        4     p_tlen       Length (in bytes) of text (code)

10        4     p_dbase      Base address of initialized data

14        4     p_dlen       Length (in bytes) of data

18        4     p_bbase      Base address of BSS uninitialized

data

1C        4     p_blen       Length of BSS uninitialized data

2C        4     p_env        Pointer to the environment string

x80       x80    p_cmdlin     Command line image

Note:  All other base page fields are reserved.

2.4  EXITING TRANSIENT PROGRAMS

GEM DOS supports two ways of exiting  a  transient  program  and  returning

control to the CCP:

*   Interactively, when the user types Ctrl-C at the console

*   A programmed return to the CCP through:

--  Program Terminate (00)

--  Terminate and Stay Resident (31)

--  Terminate Process (4C)

Typing  Ctrl-C  from  the  console  returns  control to the CCP only if the

program uses any of the following functions:

*   Read Character from Standard Input Device (01)

*   Write Character to Standard Output Device (02)

*   Read Character from Standard Input, No Echo (08)

*   Write String to Standard Output Device (09)

*   Read Edited String from Standard Input Device (0A)

*   Check Status of Standard Output Device (10)

Ctrl-C terminates execution of the main program and any additional programs

loaded beyond the CCP level.

End of Section 2

GEM DOS COMMAND FILE FORMAT

The  GEM  DOS  command  file  format  is  the  output  of the RELMOD format

conversion utility.  RELMOD accepts a CP/M-68K.. format relocatable  output

file generated by LINK68 or LO68, the GEM DOS linkers, and converts it to a

GEM DOS relocatable file.   RELMOD,  LINK68,  and  LO68  are  described  in

Section 6.

Command  files  consist  of  a  file  header  followed by the text and data

segments.    Optionally,  a  command  file  contains  a  symbol  table  and

relocation  information.    These  components  are  shown in Figure 3-1 and

described in the following sections.

______________________

|         File         |

|        Header        |

|______________________|

|       Text and       |

|     Data Segments    |

|______________________|

|     Symbol Table     |

|     (if present)     |

|______________________|

|    Relocation Data   |

|     (if present)     |

|______________________|

Figure 3-1.  GEM DOS Command File Layout

3.1  THE HEADER AND PROGRAM SEGMENTS

The  figure  3-2  shows  the  format  of  the  header   for   non-segmented

(contiguous) executable files.

+--------+--------+

00H |      601AH      |

+--------+--------+--------+--------+

02H |  Number of bytes in text segment  |

+--------+--------+--------+--------+

06H |  Number of bytes in data segment  |

+--------+--------+--------+--------+

0AH |      Number of bytes in bss       |

+--------+--------+--------+--------+

0EH |  Number of bytes in symbol table  |

+--------+--------+--------+--------+

12H |        Reserved, must be 0        |

+--------+--------+--------+--------+

16H |        Reserved, must be 0        |

+--------+--------+--------+--------+

1AH |     Reserved    |

+--------+--------+

Figure 3-2.  GEM DOS Command File Header

The  14-word,  601AH-type  header  denotes a relocatable file that contains

contiguous text, data, and block storage segments.  Note that GEM DOS  does

not support programs with non-contiguous segments.

The fields at offsets 12H, 16H, and 1AH are reserved and must be zero.  The

text segment image follows the field at offset 1AH.

The text segment contains  the  executable  instruction  code.    The  data

segment  contains  the  initialized  data  variables.    The  text and data

segments must be relocatable.

The inclusion of the symbol table and relocation information in the command

file is controlled by the GEM DOS linkers, described in Section 6.

3.2  SYMBOL TABLE

The  symbol  table  defines the symbols referenced by a command file.  Each

entry in the symbol table is associated with an index  that  indicates  its

entry  number.    Entries are numbered sequentially starting with zero.  As

shown in Figure 3-3, each symbol table entry is  composed  of  seven  words

that describe the symbol's name, type, and value.

+--------+--------+

0 |                 |

+      Symbol     +

|                 |

+                 +

|       Name      |

+                 +

6 |                 |

+--------+--------+

8 |       Type      |

+--------+--------+

A |                 |

+      Value      +

C |                 |

+--------+--------+

Figure 3-3.  Symbol Table Entry

Table 3-1 describes the fields of a symbol table entry.

Table 3-1.  Symbol Table Entry Fields

Field     Definition

Name      Symbol   name,   null-padded   right  if  less  than  eight

characters.

Type      Symbol type as indicated by the following values:

0100H:  bss-based relocatable

0200H:  text-based relocatable

0400H:  data-based relocatable

0800H:  external reference

1000H:  equated register

2000H:  global

4000H:  equated

8000H:  defined

Value     Symbol value where the value can be  an  address,  register

number, value of an expression, and so forth. Note that the

linkers interpret the value when  the  symbol  is  external

(0800H) as the size of a common region.

3.3  PRINTING THE SYMBOL TABLE

You  can  print the symbol table of an object or command file with the NM68

utility.  Use the following command line to invoke NM68:

{a}NM filename

NM68 operates on object or command files.  NM68 does not sort the  symbols;

rather, it prints them in the order in which they appear in the file.  Each

symbol name is printed, followed by its  value  and  one  or  more  of  the

following type-descriptors:

*   equ (equated)

*   global

*   equreg (equated register)

*   external

*   data

*   text

*   bss

*   abs (absolute)

You  can  use  the I/O redirection facility to store NM68 output in a file.

To redirect NM68 output to a file, specify a greater-than-sign  (>)  and  a

file  specification  following  the filename for which NM68 is to print the

symbol table.

3.4  RELOCATION INFORMATION

The relocation information indicates the offset  between  longwords  to  be

relocated.   The offset from the beginning of the text segment to the first

longword that requires relocation is stored as  a  longword  following  the

symbol  table.  Following that, bytes are used to store succeeding offsets.

If the offset between two longwords to be relocated  is  greater  than  254

bytes,  the relocation information has a byte of 1 for each multiple of 254

bytes, plus a byte whose value indicates any  remainder.    A  zero  offset

value  indicates the end of relocation information.  Note that GEM DOS does

not support 16-bit relocatable objects.

See Figure 3-3.  Table 3-2 lists the  relocation  offset  byte  values  and

their meanings.

.                          .

.                          .

.__________________________.

|       Symbol Table       |

|__________________________|

|      Offset of 1st       |

|   Longword to Relocate   |

|     (Longword Value)     |

|__________________________|

|    Succeeding Offsets    |

|       (Byte Values)      |

|__________________________|

|          End of          |

|  Relocation Information  |

|        (Zero Byte)       |

|__________________________|

Figure 3-4.  Location and Format of Relocation Information

Table 3-2.  Relocation Offset Byte Table

Byte

Value

0

1

2 through 254

255

*

Plus 254 times the number of "1" bytes that preceded this byte.

All relocation information is relative to the beginning load address (start

of text).

End of Section 3

SYSTEM FUNCTION CALLS

This section describes the GEM  DOS  system  function  calls,  their  entry

parameters, return values, and error codes.

4.1  INTRODUCTION

The  GEM  DOS  system  calls  are  divided  into  the following categories:

Character Device I/O, File  Management,  Drive  and  Directory  Management,

Program Control, Memory Management, and Time.  Table 4-1 lists each GEM DOS

system call according to these categories.  Also listed in  Table  4-1  are

references  to  the  sections  of  this  chapter that describe each call in

detail.

Table 4-1.  GEM DOS System Call Categories

Number

Character Device I/O, Section 4.4

01

02

03

04

05

06

07

08

09

0A

0B

10

11

12

13

File Management, Section 4.5

1A

2F

3C

3D

3E

3F

Table 4-1.  (continued)

Number

40

41

42

43

44

45

46

4E

4F

56

57

Drive and Directory Management, Section 4.6

0E

19

36

39

3A

3B

47

Program Control, Section 4.7

00

25

30

31

35

4B

4C

Memory Management, Section 4.8

48

49

4A

Table 4-1.  (continued)

Number

Time, Section 4.9

2A

2B

2C

2D

4.2  FUNCTION CALL PARAMETERS

The GEM DOS parameter-passing convention supports function call  references

from C and other high-level languages.

A function call reference from a high-level language constructs a parameter

block that  contains  the  number  of  the  function  call,  any  necessary

parameters,  and  other relevant data (such as a drive code or file handle)

and performs a TRAP #1 instruction.  GEM DOS then processes  the  reference

and executes the appropriate actions.

The  general  format of a GEM DOS function call parameter block is shown in

Figure 4-1.

Low Memory

+--------------------------------+ (Parameter

|         Function Number        |  Block Address)

+--------------------------------+

|          1st Parameter         |

+--------------------------------+

|          2nd Parameter         |

+--------------------------------+

|          3rd Parameter         |

+--------------------------------+

.                                .

.                                .

.                                .

+--------------------------------+ High memory

Figure 4-1.  Function Call Parameter Block Format

The positions of the parameters and  the  function  number  in  Figure  4-1

conform  to  the  order  in  which  they  are  placed on the stack by the C

language.  Specifically, the C code:

CALL (functionnumber, P1, P2, P3, ... Pn)

loads the Pn parameter into the  parameter  block  first,  followed  by  P3

through  P1,  until  the function number is the last entry in the parameter

block.

4.2.1  GEM DOS File and Device Handles

F_CREATE  (3C),  F_OPEN  (3D),  and  F_DUP  (45)  return  a  16-bit  binary

identifier  value  in register D0.L.  This identifier is called a "handle."

The handle is used to later identify files or devices that have been opened

or  created  with  the system call that originally returned it.  Although a

handle is 16 bits long, your program should test the entire long word value

returned  in  D0.L  to determine if the value represents an error code or a

valid handle.

Table 4-2 lists the handles that identify devices.

Table 4-2.  GEM DOS Device Handles

Handle

Value        Device

4          Standard list device

Output can be redirected.

3          Standard AUX device

I/O can be redirected.

2          Standard error device (stdErr)

I/O can be redirected.

1          Standard output device (stdOut)

Output can redirected at

the command line.

0          Standard input device (stdIn)

Input can be redirected at

the command line.

Your program can use these handles without previously opening them.    Your

program can redirect I/O for these devices with F_FORCE (46).

Your  program  can  open  the following devices by specifying a device name

with the F_OPEN (3D) function:

*   The console device, CON:

*   The auxiliary device, AUX:

*   The list device, PRN:

*   The bit bucket, NUL:

Note that I/O for the devices opened by name cannot be redirected.  See the

descriptions of F_OPEN (3D) and F_FORCE (46) in Section 4.5.

4.2.2  Return Values

GEM  DOS function calls return all byte, word, and long data in register D0

by default.  Those function calls that return more  information  place  the

address of an information return buffer in D0 or fill in a caller-specified

buffer.  The individual function call descriptions explain how data returns

to the caller.

4.2.3  Error Return Codes

The  GEM  DOS  function  calls place a negative number in D0 to indicate an

error return.  See the  individual  function  call  descriptions  for  more

complete information.

Table  4-3  lists the error codes returned by the GEM DOS function calls in

register D0.L.  Also listed in Table 4-3 are the  numbers  of  the  PC  DOS

function call error codes that correspond to the GEM DOS error codes.

Table 4-3.  BDOS Error Codes

GEM DOS    PC DOS

Return    Equiv.  Code         Error Report

-32L       1     EINVFN       Invalid function

-33L       2     EFILNF       File not found

-34L       3     EPTHNF       Path not found

-35L       4     ENHNDL       No handles left (too many open f

-36L       5     EACCDN       Access denied

-37L       6     EIHNDL       Invalid handle

-39L       8     ENSMEM       Insufficient memory

-40L       9     EIMBA        Invalid memory block address

-46L       15    EDRIVE       Invalid drive specified

-49L       18    ENMFIL       No more files

-64L             ERANGE       Range error

-65L             EINTRN       Internal error

-66L             EPLFMT       Invalid program load format

-67L             EGSBF        Setblock failure (due to growth

restrictions)

4.3  GEM DOS SYSTEM CALL SUMMARY

Table  4-4 lists the hexadecimal number and mnemonic of each GEM DOS system

function call.  The table includes the parameters a process  must  pass  to

the system call and the values the system returns to the calling process.

Table 4-4.  GEM DOS System Call Summary

Hex    Mnemonic             Input Parameters

0     P_TERM0              none

1     C_CONIN              none

2     C_CONOUT             Character

3     C_AUXIN              none

4     C_AUXOUT             Character

5     C_PRNOUT             Character

6     C_RAWIO              FF/Character

7     C_RAWCIN             none

8     C_NECIN              none

9     C_CONWS              *Character String

A     C_CONRS              *Input Buffer

B     C_CONIS              none

E     D_SETDRV             Drive Code

10    C_CONOS              none

11    C_PRNOS              none

12    C_AUXIS              none

13    C_AUXOS              none

19    D_GETDRV             none

1A    F_SETDTA             DTA (Long Value)

Table 4-4.  (continued)

Hex    Mnemonic             Input Parameters

25    S_SETVEC             Vector#

Address

2A    T_GETDATE            none

Bits 0-4 = day (1-31)

Bits 5-8 = month (1-12)

Bits 9-15 = year (0-119)

2B    T_SETDATE            Date Word:

Bits 0-4 = (1-31)

Bits 5-8 = (1-12)

Bits 9-15 = (0-119)

2C    T_GETTIME            none

Bits 0-4 = 2-sec increments

Bits 5-10 = minutes

Bits 10-15 = hours

2D    T_SETTIME            Time Word (binary):

Bits 0-4 = 2-sec increments

Bits 5-10 = minutes

Bits 10-15 = hours

2F    F_GETDTA             none

30    S_VERSION            none

31    P_TERMRES            Memory Size

Exit Code

35    S_GETVEC             Vector#

36    D_FREE               *Return Buffer

Drive Code           # of Clusters

Bytes/Sectors

Sects/Clusters

D0.L = Error Code

39    D_CREATE             *Path String

3A    D_DELETE             *Path String

Table 4-4.  (continued)

Hex    Mnemonic             Input Parameters

3B    D_SETPATH            *Path String

3C    F_CREATE             *Path String

Attribute

3D    F_OPEN               *Path String

Access Mode

3E    F_CLOSE              Handle

3F    F_READ               Handle

# of Bytes           Error Code

*Buffer

40    F_WRITE              Handle

# of Bytes           Error Code

*Buffer

41    F_DELETE             *Path String

42    F_SEEK               Offset

Handle               Error Code

Mode Value

43    F_ATTRIB             *Path String

Mode Value           Error Code

Attribute

44    F_IOCTL              Subfunction #

Handle or Drive Code Bit Map of Device Information

Byte Count or Bit Map

*Buffer              # of Bytes Read/Written

I/O Status

Media Type

45    F_DUP                Handle

Error Code

46    F_FORCE              Standard Handle

Duplicate Handle

Table 4-4 (continued)

Hex    Mnemonic             Input Parameters

47    D_GETPATH            *Path Buffer

Drive Code

48    M_ALLOC              # of Bytes or -1L

(for # of available bytes)

0 on Failure

49    M_FREE               Memory Block Addr

4A    M_SHRINK             Memory Block Addr

# of bytes to keep

4B    P_EXEC               Load Value

*Path String         Base Page Addr

*Command Tail        Error Code

*Environment Strings

4C    P_TERM               Status Code

4E    F_SFIRST             *Path String

Attribute

4F    F_SNEXT              none

56    F_RENAME             *Path String

*Path String (New Name)

57    F_DATIME             *Time/Date Buffer

Buffer

Handle

Get/Set Flag

Conventions used in Table 4-4:

*         =   Pointer to

#         =   Number

Addr      =   Address

DTA       =   Disk Transfer Address

Handle    =   File or Device Handle

Path String   =

directory path, and filename

4.4  CHARACTER DEVICE I/O FUNCTIONS

Table  4-5  lists  the  number,  mnemonic,  and  name of the GEM DOS system

function described in this section.

Table 4-5.  Character Device I/O Functions

Hex     Mnemonic

01      C_CONIN

02      C_CONOUT

03      C_AUXIN

04      C_AUXOUT

05      C_PRNOUT

06      C_RAWIO

07      C_RAWCIN

08      C_NECIN

09      C_CONWS

0A      C_CONRS

0B      C_CONIS

10      C_CONOS

11      C_PRNOS

12      C_AUXIS

13      C_AUXOS

Function 01:  Read Character from Standard Input Device

C Interface:

LONG

c_conin ()

{

}

Parameter Block:

+--------+--------+

0 | function code 01|

+--------+--------+

Return Values:

Register D0.L:

(in least significant byte)

Description     C_CONIN reads a character from the  standard  input  device

and echoes it to the standard output device.

If  a  GEM...  VDI compatible console is the standard input

device, C_CONIN places the console scan  code  in  the  low

byte of the high word returned in D0.  Otherwise, this byte

is set to 00.  See Figure 4-2.

If  C_CONIN  reads  a  Ctrl-C  character,  it  flushes  the

type-ahead buffer and executes a warm boot operation.

Hb Hw    Lb Hw    Hb Lw    Lb Lw

+--------+--------+--------+--------+

|   00   |  code  |   00   |  char  |

|        | or  00 |        |        |

+--------+--------+--------+--------+

Figure 4-2.  D0 Contents Returned from C_CONIN

Function 02:  Write Character to Standard Ouput Device

C Interface:

VOID

c_conout (c)

WORD     c;

{

}

Parameter Block:

+--------+--------+

0 | function code 02|

+--------+--------+

2 |        c        |

+--------+--------+

Parameters:

c         Character to be displayed in lower 8 bits,

upper 8 bits should be set to zero

Description:    C_CONOUT  displays  the  character contained in the lower 8

bits of the c parameter on the standard output device.   To

ensure  compatibility  with future extensions to the 16-bit

character set, the upper 8 bits of c should be zero.

C_CONOUT translates tab characters into  columns  of  eight

characters.    This function also checks for Ctrl-S (scroll

stop), Ctrl-Q (strat scroll), and Ctrl-P  (printer  switch)

and  processes  Ctrl-C, which aborts the operation and warm

boots the system.

Function 03:  Read Character from Standard Auxiliary Device

C Interface:

LONG

c_auxin ()

{

}

Parameter Block:

+--------+--------+

0 | function code 03|

+--------+--------+

Return Values:

Register D0.L:

Description:    C_AUXIN receives a character from the  auxiliary  port  and

returns it in D0.L.

Function 04:  Write Character to Standard Auxiliary Device

C Interface:

VOID

c_auxout (c)

WORD c;

{

}

Parameter Block:

+--------+--------+

0 | function code 04|

+--------+--------+

2 |        c        |

+--------+--------+

Parameters:

c         Character to be sent to the auxiliary port in

lower 8 bits, upper 8 bits should be set to zero

Description:

C_AUXOUT  sends the character contained in the lower 8 bits

of the c parameter  to  the  auxiliary  port.    To  ensure

compatibility   with   future   extensions  to  the  16-bit

character set, the upper 8 bits of c should be zero.

Function 05:  Write Character to Standard Print Device

C Interface:

VOID

c_prnout (c)

WORD c;

{

}

Parameter Block:

+--------+--------+

0 | function code 05|

+--------+--------+

2 |        c        |

+--------+--------+

Parameters:

c         Character to be sent to the standard printer

device in lower 8 bits, upper 8 bits should be set to zero

Description:

C_PRNOUT sends the character contained in the lower 8  bits

of  the  c  parameter  to  the standard printer device.  To

ensure compatibility with future extensions to  the  16-bit

character set, the upper 8 bits of c should be zero.

Function 06:  Raw I/O to Standard Console

C Interface:

LONG

c_rawio (parm)

WORD    parm;

{

}

Parameter Block:

+--------+--------+

0 | function code 06|

+--------+--------+

2 |      parm       |

+--------+--------+

Parameters:

parm      Specifies the operation to be performed:

FF   To read a character from StdIn

char Character to be sent to StdOut

Return Values:

Register D0.L:

(param = FF)

0 Character not available

(parm = FF)

Description:    If  parm is FF, C_RAWIO reads a character from the standard

input device and returns it it D0.W.  If a character is not

available from the standard input device D0.L returns zero.

If  parm  is  not  FF,  C_RAWIO assumes the word contains a

character to be sent to the standard output device.

Function 07:  Raw Input from Standard Input Device

C Interface:

LONG

c_rawcin ()

{

}

Parameter Block:

+--------+--------+

0 | function code 07|

+--------+--------+

Return Values:

Register D0.L:

Description:    C_RAWCIN reads a character from the standard  input  device

and  returns  it  in  D0.L  without  echoing it to standard

output device.

C_RAWCIN does not perform a check for control characters.

Function 08:  Read Character from Standard Input, No Echo

C Interface:

LONG

c_necin ()

{

}

Parameter Block:

+--------+--------+

0 | function code 08|

+--------+--------+

Return Values:

Register D0.L:

Description:    C_NECIN reads a character from the  standard  input  device

and  returns  it  in  D0.L  without  echoing it to standard

output device.

Control characters are interpreted  by  this  function  and

have  their  proper  effect; see "Line Editing Commands" in

Section 1 of the GEM DOS User's Guide.

Function 09:  Write String to Standard Output Device

C Interface:

VOID

c_conws (p)

BYTE     *p;

{

}

Parameter Block:

+--------+--------+

0 | function code 09|

+--------+--------+

2 |                 |

+        p        +

4 |                 |

+--------+--------+

Parameters:

p         Points to a null-terminated string of characters

to be written to the standard output device

Description:    C_CONWS  transmits  the  null-terminated  character  string

pointed to by p to the standard output device, character by

character.  This  function  also  processes  Ctrl-C,  which

aborts the operation and warm boots the system.

Function 0A:  Read Edited String from Standard Input Device

C Interface:

VOID

c_conrs ()

BYTE     *p;

{

}

Parameter Block:

+--------+--------+

0 | function code 0A|

+--------+--------+

2 |                 |

+        p        +

4 |                 |

+--------+--------+

Parameters:

p         Points to an input buffer

Description:    C_CONRS  reads  characters  from the caller's default input

device and writes them to the input buffer  pointed  to  by

the  p  parameter.  The format of the input buffer is shown

in Figure 4-3.

0        1        2                          MAX - 1

|--------+--------+--------+--------+ - - - +--------|

|  MAX   | NCHARS | CHARACTERS . . .|       |   0DH  |

|--------+--------+--------+--------+ - - - +--------|

Figure 4-3.  C_CONRS Occurrence

Byte  0  of  the  input  buffer  specifies  the  number  of

characters the buffer can hold.  C_CONRS sets byte 1 to the

number of characters received.  This value does not include

the terminator character, a carriage return (0DH).  C_CONRS

writes the characters it  reads  from  the  standard  input

device  into  the buffer beginning at the third byte.  When

the system call reads a carriage return, it  stops  writing

to the buffer.

When  the  input buffer fills to one less than the value of

byte 0, C_CONRS ignores additional  input  and  sounds  the

bell until it reads a carriage return.

C_CONRS  performs  line-editing functions on the characters

as they are  read  from  the  console  (Backspace,  Ctrl-E,

Ctrl-Q,  Ctrl-R,  Ctrl-S,  Ctrl-U,  Ctrl-X)  and performs a

warmboot on Ctrl-C input.

Function 0B:  Check Status of Standard Input Device

C Interface:

LONG

c_conis (int)

{

}

Parameter Block:

+--------+--------+

0 | function code 0B|

+--------+--------+

Return Values:

Register D0.L:

0 If no character available

Description:    C_CONIS checks the status of the  caller's  standard  input

device to determine if a character is ready to be received.

Function 10:  Check Status of Standard Output Device

C Interface:

LONG

c_conos ()

{

}

Parameter Block:

+--------+--------+

0 | function code 10|

+--------+--------+

Return Values:

Register D0.L:

0 If console unavailable

Description:    C_CONOS checks the status of the caller's console device to

determine if it is ready to receive a character.

This function  also  processes  Ctrl-C,  which  aborts  the

operation and warm boots the system.

Function 11:  Check Status of Standard Print Device

C Interface:

LONG

c_prnos ()

{

}

Parameter Block:

+--------+--------+

0 | function code 11|

+--------+--------+

Return Values:

Register D0.L:

0 If printer unavailable

Description:    C_PRNOS   checks  the  status  of  the  printer  device  to

determine if it is ready to receive a character.

Function 12:  Check Status of Standard Auxiliary Device Input

C Interface:

LONG

c_auxis ()

{

}

Parameter Block:

+--------+--------+

0 | function code 12|

+--------+--------+

Return Values:

Register D0.L:

0 If no character available

Description     C_AUXIS checks the input status of  the  caller's  standard

auxiliary device to determine if a character is ready to be

received.

Function 13:  Check Status of Standard Auxiliary Device Output

C Interface:

LONG

c_auxos ()

{

}

Parameter Block:

+--------+--------+

0 | function code 13|

+--------+--------+

Return Values:

Register D0.L:

0 If it is unavailable

Description:    C_AUXOS checks the status of the caller's auxiliary  device

to determine if it is ready to receive a character.

4.5  FILE ACCESS FUNCTIONS

Table  4-6 lists the number, mnemonic, and name of the GEM DOS system calls

described in this section.

Table 4-6.  File Access Function Calls

Hex     Mnemonic

1A      F_SETDTA

2F      F_GETDTA

3C      F_CREATE

3D      F_OPEN

3E      F_CLOSE

3F      F_READ

40      F_WRITE

41      F_DELETE

42      F_SEEK

43      F_ATTRIB

45      F_DUP

46      F_FORCE

4E      F_SFIRST

4F      F_SNEXT

56      F_RENAME

57      F_DATIME

Function 1A:  Set Disk Transfer Address

C Interface:

VOID

f_setdta (b)

BYTE *b;

{

}

Parameter Block:

+--------+--------+

0 | function code 1A|

+--------+--------+

2 |                 |

+       addr      +

4 |                 |

+--------+--------+

Parameters:

addr      Address of DTA buffer

Description:    The  44-byte  buffer  whose  location  is  specified   with

F_SETDTA  may  begin  on  an even or odd address.  F_SFIRST

(4E) and F_SNEXT (4F) use the DTA  to  return  file  search

information.

If  you do not set the DTA, GEM DOS uses the default DTA at

offset 80 in the Base Page.    F_GETDTA  (2F)  returns  the

current location of the DTA buffer.

Function 2F:  Get Disk Transfer Address

C Interface:

LONG

f_getdta ()

{

}

Parameter Block:

+--------+--------+

0 | function code 2F|

+--------+--------+

Return Values:

Register D0.L:

Description:    F_GETDTA  returns  the current address of the DTA buffer in

D0.L.  You can set the DTA by calling F_SETDTA (1A).   Note

that  only  F_SFIRST  (4E)  and F_SNEXT (4F) use the DTA to

return file search information.

Function 3C:  Create a File

C Interface:

LONG

f_create (name,attr)

BYTE        *name;

WORD        attr;

{

}

Parameter Block:

+--------+--------+

0 | Function code 3C|

+--------+--------+

2 |                 |

+       name      +

4 |                 |

+--------+--------+

6 |       attr      |

+--------+--------+

Parameters:

name      Points to a null-terminated string that specifies

the drive, path, and name of the file to be created

attr      Specifies the file's attributes:

01   Read-only

02   Hidden

04   System, Hidden

08   Entry is a volume label

Return Values:

Register D0.L:

-34L (Path not found)

-35L (No handles left)

-36L (Access Denied)

Description:    F_CREATE  returns  error  code  -36L  (Access  Denied)   to

indicate  that  a  file with the name specified in the name

string already exists or that the directory is full.

Function 3D:  Open a File

C Interface:

LONG

f_open (pname,mode)

BYTE    *pname;

WORD mode;

{

}

Parameter Block:

+--------+---------+

0 | function code  3D|

+--------+---------+

2 |                  |

+      pname       +

4 |                  |

+--------+---------+

6 |       mode       |

+--------+---------+

Parameters:

pname     Points to a null-terminated string that specifies

the drive, path, and name of the file or device to open

mode      File access mode:

0    Read access only

1    Write access only

2    Read/Write access

Return Values:

Register D0.L:

-33L (File not found)

-35L (No handles left)

-36L (Access Denied)

Description:    F_OPEN positions the file pointer at the beginning  of  the

file.    You  can  change  the  pointer's location with the

F_SEEK (42) function call.

Use the handle returned by F_OPEN to refer to the file  for

subsequent  read,  write,  seek, and close operations.  You

can use F_ATTRIB (43)  to  get  the  file's  attribute  and

F_DATIME to get or set its date and time stamps.

Function 3E:  Close a File

C Interface:

LONG

f_close (handle)

WORD      handle;

{

}

Parameter Block:

+--------+--------+

0 | Function code 3E|

+--------+--------+

2 |      handle     |

+--------+--------+

Parameters:

handle    File handle of file to be closed

Return Values:

Register D0.L:

-37L (Invalid handle)

Description:    The  handle  you specify in the parameter block on entry to

F_CLOSE should be  one  that  was  previously  returned  by

F_OPEN (3D).

All  disk  buffers associated with the file are flushed and

the file's directory entry is updated on successful  return

from this call.

Function 3F:  Read from a File

C Interface:

LONG

f_read (handle, cnt, pbuffer)

WORD       handle;

LONG       cnt;

BYTE       *pbuffer;

{

}

Parameter Block:

+--------+--------+

0 | Function code 3F|

+--------+--------+

2 |      handle     |

+--------+--------+

4 |                 |

+       cnt       +

6 |                 |

+--------+--------+

8 |                 |

+     pbuffer     +

A |                 |

+--------+--------+

Parameters:

handle    File handle (returned from F_OPEN) of file to be read

cnt       Number of bytes to be read

pbuffer   Points to the buffer into which the file's contents

are to be read

Return Values:

Register D0.L:

-37L (Invalid handle)

Function 40:  Write to a File

C Interface:

LONG

f_write (handle, cnt, pbuffer)

WORD       handle;

LONG       cnt;

BYTE       *pbuffer;

{

}

Parameter Block:

+--------+--------+

0 | function code 40|

+--------+--------+

2 |      handle     |

+--------+--------+

4 |                 |

+       cnt       +

6 |                 |

+--------+--------+

8 |                 |

+     pbuffer     +

A |                 |

+--------+--------+

Parameters:

handle    File handle (returned from F_OPEN)

cnt       Number of bytes to be written

pbuffer   Points to the buffer that contains

the bytes to be written

Return Values:

Register D0.L:

-36L (Access Denied)

-37L (Invalid handle)

Function 41:  Delete a File

C Interface:

LONG

f_delete (name)

BYTE        *name;

{

}

Parameter Block:

+--------+--------+

0 | function code 41|

+--------+--------+

2 |                 |

+       name      +

4 |                 |

+--------+--------+

Parameters:

name      Points to a null-terminated string that specifies the

drive, path, and name of the directory entry to be deleted

Return Values:

Register D0.L:

-33L (File not found)

-36L (Access Denied)

Function 42:  Seek File Pointer

C Interface:

LONG

f_seek (softs, handle, smode)

LONG     softs;

WORD     handle;

WORD     smode;

{

}

Parameter Block:

+--------+--------+

0 | function code 42|

+--------+--------+

2 |                 |

+      softs      +

4 |                 |

+--------+--------+

6 |      handle     |

+--------+--------+

8 |      smode      |

+--------+--------+

Parameters:

softs     A signed value, N, used to indicate byte offset

negative values are useful in modes 1 and 2 below

+N moves pointer toward end of file, -N toward

beginning of file

handle    File handle (returned from F_OPEN)

smode     Indicates how the file pointer is to be moved:

0  Move pointer N bytes from the beginning of the file

1  Move pointer N bytes from its current location

2  Move pointer N bytes from the end of the file

Return Values:

Register D0.L:

from beginning of file)

-37L (Invalid handle)

-32L (Invalid function)

Function 43:  Get/Set File Attributes

C Interface:

LONG

f_attrib (p, wrt, mod)

BYTE     *p;

WORD     wrt,mod;

{

}

Parameter Block:

+--------+--------+

0 | function code 43|

+--------+--------+

2 |                 |

+        p        +

4 |                 |

+--------+--------+

6 |       wrt       |

+--------+--------+

8 |       mod       |

+--------+--------+

Parameters:

p         Points to a null-terminated string that contains the

drive, path, and file name

wrt       Specifies the operation to be performed:

0    Return the file's attributes in D0.L

1    Set the file's attributes according to mod

mod       Indicates file attributes:

01H  Read-only

02H  Hidden from directory search

04H  System, hidden from directory search

08H  Entry contains the volume label

10H  Entry is a subdirectory

20H  Archive (file written to and closed)

Return Values:

Register D0.L:

-33L (File not found)

-34L (Path not found)

Function 44:  I/O Control

C Interface:

LONG

f_ioctl (subfun, hndrv, cntval, bufptr)

WORD subfun, hndrv, cntval;

LONG bufptr;

{

}

Parameter Block:

+--------+--------+

0 | function code 44|

+--------+--------+

2 |      subfun     |

+--------+--------+

4 |      handrv     |

+--------+--------+

6 |      cntval     |

+--------+--------+

8 |                 |

+      bufptr     +

A |                 |

+--------+--------+

Parameters:

subfun    F_IOCTL subfunction code value as follows:

-1   Substitute interrupt vector

0    Get device information

1    Reserved

2    Read from device control channel

3    Write to device control channel

4    Read from drive control channel

5    Write to drive control channel

6    Get input status

7    Get output status

8    Get media type

hndrv     Handle or drive code, subfunction dependent.  See

Table 4-7, below.

cntval    Byte count or bit map of values, subfunction dependent.

bufptr    Pointer to buffer or, for subfunction -1, the address of

device's interrupt vector.

Return Values:

Device's current interrupt vector for subfunction -1

Bit map of device information for subfuction 0

Number of bytes transferred for subfunctions 2-5

Disk file or character device input status for subfunction 6

Disk file or character device output status for subfunction 7

Media type for subfunction 8

Description:    On  entry  to  F_IOCTL,  offset  02  of the parameter block

contains a  subfunction  code  value  that  determines  the

operation  to  be  performed.   Other entry parameters vary

according to the subfunction you call.  Table 4-7 lists the

F_IOCTL  subfunction calling restrictions.  Descriptions of

the individual subfunctions follow the table.

Table 4-7.  F_IOCTL Calling Restrictions

Character              Disk

Subfunction  Device     Standard      File    Drive           Byte

Number     Handle  Handle  Handle  Number          Count/Buffer

-1         X

0         X       X       X

2         X       X                        X

3         X       X                        X

4                                 X        X

5                                 X        X

6         X       X       X

7         X       X       X

8                                 X

Subfunction -1: Specify a character device handle at offset 04, 0 at offset

06,  and  the  address of the device's interrupt handler at

offset 08 of the F_IOCTL parameter block.   Subfunction  -1

returns  the  current interrupt vector for the device whose

handle you specify.  Note  that  only  a  character  device

handle can be specified for this subfunction.

Subfunction 0:  Specify a disk file or character device handle at offset 04

of the F_IOCTL parameter block.  Subfunction 0 returns  the

bit  map  of  device  information  shown  below or an error

number (-35L, -37L) in register D0.L.

If bit 7 = 0:

Bits   Meaning

0-5    Drive number

6     Reserved

7     0 = Disk file

8-D    Reserved

E     1 = Device can process

control strings

F     Reserved

If bit 7 = 1:

Bits   Meaning

0     1 = CON: input (stdin)

1     1 = CON: output (stdout)

2     1 = NUL:

3     1 = CLK:

4-6    Reserved

7     1 = Character device

8-D    Reserved

E     1 = Device can process

control strings

F     Reserved

Subfunction 2:  Specify the handle of a character device at offset  4,  the

number  of bytes to be read at offset 6, and the address of

the  buffer  into  which  this  subfunction  is   to   read

characters from the device's control channel at offset 8 of

the F_IOCTL parameter block.

Subfunction 2 returns the number of bytes read or an  error

number in register D0.L.

Subfunction 3   Specify  the  handle  of a character device at offset 4, of

the number of bytes to be written to the  device's  control

channel  at  offset  6,  and the address of the buffer from

which the bytes are to written at offset 8 of  the  F_IOCTL

parameter block.

Subfunction  3  returns  the number of bytes written to the

device control channel or an error number in register D0.L.

Subfunction 4   Specify the number of the drive at offset 4, the number  of

bytes to be read from the drive's control channel at offset

6, and the address of a buffer at offset 8 of  the  F_IOCTL

parameter block.

The drive number can be 0 for  the  default  drive,  1  for

drive  A, and so on up to the number of drives supported by

your particular system.

Subfunction 4 returns the number of  bytes  read  from  the

drive's control channel or an error number in register D0.L

Subfunction 5   Specify  the number of the drive at offset 4, the number of

bytes to written to the drive's control channel  at  offset

6, and the buffer from which the bytes are to be written at

offset 8 of the F_IOCTL parameter block.

Subfunction 5 returns the number of bytes  written  to  the

drive's  control  channel  or  an  error number in register

D0.L.

Subfunction 6   Specify the handle of a disk file  or  device  whose  input

status  you  want  returned  at  offset  4  of  the F_IOCTL

parameter block.  If the handle you specify is a disk file,

subfunction 6 returns zero in D0.L when the file pointer is

at EOF, or a non-zero  value.    If  the  file  pointer  is

repositioned after reaching EOF, this subfunction returns a

non-zero value again.

If the handle you specify is for a  device,  subfunction  6

returns a non-zero value in D0.L to indicate a ready state,

or zero to indicate that the device is not ready.

Subfunction 7   Specify the handle of a disk file or  device  whose  output

status  you  want  returned  at  offset  4  of  the F_IOCTL

parameter block.  If the handle you specify is for  a  disk

file,  subfunction  7  returns  zero  in D0.L when the file

pointer is at EOF, or a non-zero value.  If the pointer  is

repositioned  after  reaching EOF, this subfunction returns

non-zero again.

If the handle you specify is for a  device,  subfunction  7

returns  a  non-zero  value to indicate a ready state, or a

zero value to indicate that the device  is  not  ready  for

output.

Subfunction 8   Specify  the  number  of  the  drive  whose  media type you

returned at offset 4 of the F_IOCTL parameter  block.    If

the  drive  number  you  specify is valid, this subfunction

returns zero to indicate that the drive contains  removable

media  or  a  non-zero  value  to  indicate  that the drive

contains fixed media.

The drive number you specify  may  be  0  for  the  default

drive,  1 for drive A, and so on up to the number of drives

supported by your particualr system.

Function 45:  Duplicate a File Handle

C Interface:

LONG

f_dup (stdhnd)

WORD stdhnd;

{

}

Parameter Block:

+--------+--------+

0 | function code 45|

+--------+--------+

2 |     stdhnd      |

+--------+--------+

Parameters:

stdhnd    Standard file handle

Return Values:

Register D0.L:

-35L (No handles left)

-37L (Invalid handle)

Description:    F_DUP accepts a standard file handle as input and returns a

non-standard  handle that refers to the same file or device

at the same position.

If you use F_SEEK  (42)  to  change  the  location  of  the

pointer  for either handle, the pointer of its duplicate is

also moved.

See Table 4-2 for the GEM DOS standard handle values.

Function 46:  Force File Handle

C Interface:

LONG

f_force (stdhnd, nsthnd)

WORD stdhnd, nsthnd;

{

}

Parameter Block:

+--------+--------+

0 | function code 46|

+--------+--------+

2 |     stdhnd      |

+--------+--------+

4 |     nsthnd      |

+--------+--------+

Parameters:

stdhnd    Standard file handle

nsthnd    Non-standard file handle

Return Values:

Register D0.L:

-37L (Invalid handle)

Description:    F_FORCE forces the  standard  device  handle  specified  at

offset  02 of the parameter block to point to the same file

or device as the non-standard handle  specified  at  offset

04.

If  the  file  whose  handle you specify in stdhnd is open,

F_FORCE closes before returning.  If you use F_SEEK (42) to

change  the  location of the pointer for either handle, the

pointer of its duplicate is also moved.

See Table 4-2 for the GEM DOS standard handle values.

Function 4E:  Search for First Occurrence of Filespec

C Interface:

LONG

f_sfirst (pspec, attr)

BYTE      *pspec;

WORD      attr;

{

}

Parameter Block:

+--------+--------+

0 | function code 4E|

+--------+--------+

2 |                 |

+      pspec      +

4 |                 |

+--------+--------+

6 |      attr       |

+--------+--------+

Parameters:

pspec     Points to a null-terminated string that specifies the drive,

path and name of the file to search for; only the file

name portion of pspec may contain * or ? wildcards

attr      Specifies the search attributes:

00H  File is normal file entry

01H  File is read-only

02H  File is hidden from directory search

04H  File is system file

08H  Entry is a volume label

10H  Entry is a subdirectory

20H  File's archive bit is set

Return Values:

Register D0.L:

-33L (File not found)

-49L (No more files)

Description:    F_SFIRST  searches  for  the  first  directory  entry  that

matches pspec according to the value of attr.

F_SFIRST uses the value of attr in the following manner:

*   If  attr  is  00H,  the  search is restricted to normal

directory entries.  F_SFIRST does  not  return  entries

for  volume  labels,  subdirectories,  hidden files, or

system files.

*   If attr is set to the hidden (02H) or system (04H) file

values,  they  are  included with the normal entries in

the search set.  You can use F_SFIRST to search for all

directory  entries except volume labels by setting attr

for hidden, system, and directory.

*   If attr  is  set  to  the  volume  label  value  (08H),

F_SFIRST restricts its search to volume labels.

If  F_SFIRST finds a directory entry that matches pspec and

attr, it formats the DTA as shown in Table 4-8.    F_GETDTA

(2F) returns the current location of the DTA buffer.

Table 4-8.  DTA Contents from F_SFIRST Call

DTA Offset    Contents

0 - 20      Reserved for OS use

21 File's attributes

22 - 23     File's time stamp

24 - 25     File's date stamp

26 - 29     Longword of file's size

30 - 43     File's name and extension

Function 4F:  Search for Next Occurrence of Filespec

C Interface:

LONG

f_snext()

{

}

Parameter Block:

+--------+--------+

0 | function code 4F|

+--------+--------+

Return Values:

Register D0.L:

-49L (No more files)

Description     F_SNEXT   uses  the  parameters  specified  in  a  previous

F_SFIRST (4E) or F_SNEXT call to locate the  next  matching

directory  entry.    Offsets 0 to 20 of the DTA must remain

untouched from the previous SFIRST or SNEXT call.

If F_NEXT locates a matching entry, it updates the contents

of  the  DTA buffer initialized by the previous F_SFIRST or

F_NEXT call as shown in Table 4-8, above.

Function 56:  Rename a File

C Interface:

LONG

f_rename (res, p1, p2)

WORD   res;

BYTE   *p1;

BYTE   *p2;

{

}

Parameter Block:

+--------+--------+

0 | function code 56|

+--------+--------+

2 |    Must be 0    |

+--------+--------+

4 |                 |

+        p1       +

6 |                 |

+--------+--------+

8 |                 |

+        p2       +

A |                 |

+--------+--------+

Parameters:

p1        Points to a null-terminated string that contains the

drive, path, and name of the file to be renamed

p2        Points to a null-terminated string that contains the

path and new name for the file specified in p1

Return Values:

Register D0.L:

-34L (Path not found)

-36L (Access denied)

Description:    The word at offset 2 in the parameter block is reserved for

system  use  and  must be zero.  The file name specified in

the p2 string must not exist.  If you specify  a  drive  in

the  p2  string, it must be the same drive specified in the

string pointed to by p1.

You can specify a different directory path in the p2 string

so   that   the   operation   moves  the  file  to  another

subdirectory on the same drive.

F_RENAME returns -36L in D0 if the read/only  attribute  of

the file to be renamed is set.

Function 57:  Get/Set File Date and Time Stamps

C Interface:

VOID

f_datime (buf, h, set)

BYTE     *buff;

WORD     h;

WORD     set;

{

}

Parameter Block:

+--------+--------+

0 | function code 57|

+--------+--------+

2 |                 |

+       buff      +

4 |                 |

+--------+--------+

6 |        h        |

+--------+--------+

8 |       set       |

+--------+--------+

Parameters:

buff      Points to a buffer that contains the time in the first

word and the date in the second word

h         Contains the file handle returned from F_OPEN

set       A flag that specifies the operation to be performed:

0    Set date and time

1    Return date and time to buff

Description:    The  format  of  the  date and time contained in the buffer

pointed to by buff is as follows:

Time (word 0):

Bits 00 - 04

Bits 05 - 10

Bits 11 - 15

Date (word 1):

Bits 00 - 04

Bits 05 - 08

Bits 09 - 15

4.6  DRIVE AND DIRECTORY MANAGEMENT FUNCTIONS

Table 4-9 lists the number, mnemonic, and name  of  the  GEMDOS  drive  and

directory management functions.

Table 4-9.  Drive and Directory Management Functions

Hex     Mnemonic

0E      D_SETDRV

19      D_GETDRV

36      D_FREE

39      D_CREATE

3A      D_DELETE

3B      D_SETPATH

47      D_GETPATH

Funtion 0E:  Set Default Drive

C Interface:

LONG

d_setdrv (newdrv)

WORD newdrv;

{

}

Parameter Block:

+--------+--------+

0 | function code 0E|

+--------+--------+

2 |      newdrv     |

+--------+--------+

Parameters:

newdrv    Specifies the number for the current drive:

0    Drive A

1    Drive B

3    Drive C

.

.

.

15   Drive P

Return Values:

Register D0.L:

Bit  0 = A

Bit  1 = B

Bit  2 = C

.        .

.        .

.        .

Bit 15 = p

Description:    D_SETDRV  designates the drive whose number is specified at

offset 04 in the parameter block as the current  drive  and

returns a bit map of the system drives in D0.L.

Function 19:  Get Default Drive

C Interface:

LONG

d_getdrv ()

{

}

Parameter Block:

+--------+--------+

0 | function code 19|

+--------+--------+

Return Values:

Register D0.L:

Description:    D_GETDRV  returns  the number of the current drive in D0.L,

where: 0 = A, 1 = B, 2 = C, up to 15 = P

Function 36:  Get Drive Free Space

C Interface:

LONG

d_free (pbuffer,dr)

BYTE *pbuffer

WORD dr;

{

}

Parameter Block:

+--------+--------+

0 | function code 36|

+--------+--------+

2 |                 |

+     pbuffer     +

4 |                 |

+--------+--------+

6 |       dr        |

+--------+--------+

Parameters:

pbuffer   Points to the buffer into which D_FREE returns

drive information

dr        Specifies the drive code where:

0 = Current Drive, 1 = Drive A, 2=Drive B, etc.

Return Values:

Register D0.L:

-46L (Invalid drive specified)

Description:    The  format  of  the  buffer  pointed  to  by  pbuffer   on

successful return from D_FREE is shown in Figure 4-4.

+--------+--------+--------+--------+

0 |   Number of Free Bytes on Drive   |

+--------+--------+--------+--------+

4 |   Number of Available Clusters    |

+--------+--------+--------+--------+

8 |          Bytes per Sector         |

+--------+--------+--------+--------+

C |  Number of Sectors per Cluster    |

+--------+--------+--------+--------+

Figure 4-4.  D_FREE Information Return Buffer

Function 39:  Create a Subdirectory

C Interface:

LONG

d_create (path),

BYTE   *path;

{

}

Parameter Block:

+--------+--------+

0 | function code 39|

+--------+--------+

2 |                 |

+       path      +

4 |                 |

+--------+--------+

Parameters:

path      Points to a null-terminated string that contains the

drive and pathname of the subdirectory to be created

Return Values:

Register D0.L:

-34L (Path not found)

-36L (Access denied)

Description:    D_CREATE  does not change the directory path if any element

of the pathname contained  in  the  path  string  does  not

exist.  If D0.L contains 0 on return from D_CREATE, the new

subdirectory is created at the end of the pathname.

Function 3A:  Delete a Subdirectory

C Interface:

LONG

d_delete (path)

BYTE    *path;

{

}

Parameter Block:

+--------+--------+

0 | function code 3A|

+--------+--------+

2 |                 |

+       path      +

4 |                 |

+--------+--------+

Parameters:

path      Points to a null-terminated string that contains the

drive and pathname of the subdirectory to be deleted

Return Values:

Register D0.L:

-34L (Path not found)

-36L (Access denied)

-65L (Internal error)

Description:    If the subdirectory to be deleted is  not  empty,  D_DELETE

returns  -36L  (Access  denied).   D_DELETE also returns an

error code if you attempt to delete  the  current  or  root

directories.

Function 3B:  Set Current Directory

C Interface:

LONG

d_setpath (path)

BYTE       *path;

{

}

Parameter Block:

+--------+--------+

0 | function code 3B|

+--------+--------+

2 |                 |

+       path      +

4 |                 |

+--------+--------+

Parameters:

path      Points to a null-terminated string that contains the

drive and pathname of the directory to be made current

Return Values:

Register D0.L:

-34L (Path not found)

Description:    If  D0.L  contains  0 on return from D_SETPATH, the current

directory is set to correspond with the path string.

Function 47:  Get Current Directory

C Interface:

LONG

d_getpath (pathbuf, drive)

BYTE    *pathbuf;

WORD    drive;

{

}

Parameter Block:

+--------+--------+

0 | function code 47|

+--------+--------+

2 |                 |

+     pathbuf     +

4 |                 |

+--------+--------+

6 |      drive      |

+--------+--------+

Parameters:

pathbuf   Points to a 64-byte buffer into which D_GETPATH writes

the complete pathname of the current directory

drive     Contains the drive code where:

0 = current drive, 1 = drive A, 2 = drive B, etc.

Return Values:

Register D0.L:

-46L (Invalid drive)

Description:    D_GETPATH writes the pathname (beginning from the root)  of

the  current  directory  for  the  drive  specified  in the

parameter block into the null-terminated buffer pointed  to

by pathbuf.  D_GETPATH does not include the drive letter in

the  pathname  nor  does  it  begin  the  pathname  with  a

backslash (\).

4.7  PROGRAM CONTROL FUNCTIONS

Table 4-10 lists the number, mnemonic, and name of the GEM DOS system calls

described in this section.

Table 4-10.  System and Program Control Function Calls

Hex     Mnemonic

00      P_TERM0

25      S_SETVEC

30      S_VERSION

31      P_TERMRES

35      S_GETVEC

4B      P_EXEC

4C      P_TERM

Function 00:  Program Terminate

C Interface:

VOID

p_term0 ()

{

}

Parameter Block:

+--------+--------+

0 | function code 00|

+--------+--------+

Description:    P_TERM0 terminates the calling process and returns  to  the

parent process with a return code of 0.

Function 25:  Set Exception Vector

C Interface:

LONG

s_setvec (vecnum, address)

WORD vecnum;

LONG address;

{

}

Parameter Block:

+--------+--------+

0 | function code 25|

+--------+--------+

2 |     vecnum      |

+--------+--------+

4 |                 |

+     address     +

6 |                 |

+--------+--------+

Parameters:

vecnum    Vector number

address   New address for vector number

Return Values:

Register D0.L:

Description:    S_SETVEC  associates  a  vector  number  with the specified

address.

Function 30:  Get Version Number

C Interface:

WORD

s_version ()

{

}

Parameter Block:

+--------+--------+

0 | function code 30|

+--------+--------+

Return Values:

Register D0.W:

Description:    The lower byte of the word  returned  in  D0  contains  the

major  version  number;  the  upper byte contains the minor

version number.  0001 = version 1.00.

Function 31:  Terminate and Stay Resident

C Interface:

VOID

p_termres (n_bytes,rc)

LONG n_bytes

WORD rc;

{

}

Parameter Block:

+--------+--------+

0 | function code 31|

+--------+--------+

2 |                 |

+     n_bytes     +

4 |                 |

+--------+--------+

6 |        rc       |

+--------+--------+

Parameters:

n_bytes   Memory size in bytes

rc        Exit code

Description:    P_TERMRES allows  the  calling  process  to  terminate  the

currently  running  process  and leave the amount of memory

specified in n_bytes (offset 02) resident.

The rc field at offset 06 of the P_TERMRES parameter  block

contains  an  exit  code  that  is  returned  to the parent

process in register D0.

Note that your use of P_TERMRES  might  make  your  program

difficult  to  port  to  future  versions of this operating

system.

Function 35:  Get Exception Vector

C Interface:

LONG

s_setvec (vecnum)

WORD vecnum;

{

}

Parameter Block:

+--------+--------+

0 | function code 35|

+--------+--------+

2 |     vecnum      |

+--------+--------+

Parameters:

vecnum    Vector number

Return Values:

Register D0.L:

Description:    S_GETVEC returns the address associated with the  specified

vector number.

Function 4B:  Load or Execute a Process

C Interface:

LONG

p_exec (load, pcspec, pcmdln, penvstr)

WORD      load;

BYTE      *pcspec;

BYTE      *pcdln;

BYTE      *penvstr;

{

}

Parameter Block:

+--------+--------+

0 | function code 4B|

+--------+--------+

2 |      load       |

+--------+--------+

4 |                 |

+     pcspec      +

6 |                 |

+--------+--------+

8 |                 |

+      pcdln      +

A |                 |

+--------+--------+

C |                 |

+     penvstr     +

E |                 |

+--------+--------+

Parameters:

load      Indicates the type of load function to be performed:

0    Load and execute program

3    Load overlay, do not execute

pcspec    Points to a null-terminated string that specifies

the drive, path, and name of the file to be loaded

pcdln     Points to a command tail that includes redirection

information

penvstr   Points to the ASCII strings that describe the

configuration of the process's environment

Return Values:

Register D0.L:

on load/execute (load = 0)

Base page address

on load overlay (load=3)

-33L (File not found)

-39L (Insufficient memory)

-66L (Invalid program load format)

Description:    The  address  contained  in  the  pcdln  field is placed at

offset 80H in the base page of the program being loaded.

The process environment strings pointed to by  the  penvstr

field  at  offset  0C  in  the  P_EXEC  parameter block are

formatted as a series of null-terminated  strings.    These

environment  strings  are  terminated  by a null word.  The

address contained in the penvstr field is copied  into  the

p_env field at offset 2C of the base page.

The  child  process exit code returned by P_TERMRES in D0.L

can be specified at offset 02 in the parameter  block  used

to invoke Terminate Process (4CH).

See Section for a description of the base page format.

Function 4C:  Terminate Process

C Interface:

VOID

p_term (code)

WORD code;

{

}

Parameter Block:

+--------+--------+

0 | function code 4C|

+--------+--------+

2 |       code      |

+--------+--------+

Parameters:

code      Exit code returned to parent process from

Load or Execute a Process (4B).

Return Values:

Register D0.L:

Error code on failure

Description:    P_TERM terminates the current process and transfers control

to  the  calling  process.    All  files  opened   by   the

terminating process are closed.

4.8  MEMORY MANAGEMENT FUNCTIONS

This section describes the following GEM DOS function calls:

*   Allocate Memory (M_ALLOC, 48)

*   Free Allocated Memory (M_FREE, 49)

*   Shrink Size of Allocated Memory (M_SHRINK, 4A)

M_FREE  (49)  and  M_SHRINK  calls  require  the  address of a memory block

previously returned by M_ALLOC (48) as  a  parameter.    M_ALLOC  allocates

memory  in  units  of  bytes  and  can also be used to return the amount of

currently available memory.

The GEM DOS command line interpreter allocates all of available  memory  to

most of the programs that are loaded under its control.

Function 48:  Allocate Memory

C Interface:

LONG

m_alloc (nbytes)

LONG    nbytes;

{

}

Parameter Block:

+--------+--------+

0 | function code 48|

+--------+--------+

2 |                 |

+      nbytes     +

4 |                 |

+--------+--------+

Parameters:

nbytes    The number of bytes to allocate or

-1L (FFFFFFFF) to return the maximum

amount of available memory

Return Values:

Register D0.L:

block's starting address

Number of available bytes

(nbytes = FFFFFFFF)

0 on failure

Description:    If   the  nbytes  field  of  the  M_ALLOC  parameter  block

specifies the number of bytes of memory  to  allocate,  the

function  call returns a pointer to the starting address of

a memory block in D0.L.  If the allocation operation fails,

D0.L returns zero.

If  you  set  nbytes to -1L (FFFFFFFF), M_ALLOC returns the

maximum number of available bytes.

Function 49:  Free Allocated Memory

C Interface:

LONG

m_free (maddr)

LONG    maddr;

{

}

Parameter Block:

+--------+--------+

0 | function code 49|

+--------+--------+

2 |                 |

+      maddr      +

4 |                 |

+--------+--------+

Parameters:

maddr     Starting address of memory block to be freed

Return Values:

Register D0.L:

-40L (Invalid memory block address)

Description:    M_FREE returns  a  memory  block  previously  allocated  by

M_ALLOC  (48)  to  the  system.   If the maddr field of the

parameter block  does  not  specify  an  address  that  was

returned by a previous M_ALLOC call, M_FREE returns -40L in

D0.L to indicate that you have used an invalid memory block

address.

Function 4A:  Shrink Size of Allocated Memory

C Interface:

LONG

m_shrink (res, mp, size)

res;

BYTE *mp;

LONG size;

{

}

Parameter Block:

+--------+--------+

0 | function code 4A|

+--------+--------+

2 |    Must be 0    |

+--------+--------+

4 |                 |

+       mp        +

6 |                 |

+--------+--------+

8 |                 |

+       size      +

A |                 |

+--------+--------+

Parameters:

mp        Beginning address of memory block to be returned

size      Length, in bytes, of memory to be returned

Return Values:

-40L (Invalid memory block address)

-67L (Growth restriction failure)

Description:    The word at offset 2 in the parameter block is reserved for

system use and must be zero.   mp  must  specify  a  memory

block address that was previously returned by M_ALLOC (48).

If the value you pass to M_SHRINK in the size field of  the

parameter  block does not specify a reduction in the memory

block size, M_SHRINK returns -67L in D0.L

Use M_SHRINK to return memory that your  program  does  not

require  to  the  system for allocation to other processes.

Before calling M_SHRINK, move your  program's  stack  space

down  into  an  area  of  memory  that  you have previously

reserved and know  will  accomodate  your  program's  stack

size.

If  your  program  later  requires  additional memory, call

M_ALLOC to determine the amount of memory available (nbytes

= -1).  Then call M_ALLOC with nbytes set to the portion of

available memory your requires.

4.9  TIME FUNCTIONS

This section describes the Get Date (2A), Set Date (2B), Get Time (2C), and

Set Time (2D) functions.

Function 2A:  Get Date

C Interface:

WORD

t_getdate ()

{

}

Parameter Block:

+--------+--------+

0 | function code 2A|

+--------+--------+

Return Values:

Register D0.W:

Bits 00 - 04 Day, 1 - 31

Bits 05 - 08 Month, 1 - 12

Bits 09 - 15 Year (since 1980), 0 - 119

Function 2B:  Set Date

C Interface:

LONG

t_setdate (date)

WORD date;

{

}

Parameter Block:

+--------+--------+

0 | function code 2B|

+--------+--------+

2 |       date      |

+--------+--------+

Parameters:

date      Contains the date to be set in the format:

Bits 00 - 04 Day, 1 - 31

Bits 05 - 08 Month, 1 - 12

Bits 09 - 15 Year (since 1980), 0 - 119

Return Values:

Register D0.L:

-1L (Error) if date is not valid

Function 2C:  Get Time

C Interface:

WORD

t_gettime ()

{

}

Parameter Block:

+--------+--------+

0 | function code 2C|

+--------+--------+

Return Values:

Register D0.W:

Bits 00 - 04 Binary number of two-second increments

Bits 05 - 10 Binary number of minutes

Bits 11 - 15 Binary number of hours

Function 2D:  Set Time

C Interface:

LONG

t_settime (time)

WORD time;

{

}

Parameter Block:

+--------+--------+

0 | function code 2D|

+--------+--------+

2 |       time      |

+--------+--------+

Parameters:

time     Contains the time to be set in the format:

Bits 00 - 04 Binary number of two-second increments

Bits 05 - 10 Binary number of minutes

Bits 11 - 15 Binary number of hours

Return Values:

Register D0.L:

-1L (Error) if time is not valid

End of Section 4

AS68 ASSEMBLER

5.1  ASSEMBLER OPERATION

The  GEM DOS Assembler, AS68, assembles an assembly language source program

for execution on the 68000  microprocessor.    It  produces  a  relocatable

object file and, optionally, a listing.  You can find a summary of the AS68

instruction set at the end of this section.  Exceptions  and  additions  to

the standard Motorola instruction set appear in sections 5.6 and 5.7.

5.2  INITIALIZING AS68

If  the  file AS68SYM.DAT is not on your disk, you must create this file to

initialize AS68 before you can use AS68 to assemble files.   To  initialize

AS68, specify the AS68 command, the -I option, and the filename AS68INIT as

shown below.

{a}AS68 -I AS68INIT

AS68 creates the output file AS68SYMB.DAT,  which  AS68  requires  when  it

assembles  programs.  After you create this file, you need not specify this

command line again unless you reconfigure your system to have different TPA

boundaries.

5.3  INVOKING THE ASSEMBLER (AS68)

Invoke AS68 by entering a command with the following form:

AS68 [-F pathname] [-P] [-S pathname] [-U] [-L] [-N] [-I]

[-O object filename]

source pathname [>listing pathname]

Table 5-1 lists and describes the AS68 command line options.

Table 5-1.  Assembler Options

Option         Meaning

-F pathname    Specifies  the  directory  in  which the temporary files are

created.  If this option is not specified, AS68 creates  the

temporary files in the current default directory.

-I             Initializes the assembler.  See Section 5.2 for details.

-P             If  specificed,  AS68  produces  and prints a listing on the

standard output device, which, by default, is  the  console.

Redirect  the  listing,  including error messages, to a file

with the >listing  filename  parameter.    Note  that  error

messages  are  produced  whether  or  not  the  -P option is

specified.  No listing  is  produced,  however,  unless  you

specify the -P option.

-S pathname    Indicates   the   directory   that  contains  the  assembler

initialization file, AS68SYMB.DAT.   This  file  is  created

when  you initialize AS68.  AS68 reads the file AS68SYMB.DAT

before it assembles a source file.  If you  do  not  specify

this option, AS68 assumes the initialization file is located

in the current default directory.

-U             Causes all undefined symbols in the assembly to  be  treated

as global references.

-L             Ensures  all  address  constants are generated as longwords.

Use the -L option for programs that require  more  than  64K

for  execution  or  if the TPA is not contained in the first

64K bytes of memory.  If -L is not specified, the program is

assembled  to  run  in the first 64K bytes of memory.  If an

address in the assembly does not fit  within  one  word,  an

error occurs.

-N             Disables  optimization  of  branches  on forward references.

Normally, AS68 uses  the  2-byte  form  of  the  conditional

branch  and  the  4-byte  BSR  instruction wherever possible

(instead of the 6-byte JSR  instruction)  to  speed  program

execution and reduce instruction size.

-T             Enables AS68 to accept the 68010 microprocessor opcodes.

source filename

Specifies  the  file  to  assemble;  you  must  supply  this

parameter.

>listing filename

Sends a program listing to the standard output  device.  Use

the  greater-than symbol, >, to direct the listing to a disk

file.  The listing includes assembler error messages.   Note

that  if you do not specify -P with a listing filename, only

the error messages are redirected to the listing file.

5.4  ASSEMBLY LANGUAGE DIRECTIVES

Table 5-2 lists the AS68 directives.

Table 5-2.  AS68 Directives

Directive       Meaning

.comm label, expression

The  comm (common) directive specifies a label and the size

of a common area that  programs  assembled  separately  can

share.  The LO68 linker links all areas with the same label

to the same address.  The largest common area  of  a  group

with  the  same  label  determines  the  final  size of the

program common area.

.data           The data directive instructs AS68 to change  the  assembler

base segment to the data segment.

.bss            The bss (block storage segment) directive instructs AS68 to

change the assembler base  segment  to  the  block  storage

segment.   You cannot assemble instructions and data in the

bss.  However, you can define symbols and  reserve  storage

in the bss with the ds directive.

.dc operand[,operand,...]

The dc (define constant)  directive  defines  one  or  more

constants  in  memory.    The  operands  can  be symbols or

expressions assigned numeric values by  AS68,  or  explicit

numeric  constants in decimal or hexadecimal, or strings of

ASCII characters.  You must separate operands with  commas.

You  must  enclose  string  constants  in  single quotation

marks.  Each ASCII character is assigned  a  full  byte  of

memory.  The eighth bit is always 0.

You  can  specify the length of each constant with a single

letter parameter (byte = b, word = w, longword = l).    You

must separate the letter from the dc with a period as shown

in the following explanations.

.dc.b           The constants are byte constants.   If  you

specify  an odd number of bytes, AS68 fills

the odd byte on the right with zeros unless

the   next   statement   is   another  dc.b

directive.  When the next  statement  is  a

dc.b directive, the dc.b uses the odd byte.

Byte constants are not relocatable.

Table 5-2.  (continued)

Directive       Meaning

.dc.w           The constants are word constants.   If  you

specify  an odd number of bytes, AS68 fills

the last word on the right  with  zeros  to

force  an even byte count.  The only way to

specify an odd number of bytes is  with  an

ASCII  constant.    Word  constants  can be

relocated.

.dc.l           The constants are longword constants.    If

less  than  a  multiple  of  four  bytes is

entered, AS68 fills the  last  longword  on

the right with zeros to force a multiple of

four bytes.    Longword  constants  can  be

relocated.

.ds operand     The   define   storage   directive   (ds)  reserves  memory

locations.  The contents of the memory that it reserves  is

not  initialized.    The  operand  specifies  the number of

bytes, words, or longwords that  this  directive  reserves.

The  notation for these size specifications is shown below.

.ds.b           reserves memory locations in bytes

.ds.w           reserves memory locations in words

.ds.l           reserves memory locations in longwords

.end            The end directive informs AS68 that  no  more  source  code

follows  this  directive.    Code,  comments,  or  multiple

carriage returns cannot follow this directive.

.endc           The endc directive denotes the end  of  the  code  that  is

conditionally  assembled.  It is used with other directives

that conditionally assemble code.

.equ (or =) expression

The  equate  directive  (equ or =) assigns the value of the

expression in the operand field to the symbol in the  label

field  that  precedes  the directive.  The syntaxes for the

equate directive are

label .equ expression

label = expression

Table 5-2.  (continued)

Directive       Meaning

The label and operand fields are required.  The label  must

be  unique;  it  cannot  be  defined  anywhere  else in the

program.  The expression cannot include an undefined symbol

or  one  that is defined following the expression.  Forward

references to symbols are not allowed for this directive.

.even           The even directive increments the location counter to force

an  even  boundary.    For  example,  if specified when the

location  counter  is  odd,   the   location   counter   is

incremented  by  one  so  that the next instruction or data

field begins on an even boundary in memory.

.globl label[,label...]

.xdef label[,label...]

.xref label[,label...]

These directives make the label(s) external.  If the labels

are  defined  in the current assembly, this statement makes

them available to other routines during a load by LO68.  If

the  labels  are  not defined in the current assembly, they

become undefined external references, which LO68  links  to

external  values with the same label in other routines.  If

you  specify  the  -U  option,  the  assembler  makes   all

undefined labels external.

.ifeq expression

.ifne expression

.ifle expression

.iflt expression

.ifge expression

.ifgt expression

These  directives  test  an  expression  against zero for a

specified condition.  If the expression is true,  the  code

following is assembled; if false, the code is ignored until

an  end  conditional  directive  (endc)  is  found.     The

directives and the conditions they test are:

.ifeq           equal to zero

.ifne           not equal to zero

.ifle           less than or equal to zero

.iflt           less than zero

.ifge           greater or equal to zero

.ifgt           greater than zero

Table 5-2.  (continued)

Directive       Meaning

.ifc 'string1', 'string2'

.ifnc 'string1', 'string2'

The  conditional string directive compares two strings. The

'c' condition is true if the strings are exactly the  same.

The 'nc' condition is true if they do not match.

.offset expression

The offset directive creates a  dummy  storage  section  by

defining  a  table  of  offsets  with  the  define  storage

directive (ds).  The storage definitions are not passed  to

the  linker.    The  offset  table  begins  at  the address

specified in the expression.  Symbols defined in the offset

table  are  internally  maintained.    No  instructions  or

code-generating directives, except  the  equate  (equ)  and

register  mask  (reg) directives, can be used in the table.

The offset directive is terminated by one of the  following

directives:

bss

data

end

section

text

.org expression The  absolute  origin  directive  (org)  sets  the location

counter  to  the  value  of  the  expression.    Subsequent

statements  are assigned absolute memory locations with the

new value of the location counter.  The  expression  cannot

contain any forward, undefined, or external references.

.page           The page directive causes a page break which forces text to

print on the top of the next page.  It does not require  an

operand or a label and it does not generate machine code.

The  page directive allows you to set the page length for a

listing of code.  If you use this directive and  print  the

source code by specifying the -P option in the AS68 command

line, pages break at predefined rather than random  places.

The  page  directive does not appear on the printed program

listing.

Table 5-2.  (continued)

Directive       Meaning

.reg reglist    The register mask directive builds a register mask that can

be  used  by a movem instruction.  (See Table 1-1.)  One or

more registers can be listed  in  ascending  order  in  the

format:

R?[-R[/R?[-R?...]...]]

Replace   the  R  in  the  above  format  with  a  register

reference.  Any of the following mnemonics are valid:

A0-A7

D0-D7

R0-R15

The following example illustrates a sample register list.

A2-A4/A7/D1/D3-D5

You can also use commas to separate registers as follows:

A1,A2,D5,D7

.section #      The section directive defines a base segment.  The sections

can  be numbered from 0 to 15 inclusive.  Section 14 always

maps to data.  Section  15  is  bss.    All  other  section

numbers denote text sections.

.text           The  text  directive instructs AS68 to change the assembler

base segment to the text  segment.    Each  assembly  of  a

program begins with the first word in the text segment.

5.5  SAMPLE COMMANDS INVOKING AS68

{a}as68 -u -l test.s

This  command assembles the source file TEST.S and produces the object file

TEST.O.  Error messages appear on the screen.  Any  undefined  symbols  are

treated as global.

{a}as68 -p  smpl.s > smpl.l

This  command assembles the source file SMPL.S and produces the object file

SMPL.O.  The program must run in the first  64K  of  memory;  that  is,  no

address  can  be  larger  than 16 bits.  Error messages and the listing are

directed to the file SMPL.L.

5.6  ASSEMBLY LANGUAGE DIFFERENCES

The syntax differences between the AS68 assembly  language  and  Motorola's

assembly language are described in the following list.

*   In  AS68,  all assembler directives are optionally preceded by a period

(.).  For example,

.equ or equ

.ds or ds

*   AS68 does not support, but accepts and ignores the  following  Motorola

directives:

comline

mask2

idnt

opt

*   The  Motorola  .set  directive  is  implemented as the equate directive

(equ).

*   AS68  accepts  upper-  and  lowercase  characters.    You  can  specify

instructions  and  directives  in  either  case.    However, labels and

variables are case-sensitive.  For example, the label START  and  Start

are not equivalent.

*   For AS68, all labels must terminate with a colon (:).  For example,

A:

FOO:

However,  if  a  label begins in column 1, it need not terminate with a

colon.

*   For AS68, ASCII string constants can be enclosed in  either  single  or

double quotes.  For example,

'ABCD'

"ac14"

*   For AS68, registers can be referenced with the following mnemonics:

r0-r15

R0-R15

d0-d7

D0-D7

a0-a7

A0-A7

Upper-  and  lowercase  references are equivalent.  Registers R0-R7 are

the same as D0-D7 and R8-R15 are the same as A0-A7.

*   Use caution when manipulating the location counter forward in AS68.  An

expression  can  move  the  counter  forward only.  The unused space is

filled with zeros in the text or data segments.

*   For AS68, comment lines can begin  with  an  asterisk  followed  by  an

equals  sign  (*  =),  but only if one or more spaces exist between the

asterisk and the equals sign as follows:

* = This command loads R1 with zeros.

* = Branch to subroutine XYZ.

Be sure to include a space after the asterisk, as the location  counter

is manipulated with a statement of the form:

*=expr

*   For AS68, the syntax for short form branches is bxx.b rather than bxx.s

*   The  Motorola assembler supports a programming model in which a program

consists of a maximum of 16  separately  relocatable  sections  and  an

optional  absolute  section.  The AS68 distributed with GEMDOS does not

support this model.  Instead, AS68 supports a model in which a  program

contains three segments, text, data, and bss as described in Section 3,

"Command File Format."

5.7  ASSEMBLY LANGUAGE EXTENSIONS

The following enhancements have been added to AS68  to  make  the  assembly

language more efficient:

*   When  the  instructions  add,  sub,  and  cmp  are used with an address

register in the source or destination, they generate  adda,  suba,  and

cmpa.   When the clr instruction is used with an address register (Ax),

it generates sub Ax, Ax.

*   add, and, cmp, eor, or, sub are allowed with immediate  first  operands

and  generate addi, andi, cmpi, eori, ori, and subi instructions if the

second operand is not register-direct.

*   All  branch  instructions  generate  short  relative   branches   where

possible, including forward references.

*   Any  shift  instruction  with  no shift count specified assumes a shift

count of one.  For example, asl rl is equivalent to asl #1,rl.

*   A jsr instruction is changed to a bsr instruction if the resulting  bsr

instruction is shorter than the jsr instruction.

*   The   .text   directive   causes  the  assembler  to  begin  assembling

instructions in the text segment.    The  .data  directive  causes  the

assembler to begin assembling initialized data in the data segment.

*   The .bss directive instructs the assembler to begin defining storage in

the bss.  No instructions or constants can be placed in the bss because

the  bss  is  for uninitialized data only.  However, the .ds directives

can be used to define storage locations, and the location  counter  (*)

can be incremented.

*   The .globl directive in the form:

.globl label[,label] ...

makes  the  labels  external.    If  they  are  otherwise  defined  (by

assignment or appearance as a label),  they  act  within  the  assembly

exactly  as  if  the  .globl  directive  were not given.  However, when

linking this program with other programs, these symbols  are  available

to  other  programs.   Conversely, if the given symbols are not defined

within the current assembly, the linker can combine the output of  this

assembly with that of others which define the symbols.

*   The  common  directive  (comm)  defines  a  common region, which can be

accessed by programs that are assembled separately.  The syntax for the

common directive is

.comm label, expression

The  expression  specifies  the number of bytes allocated in the common

region.  If several programs  specify  the  same  label  for  a  common

region,  the  size  of  the  region  is  determined by the value of the

largest expression.

The common directive assumes the label is an undefined external  symbol

in  the current assembly.  However, the linker, LO68, is special-cased,

so all external symbols, which are not  otherwise  defined,  and  which

have  a  non-zero value, are defined to be in the bss, and enough space

is left after the symbol to hold expression bytes.  All  symbols  which

become  defined  in  this  way  are  located  before all the explicitly

defined bss locations.

*   The .even directive causes the location counter (*), if  positioned  at

an  odd  address,  to  be advanced by one byte so the next statement is

assembled at an even address.

*   The instructions move, add, and sub, specified with an immediate  first

operand  and  a  data  (D)  register as the destination, generate Quick

instructions, where possible.

5.8  ERROR MESSAGES

Appendix D lists the error messages generated by AS68.

5.9  INSTRUCTION SET SUMMARY

This section contains two tables that describe  the  assembler  instruction

set  distributed  with  GEMDOS.   Table 5-3 summarizes the assembler (AS68)

instruction set.  Table 5-4 lists variations on the instruction set  listed

in  Table  5-3.   For details on specific instructions, refer to Motorola's

16-Bit Microprocessor User's Manual, third edition, MC68000UM(AD3).

Table 5-3.  Instruction Set Summary

Instruction     Description

abcd            Add Decimal with Extend

add             Add

and             Logical AND

asl             Arithmetic Shift Left

asr             Arithmetic Shift Right

bcc             Branch Conditionally

bchg            Bit Test and Change

bclr            Bit Test and Clear

bra             Branch Always

bset            Branch Test and Set

bsr             Branch to Subroutine

btst            Bit Test

chk             Check Register Against Bounds

clr             Clear Operand

cmp             Compare

dbcc            Test Condition, Decrement, and Branch

divs            Signed Divide

divu            Unsigned Divide

eor             Exclusive OR

exg             Exchange Registers

ext             Sign Extend

illegal         Illegal Instruction

jmp             Jump

jsr             Jump to Subroutine

lea             Load Effective Address

link            Link Stack

lsl             Logical Shift Left

lsr             Logical Shift Right

move            Move

movem           Move Multiple Registers

movep           Move Peripheral Data

muls            Signed Multiply

mulu            Unsigned Multiply

Table 5-3. (continued)

Instruction     Description

nbcd            Negate Decimal with Extend

neg             Negate

nop             No Operation

no              One's Complement

or              Logical OR

pea             Push Effective Address

reset           Reset External Devices

rol             Rotate Left without Extend

ror             Rotate Right without Extend

roxl            Rotate Left with Extend

roxr            Rotate Right with Extend

rte             Return From Exception

rtr             Return and Restore

rts             Return from Subroutine

sbcd            Subtract Decimal with Extend

scc             Set Conditional

stop            Stop

sub             Subtract

swap            Swap Data Register Halves

tas             Test and Set Operand

trap            Trap

trapv           Trap on Overflow

tst             Test

unlk            Unlink

Table 5-4.  Variations of Instruction Types

Instruction         Variation

add       add       Add

adda      Add Address

addq      Add Quick

addi      Add Immediate

addx      Add with Extend

Table 5-4. (continued)

Instruction         Variation

and       and       Logical AND

andi      AND Immediate

andi to ccr

andi to sr

cmp       cmp       Compare

cmpa      Compare Address

cmpm      Compare Memory

cmpi      Compare Immediate

eor       eor       Exclusive OR

eori      Exclusive OR Immediate

eori to ccr

eori to sr

move      move      Move

movea     Move Address

moveq     Move Quick

move to ccr

move to sr

move from sr

move to usp

neg       neg       Negate

negx      Negate with Extend

or        or        Logical OR

ori       OR Immediate

ori to ccr

ori to sr OR Immediate to Status Register

sub       sub       Subtract

suba      Subtract Address

subi      Subtract Immediate

subq      Subtract Quick

subx      Subtract with Extend

End of Section 5

W

GEM DOS LINKERS

Two linkers are provided with GEM DOS:   LINK68  and  LO68.    Both  create

relocatable command files in the format required by the operating system.

Note  that  both  LINK68  and  LO68  produce  files in CP/M-68K relocatable

format.  Use the RELMOD  format  conversion  utility  to  translate  linker

output  files  from  CP/M-68K  format  to  the  GEM DOS relocatable format.

RELMOD is described in Section 6.3.

6.1  LINK68

LINK68 is a linkage editor that combines object files into a command  file.

LINK68  accepts object files produced by other language processors, such as

AS68 or the C compiler.  LINK68  produces  an  executable  file  in  either

contiguous  or  noncontiguous  command  file  format. Noncontiguous command

files are not supported by GEM DOS.  See Section 3 for a description of the

contiguous file format.

LINK68  resolves  all  references  to external symbols and concatenates the

object files in the order specified.  The  entry  point  of  the  resulting

command file is the first instruction in the first object file.

Note:  You must use LINK68 to create an executable command file even when a

single object file contains no unresolved references.

If you use the AS68 common directive to specify a  common  area  shared  by

separate  modules,  LINK68  resolves all common areas with the same name to

the same address in the bss segment.   If  more  than  one  file  specifies

static  storage  with  the  same  name,  LINK68  uses  the largest size for

allocation.

6.1.1  Invoking LINK68

To invoke LINK68, enter a command of the following form:

LINK68 [file =] object-file-1 [,object-file-2,...object-file-n]

If you invoke LINK68  without  any  command  tail,  the  linker  lists  the

options, and returns to the operating system.

The  first  file  specification is the name of the command file you want to

create, and object-file-1 through object-file-n are  the  object  files  to

link. For example, the following command creates the output file MATH:

{a}link68 math = sin,cos,tan

If  you omit the filename to the left of the equal sign, LINK68 creates the

output file using the first filename in the command line, and  assigns  the

default filetype 68K.  For example, the following command creates SIN.68K:

{a}link68 sin,cos,tan

6.1.2  LINK68 Command-Line Options

When  you  invoke LINK68, you can specify command-line options that control

the link operation.  There are two kinds  of  options:  global  and  local.

Global  options  apply  to  the entire link operation.  Local options apply

only to the individual files being linked. You enclose  global  options  in

square brackets immediately preceding the output filename (if specified) in

the command line.  You enclose local options in square brackets immediately

following the filename to which they apply.

You  can use spaces between filenames to improve readability in the command

line, and you can put more than one option in square brackets by separating

them  with  commas.  LINK68 also allows you to abbreviate an option name to

its shortest unambiguous form.  Table 6-1  lists  the  LINK68  options  and

explains their use.  Abbreviated forms are in parentheses.

Note:  GEM  DOS  requires that the linked file have the magic number 601AH.

LINK68 produces the output file  with  this  magic  number  only  when  you

request  a  relocatable  file with contiguous text, data, and bss segments.

Consequently, to create a command file to run under GEM DOS, do not  select

the  ABSOLUTE,  DATABASE[n],  BSSBASE[n],  or  TEXTBASE[n] options when you

invoke LINK68.

Table 6-1.  LINK68 Command-line Options

Option          Function

ABSOLUTE (AB)   Tells  LINK68  to  generate  an  absolute  file   with   no

relocation  bits.    The  default is a relocatable program.

Note that command files must be relocatable  to  run  under

GEM DOS.

ALLMODS (AL)    Tells  LINK68  to  load all modules from a library, even if

they are not referenced.  The default is  to  include  only

those modules that are actually referenced.

BSSBASE[n] (B[n])

Sets the base address for the  uninitialized  data  segment

(bss)  in  discontiguous  programs.  The n is a hexadecimal

value.  The default value is the first even word after  the

data segment.

Table 6-1. (continued)

Option          Function

COMMAND (C)     Tells  LINK68  that  the  following named file contains the

rest of the command line.  LINK68 ignores the rest  of  the

main  command  line.  Nested command files are not allowed.

The format of this option is:

COMMAND [filename]

where filename is the  file  containing  the  rest  of  the

command line.

DATABASE[n] (D[n])

Specifies  the  base  address  of  the  data   segment   in

discontiguous programs.  The n is a hexadecimal value.  The

default is the first even word after the text segment.

IGNORE (IG)     Tells LINK68 to ignore 16-bit address overflow.

INCLUDE (IN)    Tells LINK68 to load an unreferenced module from a library.

The   format  for  this  option  is  the  following,  where

module-name is the module you want to load:

filename [INCLUDE [module-name]]

LOCALS (L)      Tells LINK68 to put local symbols in the symbol table.  The

default  is no local symbols.  LOCALS only applies from the

point in the command line that it appears.

The NOLOCALS option turns this option off.  Use LOCALS  and

NOLOCALS  in combination to put local symbols from specific

files in the symbol table.   LINK68  always  ignores  local

symbols starting with L.

NOLOCALS (NO)   See LOCALS.

SYMBOLS (S)     Tells  LINK68  to  put the symbol table in the output file.

The default is no symbol table in the output file.

TEMPFILES[d:]  (TEM[d:])

Tells  LINK68  to  use  drive  d  for temporary files.  The

default is the currently  logged-in  drive.    If  you  use

TEMPFILES,  it  must precede any input files on the command

line.

Table 6-1. (continued)

Option          Function

TEXTBASE[n] (TEX[n])

Specifies  the base address for the text segment.  The n is

a hexadecimal value.  The default is 0H.

UNDEFINED (U)   Tells LINK68 to ignore the presence of undefined symbols in

the  input files.  LINK68 lists the undefined symbols, then

continues processing.  The default action is  to  list  any

undefined symbols, then stop processing.

The  following  are  examples  of  LINK68  command lines.  Addresses are in

hexadecimal.  The first command line example links the  files  FOOMAIN  and

FOOLIB  into  a command file named FOOBAZ.  It also tells LINK68 to include

the symbol table in FOOBAZ, and place the temporary files on drive B.

{a}link68 [sym, tem[b:]] foobaz = foomain, foolib

The next example tells LINK68 to  read  the  command  line  from  the  file

LINKIT.INP.  Note that closing brackets are not needed.

{a}link68 [com[linkit.inp

The file LINKIT.INP might contain the following commands:

{a}link68 [ab, tex[500], d[2a00], b[3000]] screen =

scrnsl [l], iolib[al]

This  command creates the file SCREEN from the files SCRNS1 and IOLIB.  The

command tells LINK68 to create SCREEN as an absolute  file  with  the  text

segment  starting  at  500H,  the  data  segment starting at 2A00H, and the

initialized data segment starting at  3000H.    It  also  tells  LINK68  to

include local symbols from SCRNS1 and all the modules in IOLIB.

6.1.3  REDIRECTING DIAGNOSTIC OUTPUT

Normally,  LINK68 sends all diagnostic output to the console.  However, you

can redirect this output by using the > character in the command line.  For

example, the following command creates MYFILE.68K on drive A, using drive B

for the temporary files, and sends  the  diagnostics  output  to  the  file

LNKMSGS.TXT on drive D:

{a}link68 [tem[b:] myfile.68k = moda, modb >d:lnkmsgs.txt

6.2  LO68

The  LO68  linker  combines  AS68  assembled (object) programs to produce a

relocatable command file.  All  external  references  are  resolved.    The

linker  must  be  used  to  create  executable programs, even when a single

object program contains no unresolved references.   The  argument  routines

are  concatenated in the order specified.  The entry point of the output is

the first instruction of the first routine.

Note: GEM DOS requires that the linked file have the  magic  number  601AH.

LO68  produces the output file with this magic number only when you request

a  relocatable  file  with  contiguous  text,  data,  and   bss   segments.

Consequently,  to create a command file to run under GEM DOS, do not select

the -T, -Z, -D, -B, and -S options and select the -R option when you invoke

LO68.

Appendix D lists the error messages that LO68 displays.

6.2.1  Invoking LO68

Invoke  LO68  by  entering  a  command  of  the  following  form. Table 6-2

describes the options.

LO68 [-F pathname] [-R] [-S] [-I] [-Uname]

[-O filename] [-X] [-Zaddress]

[-Daddress] [-Baddress] object filename [object filename]

[>message filename]

Table 6-2.  LO68 Options

Option          Meaning

-F pathname     Specifies the path name to the directory in which temporary

files are created.

-R              Preserves  the  relocation bits so the resulting executable

program is relocatable.

-S              Strips output (if specified); the output is stripped of the

symbol table and relocation bits.

-I              Does  not  generate 16-bit address overflow messages.  When

you assemble a program without  the  AS68  -L  option,  the

linker  might  generate  address  overflow  messages if the

program contains addresses longer than 16 bits.

-Uname          Forces linking of a library module that resolves  the  name

parameter, even if the name is not referred to by any other

module being linked.  Normally, library  modules  are  only

linked  when they are needed to resolve references in other

modules.  You can use this option to create a program  from

a  library if the module resolving the name parameter calls

other modules in the library.

-O filename     Gives the object file produced by LO68  the  filename  that

you  specify  following  the  -O option.  The -O option and

filename are separated by one or more spaces.   If  you  do

not specify a filename, the object file has the name C.OUT.

-X              Includes all local symbols in the symbol table except those

that begin with the letter L. If not specified,  LO68  puts

only  global  symbols  in  the  symbol  table.  This option

allows you to discard compiler internally generated  labels

that  begin with the letter L while retaining symbols local

to routines.

-Taddress*

-Zaddress*

The -T and -Z options  are  equivalent.    The  hexadecimal

address  given  is defined by LO68 as the beginning address

for the text segment.  This address defaults to zero, or it

can  be  specified as any even hexadecimal number between 0

and FFFFFFFF.  This option is useful for  putting  programs

in  ROM.    Hexadecimal  characters  can be in uppercase or

lowercase.

Table 6-2. (continued)

Option          Meaning

-Daddress*      The hexadecimal address given is defined  by  LO68  as  the

beginning  address  for  the  data  segment.   This address

defaults to the  next  byte  after  the  end  of  the  text

segment,  or  it  can  be specified as any even hexadecimal

number between 0 and FFFFFFFF.  This option  is  especially

useful  for  for  putting  programs  in  ROM.   Hexadecimal

address characters can be in uppercase or lowercase.

-Baddress*      The hexadecimal address given is defined  by  LO68  as  the

beginning  address  for  the bss.  This address defaults to

the next byte after the end of the data segment, or it  can

be  specified  as any even hexadecimal number between 0 and

FFFFFFFF.

object filename [object filename]

The  name  of  one  or  more  object  files produced by the

assembler AS68.  These  are  the  object  files  that  LO68

links.

>message filename

If  specified,  error  messages  produced   by   LO68   are

redirected  to  the file that you specify immediately after

the greater-than sign  (>).    If  you  do  not  specify  a

filename,  error  messages  are  written  to  the  standard

default output device,  which  typically  is  your  console

terminal.

6.2.2  Sample Commands Invoking LO68

{a}lo68 -s -o test.68k test.o

This  command links assembled file TEST.O into file TEST.68K and strips out

the symbol table and relocation bits.

{a}lo68 -t4000 -d8000 -bc000 a.o b.o c.o

This command links assembled files A.O, B.O, and C.O to the default  output

file  C.OUT.    The text segment starts at location 4000H; the data segment

starts at location 8000H; and the bss starts at location C000H.

{a}lo68 -i -o test.68k test.o test1.o > error

This command links assembled files TEST.O and  TEST1.O  to  file  TEST.68K.

Any 16-bit address overflow errors are ignored; error messages are directed

to the file ERROR.

6.3  RELMOD FORMAT CONVERSION UTILITY

LINK68 and LO68 produce files in CP/M-68K  relocatable  format.    Use  the

RELMOD  command to translate linker output file from CP/M-68K format to GEM

DOS relocatable format.

The RELMOD command line is in the following form:

RELMOD inputfile outputfile

where inputfile is the CP/M-68K format file  produced  by  the  linker  and

outputfile is the name of the file translated to GEM DOS format.

RELMOD can also be used to translate GEM DOS format files back to CP/M-68K.

This is done by simply specifying the GEM DOS file as the  input  file  and

the CP/M-68K file as the output file.

End of Section 6

ARCHIVE UTILITY

7.1  INTRODUCTION

The  Archive  utility,  AR68, creates a library or replaces, adds, deletes,

lists, or extracts object modules in an existing library.  AR68 can be used

on  the  C  Run-time Library distributed with GEM DOS and documented in the

GEM DOS Supplement to the C Language Programmer's Guide for CP/M-68K.

7.2  AR68 SYNTAX

To invoke AR68, specify the  components  of  the  following  command  line.

Optional components are enclosed in square brackets ([]).

AR68 DRTWX[AV][F pathname][OPMOD]ARCHIVE

OBMOD1[OBMOD2...][>filespec]

You  can  specify  multiple  object  modules in a command line provided the

command line does not exceed 127 bytes.  The  delimiter  character  between

components consists of one or more spaces.

Table 7-1 lists and describes the components of the AR68 command line.

Table 7-1.  AR68 Command Line Components

Component Meaning

AR68      Invokes  the Archive utility.  However, if you specify only

the AR68 command, AR68 returns the following  command  line

syntax and system prompt.

{a}ar68

usage:  AR68 DRTWX[AV][F pathname] [OPMOD] ARCHIVE

OBMOD1 [OBMOD2...][>filespec]

{a}

DRTWX     Indicates  you must specify one of these letters as an AR68

command.  Each of these one-letter commands and its options

are described in Section 7.4.

AV        Indicates  you  can specify one or both of these one-letter

options.  These options are described with the commands  in

Section 7.4.

F pathname

Specifies the path to the directory in which the  temporary

file  created  by  AR68  resides.    If  no  path  name  is

specified, the current default directory  is  used.    AR68

creates  a temporary file called AR68.TMP that AR68 uses as

a scratch pad area.

OPMOD     Indicates an object module  within  the  library  that  you

specify.    The  OPMOD  parameter indicates the position in

which additional object modules reside when you incorporate

modules in the library and specify the A option.

ARCHIVE   File specification of the library.

OBMOD1 [OBMOD2 ...]

Indicates one or more object modules in a library that AR68

deletes, adds, replaces, or extracts.

>filespec Redirects the output to the file specification you specify,

rather than sending  the  output  to  the  standard  output

device, which is usually the console device (CONSOLE).  You

can redirect the  output  for  any  of  the  AR68  commands

described in Section 7.4.

7.3  AR68 OPERATION

AR68  sequentially  parses  the  command  line  once.    AR68 searches for,

inserts, replaces, or deletes object modules in the library in the sequence

in  which you specify them in the command line.  Section 7.4 describes each

of the commands AR68 supports.

When AR68 processes a command, it creates a temporary file called AR68.TMP,

which  it  uses  as  a  scratch  pad.  After the operation is complete AR68

erases AR68.TMP.  However, AR68.TMP  is  not  always  erased  if  an  error

occurs.    If this occurs, erase AR68.TMP with the ERA command and refer to

Appendix D for error messages output by AR68.

7.4  AR68 COMMANDS AND OPTIONS

This  section  describes  AR68  commands  and  their  options.     Examples

illustrate  the effect and interaction between each command and the options

it supports.

7.4.1  The D Command

The D command deletes from the library one or more object modules specified

in the command.  The D command supports the following option:

V          Lists  the  modules  in  the  library  and  indicates which

modules are retained and deleted by the D command.   The  V

option  precedes  modules  retained in the library with the

lowercase letter c and modules  deleted  from  the  library

with the lowercase letter d as follows:

{a}ar68 dv myrah.arc orc.o

c red.o

c blue.o

d orc.o

c white.o

{a}

The  D  command  deletes  the module ORC.O from the library

MYRAH.ARC.  In addition  to  listing  the  modules  in  the

library,  the V option indicates which modules are retained

and deleted.

7.4.2  The R Command

The R command creates a library when the one specified in the command  line

does  not exist, or replaces or adds object modules to an existing library.

You must specify one or more object modules.

You can replace more than one object module in the  library  by  specifying

the  module  names in the command line.  However, when the library contains

two or more modules with the same name, AR68 replaces only the first module

it finds that matches the one specified in the command line.  AR68 replaces

modules already in the library only if you specify their names prior to the

names  of  new  modules  to  be  added to the library.  For example, if you

specify the name of a module you want replaced after the name of  a  module

you  are  adding  to  the library, AR68 adds both modules to the end of the

library.

By default, the R command adds new modules to the end of the library.   The

R command adds an object module to a library if:

*   The object module does not already exist in the library.

*   You specify the A option in the command line.

*   The  name  of  the  module  follows  the name of a module that does not

already exist in the library.

The A option indicates where AR68 adds modules to the library.  You specify

the relative position by including the OPMOD parameter with the A option.

The  R  command  also supports the V option, which lists the modules in the

library and indicates the result of the operation performed on the library.

Both the A and V options are described below.

A           The A option adds one or more object modules following the

module specified in the command line:

{a}ar68 rav sdav.o myrah.arc work.o mail.o

c much.o

c sdav.o

a work.o

a mail.o

c less.o

The RAV command adds the object modules WORK.O  and  MAIL.O

after  the  module  SDAV.O in the library MYRAH.ARC.  The V

option, described below,  lists  all  the  modules  in  the

library.   New modules are preceded by the lowercase letter

a and existing modules are preceded by the lowercase letter

c.

V          The  V  option  lists the object modules that the R command

replaces or adds.

{a}ar68 rv jnnk.man nail.o wrench.o

c saw.o

c ham.o

r nail.o

c screw.o

a wrench.o

{a}

The R command replaces the object module  NAIL.O  and  adds

the  module WRENCH.O to the library JNNK.MAN.  The V option

lists object modules in the  library  and  indicates  which

modules  are replaced or added.  Each object module that is

replaced is preceded with the lowercase letter r  and  each

one that is added is preceded with the lowercase letter a.

7.4.3  The T Command

The  T  command  requests  that AR68 print a table of contents or a list of

specified modules in the library.  The T command prints a table of contents

of  all modules in the library only when you do not specify names of object

modules in the command line.  It supports the following option.

V          The V option displays the size of each file in the table of

contents as shown in the following example.

{a}ar68 tv wine.bad

rw-rw-rw- 0/0    6818 rose.o

rw-rw-rw- 0/0    2348 white.o

rw-rw-rw- 0/0     396 red.o

{a}

The  T  command  prints  a table of contents in the library

WINE.BAD.  In  addition  to  listing  the  modules  in  the

library,  the  V  option  requests the size of each module.

The character string rw-rw-rw- 0/0 that precedes the module

size  is  meaningless  for GEM DOS. However, if the file is

transferred to  a  UNIX...  system,  the  character  string

denotes  the  file  protection  and  file  owner.  The size

specified by the decimal number that  precedes  the  object

module name indicates the number of bytes in the module.

7.4.4  The W Command

The  W  command  writes  a  copy  of an object module in the library to the

>filespec parameter specified in the command line.  This command allows you

to  extract  a copy of a module from a library and rename the copy when you

write it to another disk, as shown below.  For this command, the  >filespec

parameter is required.

{a}ar68 w go.arc now.o > b:\root\newd\file.o

The  W  command  writes  a copy of the object module NOW.O from the library

GO.ARC to the file FILE.O in the NEWD subdirectory on drive B.

7.4.5  The X Command

The X command extracts a copy of one or more object modules from a  library

and writes them to the default disk.  If no object modules are specified in

the command line, the X command extracts a  copy  of  each  module  in  the

library. The X command supports the following option.

V          The  V  option  lists  only  those  modules  the  X command

extracts from the library.    It  precedes  each  extracted

module with the lowercase letter x as follows:

{a}ar68 xv jnnk.man saw.o ham.o screw.o

x saw.o

x ham.o

x screw.o

7.5  AR68 ERRORS

When  AR68  incurs  an  error  during  an  operation,  the operation is not

completed.  The original library is not modified  if  the  operation  would

have  modified  the  library.  Thus, no modules in the library are deleted,

replaced, added, or extracted.  Refer to  Appendix  D  for  error  messages

output by AR68.

When  you  specify  the >filespec parameter in the command line to redirect

the output and one or more errors occur, the error messages are sent to the

output  file.    Thus,  you  cannot detect the errors without displaying or

printing the file to which the output was sent.  If  the  contents  of  the

output  file  is  an object file (see the W command), you must use the DUMP

utility described in Section 8 to read any error messages.

End of Section 7

MORE PROGRAMMING UTILITIES

8.1  INTRODUCTION

This section describes the DUMP,  SIZE68,  SENDC68,  and  XREF  programming

utilities.    DUMP  displays the contents of files in hexadecimal and ASCII

notation.  SIZE68 displays the total size of a memory  image  command  file

and  the  size  of  each  of  its  program segments.  XREF produces a cross

reference symbol table for GEM DOS object files.  SENDC68 creates a file of

Motorola  S-records  from  a  command  file.    (Refer  to Appendix E for a

detailed description of the S-record format.)

8.2  DUMP UTILITY

The DUMP utility (DUMP) displays the contents of a GEM  DOS  file  in  both

hexadecimal  and  ASCII  notation.  You can use DUMP to display any GEM DOS

file regardless of the format of its contents (binary data, ASCII text,  an

executable file).

8.2.1  Invoking DUMP

Invoke  DUMP  by  entering a command with the following input components in

the following format:

DUMP [ -sxxxx ] filename1 [ >filename2 ]

Table 8-1 lists the DUMP command line components and their meanings.

Table 8-1.  DUMP Command Line Components

Component Meaning

-sxxxx    xxxx is an optional offset (in hexadecimal) into the  file.

If  specified, DUMP starts dumping the contents of the file

from the byte-offset xxxx and continues until  it  displays

the  contents  of the entire file.  By default, DUMP starts

dumping the contents of the file from the beginning of  the

file until it dumps the contents of the entire file.

filename1 Name of the file you want to dump.

>filename2

The greater than sign (>) followed by a filename or logical

device  optionally  redirects  the output of DUMP.  You can

specify any valid GEM DOS  specification,  or  one  of  the

logical device names, CON: (console) or LST: (list device).

If you do not specify this optional parameter,  DUMP  sends

its output to the console.

8.2.2  DUMP Output

DUMP  sends  its  output  to  the  console  (or  to  a  file  or device, if

specified), 8 words per line, in the following format:

rrrr oo (ffffff):  hhhh hhhh hhhh hhhh hhhh hhhh hhhh hhhh

*aaaaaaaaaaaaaaaa*

The components of a DUMP command line are as follows:

Component       Meaning

rrrr            Record number (GEM  DOS  records  are  128  bytes)  of  the

current line of the display.

oo              Offset  (in  hex  bytes)  from the beginning of the GEM DOS

record.

ffffff          Offset (in hex bytes) from the beginning of the file.

hhhh            Contents of the file displayed in hexadecimal.

aaaaaaaa        Contents of the file displayed as ASCII characters.  If any

character is not representable in ASCII, it is displayed as

a period (.).

8.2.3  DUMP Examples

In the following example, DUMP is invoked to  display  the  contents  of  a

command file that contains data in both binary and ASCII form.

{a}dump dump.68k

0000 00 (000000):  601a 0000 1b34 0000 011d 0000 0e5e 0000

*`....4.......^..*

0000 10 (000010):  0000 0000 0000 0000 0900 ffff 6034 4320

*............`4C *

0000 20 (000020):  5275 6e74 696d 6520 436f 7079 7269 6768 *Run-

time Copyright*

0000 30 (000030):  7420 3139 3832 2062 7920 4469 6769 7461 *t

1982 by Digita*

0000 40 (000040):  6c20 5265 7365 6172 6368 2056 3031 2c30 *l

Research V01.0*

0000 50 (000050):  3320 206f 0004 2268 0018 2649 d3e8 001c *3

o.."h..&ISh..*

. . . . (and so on) . . .

8.3  SIZE68 UTILITY

The SIZE68 utility (SIZE68) indicates if the program segments within one or

more command files are contiguous or non-contiguous, displays the  size  of

each program segment and the symbol table, and reports if the command files

are relocatable or non-relocatable.    SIZE68  displays  both  decimal  and

hexadecimal  values  for  the  sizes of the program segments and the symbol

table.  GEM DOS command files usually have a filetype of .PRG or .REL.  The

total  size of a command file's segments returned by SIZE68 and the size of

a command file returned by the DIR command are not equal.   The  file  size

returned  by  SIZE68  includes  the size of the text, data, and bss program

segments and the size of the symbol table but does not include the size  of

the  header  and and relocation bits.  For more details on the DIR command,

refer to the GEM DOS User's Guide.

8.3.1  Invoking SIZE68

Invoke SIZE68 by entering a command line with the following format:

SIZE68 filename [filename2 filename3, ... ] [ >outfile ]

The SIZE68 command line components have the following meaning:

Component       Meaning

filename        File specification  of  a  file  whose  size  you  want  to

determine.

filename2

filename3 ....

One  or  more additional file specifications of files whose

sizes you want to determine. SIZE68  can  process  multiple

files, provided the command line does not exceed 128 bytes.

Note that SIZE68 also accepts wildcard file specifications.

>outfile        Specifies the file specification to which SIZE68 sends  its

output.      If   you   do   not  specify  an  output  file

specification, SIZE68 sends the output to the console.  For

the  output file specification, you can specify a valid GEM

DOS filename, or one of  the  logical  device  names,  CON:

(console) or LST: (list device).

8.3.2  SIZE68 Examples and Output

This  section  contains  two  examples  that  show SIZE68 command lines and

output.

1.  The following SIZE68 command line example returns information about the

program segments in one command file.

{a}SIZE68 SIZE68.PRG

SIZE68.PRG:

Contiguous

.text length            =   9312        2460

.data length            =   1178         49A

.bss length             =   9140        23B4

Symbol table length     =      0           0

Start of .text          =      0           0

File is relocatable.

SIZE68.PRG  contains  a  9312-byte  (decimal) text segment, a 1179-byte

(decimal) data segment, and a 9140-byte (decimal) bss; the segments are

contiguous  and  SIZE68  is relocatable.  Hexadecimal notations for the

decimal values are displayed in the last column of SIZE68 output.

2.  The following SIZE68 command line uses a wildcard file specfication  to

return information on an object file, a command file, and a text file.

{a}SIZE68 FI*.*

FIND.O:

Contiguous

.text length            =   1072         430

.data length            =    188          BC

.bss length             =      0           0

Symbol table length     =   1708         6AC

Start of .text          =      0           0

File is relocatable.

FIND.PRG:

Contiguous

.text length            =   9888        26A0

.data length            =   1060         424

.bss length             =   9396        24B4

Symbol table length     =      0           0

Start of .text          =      0           0

File is relocatable.

FILE.MSG:

Not a program file.

Notice  that  when  you  specify  a  file  that  is  not a command file

(FILE.MSG, an ASCII file, for example), SIZE68 displays:

Not a program file.

When you specify an absolute program file whose segments are non-contiguous

in  a  SIZE68  command  line, SIZE68 includes the following messages in its

output:

Non-contiguous

No relocation information in file.

8.4  SENDC68 UTILITY

SENDC68 creates a file with  Motorola  S-record  format  from  an  absolute

command  file.    S-records  are  a way to represent an absolute program in

ASCII character form.  For a detailed description of the  S-record  format,

refer to Appendix E.

8.4.1  Invoking SENDC68

Invoke SENDC68 by entering a command line in the following format:

SENDC68  [-]  inputfile [outputfile]

The SENDC68 command line components are described below.

Component       Meaning

-               A  hyphen  is  optional.   If you specify a hyphen, SENDC68

does not create any S-records for the  bss  segment.    The

result is a smaller S-record file.  If you do not specify a

hyphen, SENDC68 fills the bss segment with zeros.

inputfile       File that SENDC68 converts to the  S-record  format.    The

command  file  must  be  an  absolute  file  in  the format

produced by LINK68, LO68, or RELOC.

outputfile      File that SENDC68 sends the new S-record file to.   If  you

do  not specify an output file, SENDC68 sends the S-records

to the console screen.

8.4.2  SENDC68 Example

The following command line example illustrates how to convert  an  absolute

command file into a file in the Motorola S-record format.  In this example,

SENDC68 creates an S-record file named PROG.SR  from  an  absolute  command

file named PROG.68K.

{a}>sendc68 - prog.68k prog.sr

Note  that  the  hyphen directs SENDC68 not to create S-records for the bss

segment.

8.5  XREF UTILITY

The XREF utility generates a cross-reference table of symbols for  GEM  DOS

object files.  XREF output lists the symbols, the object file in which they

are defined, and the object  files  in  which  they  are  accessed.    XREF

provides a separate listing for undefined symbols and the object files that

call them.  XREF accepts wildcards in the object file specifications.

8.5.1  Invoking XREF

To use XREF, enter a command of the following format:

XREF objectfile1 [objectfile2 ... objectfilelast]

Where "objectfile1" is the name of the object file for  whose  symbols  you

want  a  cross-referenced  table.    Multiple  object  file  names  may  be

specified.

8.5.2  XREF Examples

This section contains two example XREF command lines and shows  the  format

of XREF output.

1.  The  following  XREF  command  generates  a  cross-referenced  table of

symbols for the file GSXVAR.O.

{a}XREF GSXVAR.O

Block Data:

Symbol       Defined In   Accessed In

cur_ms_s       GSXVAR

disab_cn       GSXVAR

draw_fla       GSXVAR

.                                       .

.                                       .

.                                       .

_angle         GSXVAR     MONOBJ   MONOUT

_beg_ang       GSXVAR     MONOBJ   MONOUT

.                                       .

.                                       .

.                                       .

Undefined External References:

Symbol      Defined In  Accessed In

_ABLINE       ********  MONOUT

_CHUP         ********  MONOUT    MONOBJ

.                                      .

.                                      .

.                                      .

2.  The example XREF command shown below uses a wildcard file specification

to generate a cross-referenced table of symbols for each object file in

the current directory.

{a}XREF *.O

Functions:

Symbol      Defined In  Accessed In

_arb_cor      MONOBJ     MONOUT

_arrow        MONOUT

_Calc_pt      MONOUT

.                                  .

.                                  .

.                                  .

End of Section 8

SID68 DEBUGGER

SID68 allows you to test and debug programs interactively in  the  GEM  DOS

environment.    The presentation of information in this section assumes you

are familiar with the MC68000 microprocessor, the assembler (AS68), and the

GEM DOS operating system.

9.1  INVOKING SID68

Invoke SID68 by entering one of the following commands:

SID

SID filename

The  first  command  loads  and  executes SID68.  After it is loaded, SID68

displays its sign-on message and the hyphen (-) prompt character to show it

is ready to accept commands.

The  second command invokes SID68 and loads the file specified by filename.

If the filetype is not specified, it defaults to the  68K  filetype.    The

second form of the command is equivalent to the following sequence in which

the first command is issued and then, at the SID68 prompt, the E command is

issued to load a file for execution under SID68:

{a}SID

SID68

Copyright 1982, Digital Research

-Efilename

9.1.1  SID68 Command Conventions

When  SID68 is ready to accept a command, it prompts you with a hyphen (-).

In response, you can type a command line  or  a  Ctrl-C  (^C)  to  end  the

debugging  session.   A command line can have as many as 64 characters, and

must be terminated with a Return.  When entering the command, use  standard

GEM  DOS  line-editing  functions to correct typing errors.  SID68 does not

process the command line until you enter a Return.

Table 9-1 summarizes SID68 commands.  They are defined  individually  later

in this section.

Table 9-1.  SID68 Command Summary

Command                             Action

D      Display memory in hexadecimal and ASCII.

E      Load program for execution.

F      Fill memory block with a constant.

G      Begin execution with optional breakpoints.

H      Use hexadecimal arithmetic.

I      Set up file control block and command tail.

L      List memory using MC68000 mnemonics.

M      Move memory block.

P      Set and remove permanent breakpoints.

R      Read disk file into memory.

S      Set memory to new values.

T      Trace program execution.

U      Untrace program monitoring.

V      Show memory layout of disk file read.

W      Write contents of memory block to disk.

X      Examine and modify CPU state.

The  command  character can be followed by one or more arguments, which can

be hexadecimal values, filenames, or other information,  depending  on  the

command.    Some commands can operate on byte, word, or longword data.  The

letters W for word or L for  longword  must  be  appended  to  the  command

character  for  commands  that operate on multiple data lengths.  Arguments

are separated from each other by commas or spaces.

See Section 9.2 for more details on each command.

9.1.2  Address Specifications

Most SID68 commands require  one  or  more  addresses  as  operands.    All

addresses  are  entered  as  hexadecimal numbers of up to eight hexadecimal

digits (32 bits).

9.1.3  Symbol References

SID68 allows you to reference symbols in  place  of  addresses.    You  can

display the symbols, along with the other disassembled instructions in your

executable file, by using the L command described in Section 9.2.7.    When

entering  a command that specifies a symbol, precede the symbol name with a

period as follows:

command.symbol

For example, to direct the GO command to execute from the  current  Program

Counter (PC) to the symbol QUIT in the object file, enter:

g,.quit

The C compiler puts an underscore (_) at the beginning of external symbols.

For example, to specify the GO command with a breakpoint at the C  function

BLIVOT, enter:

g,._blivot

9.1.4  Stopping SID68

Stop  SID68  by typing a ^C in response to the hyphen prompt.  This returns

control to the CCP.

9.1.5  SID68 Operation with Interrupts

SID68 operates with interrupts  enabled  or  disabled,  and  preserves  the

interrupt  state of the program being executed under SID68.  When SID68 has

control of the CPU, either when it is initially invoked or when it  regains

control  from the program being tested, the condition of the interrupt mask

is the same as it was when SID68 was invoked, except  for  a  few  critical

regions  where interrupts are disabled.  While the program being tested has

control of the CPU, the user's CPU state, which can be displayed with the X

command, determines the state of the interrupt mask.

Note  that  SID68  uses  the  Trace  and  Illegal  Instruction  exceptions.

Therefore, programs debugged under test should not use these.

9.2  SID68 COMMANDS

This section defines SID68 commands and their arguments.    SID68  commands

allow you to control program execution and display and modify system memory

and the CPU state.

9.2.1  The D (Display) Command

The D command displays the contents of memory as 8-bit, 16-bit,  or  32-bit

hexadecimal values and in ASCII.  The forms are

D

Ds

Ds,f

DW

DWs

DWs,f

DL

DLs

DLs,f

where  s  is  the  starting  address,  and f is the last address that SID68

displays.

Memory is displayed on one or more lines.  Each line shows the values of up

to  16  memory  locations.    For  the  first three forms, the display line

appears as follows:

aaaaaaaa bb bb ... bb cc ... cc

where aaaaaaaa is the address of  the  data  being  displayed.    The  bb's

represent the contents of the memory locations in hexadecimal, and the cc's

represent  the  contents  of  memory  in  ASCII.    Any  non-graphic  ASCII

characters are represented by periods.

In response to the Ds form of the D command, shown above, SID68 displays 12

lines that start from the current address.

Form Ds,f displays the memory block between locations s and f.   Forms  DW,

DWs,  and  DWs,f  are  identical  to D, Ds, and Ds,f except the contents of

memory are displayed as 16-bit or word values, as shown below:

aaaaaaaa  wwww  wwww  ...  wwww  cccc  ...  cccc

Forms DL, DLs, and DLs,f are identical  to  D,  Ds,  and  Ds,f  except  the

contents  of  memory  are  displayed as 32-bit or longword values, as shown

below:

aaaaaaaa llllllll llllllll ... llllllll cccccccc ...

During a display, you can abort the D command by typing  any  character  at

the console.

9.2.2  The E (Load for Execution) Command

The  E  command  loads  a  file  in  memory so that a subsequent G, T, or U

command can begin program execution.  The syntax for the E command is

Efilename

where filename is the name of the file to be loaded.   If  no  filetype  is

specified, the filetype 68K is assumed.

An  E  command reuses memory used by any previous E command.  Thus, you can

load only one file at a time for execution.

When the load is complete, SID68 displays the starting and ending addresses

of each segment in the file.  Use the V command to display this information

at a later time.

If the file does  not  exist  or  cannot  be  successfully  loaded  in  the

available  memory,  SID68  displays  an  error message.  See Appendix A for

error messages returned by SID68.

9.2.3  The F (Fill) Command

The F command fills an area of  memory  with  a  byte,  word,  or  longword

constant.  The forms are

Fs,f,b

FWs,f,w

FLs,f,l

where  s  is  the  starting address of the block to be filled, and f is the

address of the final byte of the block within the segment specified in s.

In response to the first form, SID68 stores the 8-bit value b in  locations

s through f.  In the second form, the 16-bit value w is stored in locations

s through f in standard form: the high 8 bits are first,  followed  by  the

low 8 bits.  In the third form, the 32-bit value l is stored in locations s

through f with the Most Significant Byte first.

If s is greater than f, SID68 responds with a question mark.  Also, if b is

greater  than  FF  hexadecimal  (255),  w  greater  than  FFFF  hexadecimal

(65,535), or l greater than  FFFFFFFF  hexadecimal  (4,294,967,295),  SID68

responds  with  a  question  mark.    SID68 displays an error message if it

cannot read back the value stored  in  memory  successfully.    This  error

indicates a faulty or non-existent RAM location.

9.2.4  The G (Go) Command

The G command transfers control to the program being tested, and optionally

sets one to ten breakpoints.  The forms are as follows:

G

G,b1,...b10

Gs

Gs,b1,...b10

where s is the address at which the program begins executing and b1 through

b10 are addresses of breakpoints.

In  the  first  two  forms, no starting address is specified.  SID68 starts

executing the program at the address specified by the Program Counter (PC).

The  first  form  transfers  control  to  the  program  without setting any

breakpoints.  The second form sets breakpoints before  passing  control  to

the program.  The last two forms are analogous to the first two except that

the PC is first set to s.

Once control has been transferred to the program under test, it executes in

real-time  until  it encounters a breakpoint.  At this point, SID68 regains

control, clears all breakpoints, and displays the CPU  state  in  the  same

form  as  the  X  command.  When a breakpoint returns control to SID68, the

instruction at the breakpoint address has not yet been executed.  To set  a

breakpoint at the same address, you must specify a T or U command first.

9.2.5  The H (Hexadecimal Math) Command

The  H  command  computes the sum and difference of two 32-bit values.  The

form is

Ha,b

where a and b are the values  whose  sum  and  difference  SID68  computes.

SID68  displays  the sum (ssssssss) and the difference (dddddddd) truncated

to 16 bits as follows:

ssssssss dddddddd

9.2.6  The I (Input Command Tail) Command

The I command prepares a file control block (FCB) and command  tail  buffer

in  the  base page of the last file loaded with the E command.  The form is

as follows:

Icommandtail

where commandtail is the character string which  usually  contains  one  or

more filenames.  The first filename is passed into the default File Control

Block at 005CH.  The optional second filename, if specified, is passed into

the  second  default File Control Block beginning at 0038H.  The characters

in the command tail are also copied to the default command buffer at 0080H.

The  length  of  the  command  tail  is  stored  at  0080H, followed by the

character string terminated with a binary zero.

If a file has been loaded with the E command, SID68 copies the File Control

Block  and  command  buffer from the base page of SID68 to the base page of

the program loaded.

9.2.7  The L (List) Command

The L command lists the contents of memory in assembly language.  The forms

are as follows:

L

Ls

Ls,f

where s is the starting address, and f is the last address in the list.

The first form lists 12 lines of disassembled machine code from the current

address.  The second form sets the list address to  s  and  then  lists  12

lines of code.  The last form lists disassembled code from s through f.  In

all three cases, the list address is set to the next unlisted  location  in

preparation  for a subsequent L command.  When SID68 regains control from a

program being tested (see G, T, and U commands), the list address is set to

the address in the Program Counter (PC).

Long  displays  can be aborted by pressing any key during the list process.

Or, enter Ctrl-S (^S) to halt the  display  temporarily.    A  Ctrl-Q  (^Q)

restarts the display after ^S halts it.

The syntax of the assembly language statements produced by the L command is

described in  the  Motorola  16-Bit  Microprocessor  User's  Manual,  third

edition,  MC68000UM(AD3).   Section 9.2.17 describes some minor differences

between the assembly language statements produced  by  the  L  command  and

standard Motorola 68000 assembly language.

9.2.8  The M (Move) Command

The  M  command  moves  a  block  of data values from one area of memory to

another.  The form is as follows:

Ms,f,d

where s is the starting address of the block to be moved, f is the  address

of  the  final  byte  of the block to be moved, and d is the address of the

first byte of the area to receive the data.  Note that if d  is  between  s

and  f,  part  of  the  block  being moved will be overwritten before it is

moved, because data is transferred starting from location s.

9.2.9  The P (Pass Points) Command

The P command sets, clears, and displays pass points.   The  forms  are  as

follows:

Pa,n

Pa

-P

A  pass  point  is  a permanent breakpoint that remains in effect until you

explicitly remove it, as opposed to breakpoints set with the G command that

must  be  reentered  with each G command.  Pass points have associated pass

counts ranging from 1 to 0FFFFH.  The pass count indicates how  many  times

the  instruction  at  the pass point executes before the control returns to

the console.  SID68 can set up to 16 pass points at a time.

An important distinction between breakpoints and pass points is  that  when

execution  stops at a breakpoint, the instruction at the breakpoint has not

been executed.  When execution stops due to a pass point whose  pass  count

has  reached  1, the instruction at the pass point has been executed.  This

makes it simple to proceed from a pass  point  with  a  G  command  without

encountering the same pass point.

Forms  Pa,n and Pa set pass points.  Form Pa,n sets a pass point at address

a (pass point address) with a pass count of n (from 1 to OFFFH).  If a pass

point  is  already  active  at  a,  the  pass count is changed to n.  SID68

responds with a question mark if there are already 16 active pass points.

Form Pa sets a pass point at address a with a pass count of 1.  If  a  pass

point  is already active at address a, the pass count is changed to 1 if it

is not 1.  SID68 responds with a question mark  if  there  are  already  16

active pass points.

The -P form is used to clear pass points.

9.2.10  The R (Read) Command

The R command reads a file to a contiguous block in memory.  The format is

Rfilename

where filename is the name and type of the file to be read.

SID68  reads  the  file  into  memory  and displays the starting and ending

addresses of the block of memory occupied by the file.  A Value (V) command

can  redisplay  the  information  at  a  later  time.   The default display

pointer, for subsequent Display (D) commands is set to  the  start  of  the

block occupied by the file.

9.2.11  The S (Set) Command

The  S  command  can  change  the contents of bytes, words, or longwords in

memory.  The forms are

Ss

SWs

SLs

where s is the address at which the change is to occur.

SID68 displays the memory address and its current contents.  In response to

the first form, the display is

aaaaaaaa bb

In response to the second form, the display is

aaaaaaaa wwww

In response to the third form, the display is

aaaaaaaa llllllll

where  bb, wwww, and llllllll are the contents of memory in byte, word, and

longword formats, respectively.

In response to one of the above displays, you can alter the memory location

or  leave  it  unchanged.    If  you  enter  a valid hexadecimal value, the

contents of the byte, word, or longword in memory  is  replaced  with  that

value.   If you do not enter a value, the contents of memory are unaffected

and the contents of the next address are displayed.  In either case,  SID68

continues  to display successive memory addresses and values until either a

period or an invalid value is entered.

SID68 displays an error message if it cannot read back the value stored  in

memory  successfully.    This  error indicates a faulty or non-existent RAM

location.

9.2.12  The T (Trace) Command

The T command traces program execution for 1 to 0FFFFFFFFH  program  steps.

The forms are

T

Tn

Tw

where  n  is the number of instructions to execute before returning control

to the console.

After SID68 traces each instruction, it displays the current CPU state  and

the disassembled instruction in the same form as the X command display.

Control transfers to the program under test at the address indicated in the

program counter. If you do not specify  n,  one  instruction  is  executed.

Otherwise,  SID68 executes n instructions and displays the CPU state before

each step.  You can abort a long trace  before  all  the  steps  have  been

executed by pressing any character at the console.

The  Tw  form  traces  execution  without  calls to subroutines. The entire

subroutine called from the program level  being  traced  is  treated  as  a

single  program  step  and executed in real time.  This allows tracing at a

high level of the program, ignoring subroutines that are already debugged.

After a T command, the list address used in the L command  is  set  to  the

address of the next instruction to be executed.

Note  that  SID68 does not trace through a BDOS interrupt instruction since

SID68 itself makes BDOS calls and the BDOS is not reentrant.  Instead,  the

entire  sequence of instructions from the BDOS interrupt through the return

from BDOS is treated as one traced instruction.

9.2.13  The U (Untrace) Command

The U command is identical to the Trace (T) command  except  that  the  CPU

state is displayed only after the last instruction is executed, rather than

after every step.  The forms are

U

Un

where n is the number of instructions to execute before control returns  to

the  console.    You  can  abort the command before all the steps have been

executed by pressing any key at the console.

9.2.14  The V (Value) Command

The V command displays information about the last file loaded with the Load

For Execution (E) or Read (R) commands.  The form is

V

If  the last file was loaded with the E command, the V command displays the

starting address and length of each of the segments contained in the  file,

the  base  page  pointer, and the initial stack pointer.  The format of the

display is

Text base=00000500 data base=00000B72 bss base=00003FDA

text length=00000672 data length=00003468 bss length=0000A1B0

base page address=00000400  initial stack pointer=000066D4

If no file has been loaded, SID68 responds to the V command with a question

mark.

9.2.15  The W (Write) Command

The  W command writes the contents of a contiguous block of memory to disk.

The forms are

Wfilename

Wfilename,s,f

The filename is the file specification of the disk file that  receives  the

data.

If  you  use  the first form, SID68 assumes the values for s and f from the

last file read with an R command.  If  no  file  has  been  read  by  an  R

command,  SID68  responds  with  a  question mark.  This form is useful for

writing out files after patches have been installed, assuming  the  overall

length of the file is unchanged.

In  the second form the letters s and f are the first and last addresses of

the block to be written.  If f does not specify  the  last  address,  SID68

uses the same value that was used for s.

If  the  file  specified  in  the  W  command already exists on disk, SID68

deletes the existing file before it writes the new file.

9.2.16  The X (Examine CPU State) Command

The X command displays the entire state of the CPU, including  the  Program

Counter  (PC), User Stack Pointer (USP), System Stack Pointer (SSP), Status

Register (ST, displayed by field), all  eight  data  registers,  all  eight

address  registers,  and the disassembled instruction at the memory address

currently in the PC.  The forms are

X

Xr

where r is one of the following registers:

D0 to D7, A0 to A7, PC, USP, or SSP

The first form displays the CPU state as follows:

PC=00016000 USP=00001000 SSP=00002000 ST=FFFF=> (etc.)

D  00001000 00000D01 ... 00000001

A  000B0A00 000A0010 ... 00000000

lea $16028,A0

The first line includes:

PC              program counter

USP             user stack pointer

SSP             system stack pointer

ST              status register

Following the status register contents on the first display line,  you  see

the  values  of  each bit in the status register, as shown in the following

sample:

TR SUP IM=7 EXT NEG ZER OFL CRY

This sample display includes:

TR              Trace Bit

SUP             Supervisor Mode Bit

IM=7            Interrupt Mask=7

EXT             Extend

NEG             Negative

ZER             Zero

OFL             Overflow

CRY             Carry

The second form, Xr, allows you to change the value in the registers of the

program  being  tested.   The r identifies the register.  SID68 responds by

displaying the current contents of the register, leaving the cursor on that

line.    If you type a Return, the value is not changed.  If you type a new

valid value and then a Return, the register is changed to  the  new  value.

You can change the contents of all registers except the status register.

9.2.17  Assembly Language Syntax for the L Command

In  general,  the  syntax of the assembly language statements used in the L

command is standard  Motorola  68000  assembly  language.    Several  minor

exceptions are given in the following list:

*   SID68 prints all numeric values in hexadecimal.

*   SID68 uses lowercase mnemonics.

*   SID68  assumes  word operations unless a byte or longword specification

is explicitly stated.

End of Section 9

SUMMARY OF BIOS FUNCTIONS

Table A-1 lists the BIOS functions supported by GEM DOS.

Table A-1.  Summary of BIOS Functions

00   Get Memory Parameter Block

VOID      Get_MPB(p_MPB)

MPB       *p_MPB;

{

Fill in Memory Parameter Block at p_MPB.

}

01  Character input status

LONG    Char_In_Stat(handle)

WORD    handle;

{

if (device is ready)

return (-1);

return (0);

}

02  Character input

LONG    Char_Input(handle)

WORD    handle;

{

return (character for device 'handle');

03  Character output

VOID    Char_output(handle, char)

WORD    handle, char;

{

Output 'char' to device 'handle'.  Character may be

output directly for polled output, or enqued for

interrupt driven output.

}

Table A-1.  (continued)

04  Read/Write disk sectors

LONG    Read_Write_sectors(wrtflg, buffer, count, begin, drive)

WORD    wrtflg;           /* 0: read, 1: write */

BYTE    *buffer;          /* address of transfer buffer */

WORD    count;            /* number of sectors to transfer */

WORD    begin;            /* beginning sector number */

WORD    drive;            /* 0: A, 1: B, ... */

{

Perform indicated disk operation.

return (error indication or E_OK);

}

05  Get/Set exception vector

LONG    Get_Set_Vector(n, addr)

WORD    n;                /* vector number for operation  */

LONG    addr;             /* address/parameter value      */

{

temp = vector[n];

if (addr != -1L)

vector[n] = addr;

return (temp);

}

Extended vectors

0x101   Critical error handler

It is the responsibility of the BIOS to invoke the

routine at this vector whenever a disk related critical

error occurs, such as write fault, drive not read, etc.

It is the responsibility of the handler that installs

itself to save d3-d7/a3-a7 if they are used.  The error

number of the critical error is a WORD parameter on the

stack.

To ignore the error, set D0.L to 0 and return.

To retry, set D0.L to 0x10000 and return.

To abort, move error parameter to D0 and sign

extend it to a LONG.

Table A-1.  (continued)

If the target system is intended to support character-

based (that is, non-GEM related) applications, it is the

responsibility of the BIOS implementor to make the console

I/O portion of the BIOS re-entrant from this handler

so that user prompting can be done.

0x102   Terminate handler

It is the responsibility of the handler to determine whether

it should allow the process termination to continue.

To terminate, simply RTS. Otherwise, 'longjump' back

into the main code of the process.

06  Get tick info

LONG    get_ticks()

{

return (number of milliseconds per tick);

}

07  Get BIOS Parameter Block

BPB     *get_bpb(drive)

WORD    drive;

{

If multiple media types are supported on 'drive',

determine type of media.

return(pointer to BPB for this disk);

}

08  Character output status

LONG    Char_Out_Stat(handle)

WORD    handle;

{

If (device is ready for output)

return (-1);

return (0);

}

Table A-1.  (continued)

09  Media change

LONG    Media_Change(drive)

WORD    drive;

{

if (media has not changed)

return (0);

if (media may have changed)

return (1);

if (media has definitely changed)

return (2);

}

0A  Get drive map

LONG    Get_Drive_Map()

{

return (bit map of accessible drives on system)

}

Where bit 0 represents drive A, etc.

NOTE:  If the BIOS supports logical drives A and B on a single physical

it should return both bits set if a floppy disk is present.

0B  Get/Set shift keys (console)

LONG    Get_Set_Shift(flag)

WORD    flag;

{

temp = state;

if (flag != -1)

state = flag;

return (temp);

}

The bits in 'flag' are assigned to be:

0:        Right shift key

1:        Left shift key

2:        Control key

3:        Alt key

4:        Caps lock key

Table A-1.  (continued)

0C  Character control input

LONG    Char_Ctl_In(handle, length, buffer)

WORD    handle, length;

BYTE    *buffer;

{

Read up to 'length' bytes from 'handle's control

channel into 'buffer.

Called with 'length' = 0 to determine if

device accepts control strings.

}

0D  Character control output

LONG    Char_Ctl_Out(handle, length, buffer)

WORD    handle, length;

BYTE    *buffer;

{

Write 'length' bytes to 'handle's control channel from 'buffer'.

}

0E  Disk control input

LONG    Disk_Ctl_In(drive, length, buffer)

WORD    drive, length;

BYTE    *buffer;

{

Read up to 'length' bytes from 'handle's control channel

into 'buffer'.  Called with 'length' = 0 to determine if

'handle' accepts control strings.

}

0F  Disk control output

LONG    Disk_Ctl_Out(drive, length, buffer)

WORD    drive, length;

BYTE    *buffer;

{

Write 'length' bytes to 'handle's control

channel from 'buffer'.

}

Table A-1.  (continued)

10  Character vector exchange

LONG    Char_Vec_Exchange(handle, address)

WORD    handle;

LONG    address;

{

temp = 'handle's old handler address;

Install 'address' as 'vector's

new logical interrupt handler.

return (temp);

}

It is not strictly necessary to implement

this function for CON: in the current release of

GEM DOS.

Stack format for invocation of character vector handler

sp+8   LONG (see below)

sp+4   LONG flags

sp     LONG return address

Interpretation of bits in 'flags' LONG:

0      Packet received

If this is 1, then the LONG at sp+8 is defined as

For CON:, AUX:, or PRN:

The LONG character info that BIOS

function 0x02 would have returned.

For CLOCK

The number of milliseconds since the

last tick.

For MOUSE

A pointer to a parameter block that contains:

A BYTE of button status

A BYTE of delta-X

A BYTE of delta-Y

1      Error

If no other error status bit is on,

this is just an error that couldn't be

described any better with the available

defined status bits.

2      Out of paper

3      Off line

4      Timeout

5      Framing Error

6      Parity error

7-15   RESERVED

If no packet received and no error then state changed

on one of the following:

16     Carrier detect

17     Clear to send

18-31  RESERVED

Only register A7 must be preserved by a vector handler.

The handler must jump to the next handler in  the  chain  (at  the  address

returned  by  the F_IOCtl character vector exchange function), particularly

for the clock tick.

System Initialization Sequence :

1.  Disable interrupts if necessary.

2.  Perform necessary chip and peripheral  initialization  to  put  the  HW

environment in a quiescent state.

3.  Initialize TRAP #13 vector to the BIOS's handler address.

4.  Initialize sector buffers similar to the following code:

char secbuf[4][512];

BCB bcbx[4];

BCB *bufl[2];

bcbx[0].b_link = &bcbx[1];

bcbx[2].b_link = &bcbx[3];

bcbx[0].b_bufdrv = -1;

bcbx[1].b_bufdrv = -1;

bcbx[2].b_bufdrv = -1;

bcbx[3].b_bufdrv = -1;

bcbx[0].b_bufr = &secbuf[0][0];

bcbx[1].b_bufr = &secbuf[1][0];

bcbx[2].b_bufr = &secbuf[2][0];

bcbx[3].b_bufr = &secbuf[3][0];

bufl[0] = &bcbx[0];      /* fat buffers */

bufl[1] = &bcbx[2];      /* dir/data buffers */

(The  array of BCB pointers, bufl[], which has the linkable name _bufl,

must be a global label. It will be linked with the GEM DOS (tm)  object

modules.)

5.  Invoke osinit() (or '_osinit' from assembler).

6.  Perform BDOS D_SetDrv(default_drive) via TRAP #1.

7.  If  CLI  is  to  be  executed  normally  (that is, loaded from disk and

executed):

Perform BDOS P_Exec(0,"command.prg","","path=default_path\0")

via TRAP #1.

Else, to execute a linked in CLI:

*   Include the modules COMA.O, and COMMAND.O, and LMUL.O when  linking

the system.

*   Perform  BDOS  P_Exec(5,"","","")  via  TRAP  #1, which returns the

address of a partially initialized base page.

*   Move the address of cli() (that is, _cli) to base  page  +  p_tbase

(8).

*   Perform BDOS P_Exec(4,"","","path=default_path\0").

Linking  the  CLI  with  the  system  image allows new types of media to be

'INIT'ed from the command line, provided that formatting has  already  been

done.

Note:    When  the  system is linked, the label _GSX_ENT will be undefined.

This does not affect the operation of the system.

If the system is to be generated  for  68010  (or  similar)  hardware,  the

module RWA10.O should be used instead of RWA.O.

End of Appendix A

BASE PAGE FORMAT

Table  B-1  shows  the  format of the base page.  The base page describes a

program's operating environment.  The Load and Execute a  Process  function

(4B)  initializes  the base page when invoked to load an executable command

file.  For more details on the Load and  execute  a  Process  function  and

command files, refer to the appropriate sections of this guide.

Table B-1.  Base Page Format:  Offsets and Contents

Offset

00

04

08

0C

10

14

18

1C

2C

80

the program

Note  that  all  other base page fields are reserved and not to be tampered

with.

End of Appendix B

ASCII AND HEXADECIMAL CONVERSIONS

ASCII stands for American Standard Code for Information Interchange.    The

code contains 96 printing and 32 non-printing characters used to store data

on a disk.   Table  C-1  defines  ASCII  symbols.    Table  C-2  shows  the

conversions of the ASCII character code in binary, decimal, and hexadecimal

notations.

Table C-1.  ASCII Symbols

Symbol      Meaning                   Symbol

ACK   acknowledge        FS      file separator

BEL   bell               GS      group separator

BS    backspace          HT      horizontal tabulation

CAN   cancel             LF      line feed

CR    carriage return    NAK     negative acknowledge

DC    device control     NUL     null

DEL   delete             RS      record separator

DLE   data link escape   SI      shift in

EM    end of medium      SO      shift out

ENQ   enquiry            SOH     start of heading

EOT   end of transmission        SP

ESC   escape             STX     start of text

ETB   end of transmission        SUB

ETX   end of text        SYN     synchronous idle

FF    form feed          US      unit separator

VT      vertical tabulation

Table C-2.  ASCII Conversion Table

Binary       Decimal                 Hexadecimal

0000000       0           0           NUL

0000001       1           1           SOH (CTRL-A)

0000010       2           2           STX (CTRL-B)

0000011       3           3           ETX (CTRL-C)

0000100       4           4           EOT (CTRL-D)

0000101       5           5           ENQ (CTRL-E)

0000110       6           6           ACK (CTRL-F)

0000111       7           7           BEL (CTRL-G)

0001000       8           8           BS  (CTRL-H)

0001001       9           9           HT  (CTRL-I)

0001010       10          A           LF  (CTRL-J)

0001011       11          B           VT  (CTRL-K)

0001100       12          C           FF  (CTRL-L)

0001101       13          D           CR  (CTRL-M)

0001110       14          E           SO  (CTRL-N)

0001111       15          F           SI  (CTRL-O)

0010000       16          10          DLE (CTRL-P)

0010001       17          11          DC1 (CTRL-Q)

0010010       18          12          DC2 (CTRL-R)

0010011       19          13          DC3 (CTRL-S)

0010100       20          14          DC4 (CTRL-T)

0010101       21          15          NAK (CTRL-U)

0010110       22          16          SYN (CTRL-V)

0010111       23          17          ETB (CTRL-W)

0011000       24          18          CAN (CTRL-X)

0011001       25          19          EM  (CTRL-Y)

0011010       26          1A          SUB (CTRL-Z)

0011011       27          1B          ESC (CTRL-[)

0011100       28          1C          FS  (CTRL-\)

0011101       29          1D          GS  (CTRL-])

0011110       30          1E          RS  (CTRL-^)

0011111       31          1F          US  (CTRL-_)

0100000       32          20          (SPACE)

Table C-2.  (continued)

Binary       Decimal                 Hexadecimal

0100001       33          21          !

0100010       34          22          "

0100011       35          23          #

0100100       36          24          $

0100101       37          25          %

0100110       38          26          &

0100111       39          27          '

0101000       40          28          (

0101001       41          29          )

0101010       42          2A          *

0101011       43          2B          +

0101100       44          2C          ,

0101101       45          2D          -

0101110       46          2E          .

0101111       47          2F          /

0110000       48          30          0

0110001       49          31          1

0110010       50          32          2

0110011       51          33          3

0110100       52          34          4

0110101       53          35          5

0110110       54          36          6

0110111       55          37          7

0111000       56          38          8

0111001       57          39          9

0111010       58          3A          :

0111011       59          3B          ;

0111100       60          3C          <

0111101       61          3D          =

0111110       62          3E          >

0111111       63          3F          ?

1000000       64          40          @

Table C-2.  (continued)

Binary       Decimal                 Hexadecimal

1000001       65          41          A

1000010       66          42          B

1000011       67          43          C

1000100       68          44          D

1000101       69          45          E

1000110       70          46          F

1000111       71          47          G

1001000       72          48          H

1001001       73          49          I

1001010       74          4A          J

1001011       75          4B          K

1001100       76          4C          L

1001101       77          4D          M

1001110       78          4E          N

1001111       79          4F          O

1010000       80          50          P

1010001       81          51          Q

1010010       82          52          R

1010011       83          53          S

1010100       84          54          T

1010101       85          55          U

1010110       86          56          V

1010111       87          57          W

1011000       88          58          X

1011001       89          59          Y

1011010       90          5A          Z

1011011       91          5B          [

1011100       92          5C          \

1011101       93          5D          ]

1011110       94          5E          ^

1011111       95          5F          <

1100000       96          60          '

Table C-2.  (continued)

Binary       Decimal                 Hexadecimal

1100001       97          61          a

1100010       98          62          b

1100011       99          63          c

1100100       100         64          d

1100101       101         65          e

1100110       102         66          f

1100111       103         67          g

1101000       104         68          h

1101001       105         69          i

1101010       106         6A          j

1101011       107         6B          k

1101100       108         6C          l

1101101       109         6D          m

1101110       110         6E          n

1101111       111         6F          o

1110000       112         70          p

1110001       113         71          q

1110010       114         72          r

1110011       115         73          s

1110100       116         74          t

1110101       117         75          u

1110110       118         76          v

1110111       119         77          w

1111000       120         78          x

1111001       121         79          y

1111010       122         7A          z

1111011       123         7B          {

1111100       124         7C          |

1111101       125         7D          }

1111110       126         7E          ~

1111111       127         7F          DEL

End of Appendix C

ERROR MESSAGES

This appendix lists the error messages returned by the internal  components

of  GEM  DOS  and  by  the GEM DOS programmer's utilities.  subsections are

arranged alphabetically by the name of the internal component  or  utility.

Each subsection has error messages listed alphabetically, with explanations

and suggested user responses.

D.1. AS68 ERROR MESSAGES

The GEM DOS  assembler,  AS68,  returns  both  nonfatal,  diagnostic  error

messages  and  fatal error messages. Fatal errors stop the assembly of your

program.  There are two types of  fatal  errors:    user-recoverable  fatal

errors and fatal errors in the internal logic of AS68.

D.1.1. AS68 Diagnostic Error Messages

Diagnostic  messages report errors in the syntax and context of the program

being assembled without interrupting  assembly.    Refer  to  the  Motorola

16-Bit  Microprocessor  User's Manual for a full discussion of the assembly

language syntax.

Diagnostic error messages appear in the following format:

& line no.  error message text

The ampersand (&) indicates that the message comes from AS68.  The line no.

indicates  the line in the source code where the error occurred.  The error

message text describes the error.  Diagnostic error messages appear at  the

console  after  assembly, followed by a message indicating the total number

of errors.  In a print-out, they print on the  line  preceding  the  error.

Table D-1 lists the AS68 diagnostic error messages in alphabetical order.

Table D-1.  AS68 Diagnostic Error Messages

Message   Meaning

& line no.  backward assignment to *

The assignment statement in the  line  indicated  illegally

assigns  the  location  counter  (*)  backward.  Change the

location counter to a forward assignment and reassemble the

source file.

Table D-1.  (continued)

Message   Meaning

& line no.  bad use of symbol

A symbol in the source line indicated has been  defined  as

both  global  and common.  A symbol can be either global or

common, but not both.  Delete one  of  the  directives  and

reassemble the source file.

& line no.  constant required

An expression on the line indicated  requires  a  constant.

Supply a constant and reassemble the source file.

& line no.  end statement not at end of source

The end statement must be at the end of  the  source  code.

The  end  statement cannot be followed by a comment or more

than one carriage return.  Place the end statement  at  the

end  of the source code, followed only by a single carriage

return, and reassemble the source file.

& line no.  illegal addressing mode

The  instruction  on  the  line  indicated  has  an invalid

addressing  mode.  Provide  a  valid  addressing  mode  and

reassemble the source file.

& line no.  illegal constant

The line indicated contains an illegal constant.  Supply  a

valid constant and reassemble the source file.

& line no.  illegal expr

The line indicated contains an illegal expression.  Correct

the expression and reassemble the source file.

& line no.  illegal external

The line indicated illegally contains an external reference

to  an  8-bit  quantity.  Rewrite the source code to define

the  reference  locally  or  use  a  16-bit  reference  and

reassemble the source file.

& line no.  illegal format

An expression or  instruction  in  the  line  indicated  is

illegally  formatted.    Examine  the line.  Reformat where

necessary and reassemble the source file.

& line no.  illegal index register

The  line  indicated  contains  an  invalid index register.

Supply a valid register and reassemble the source file.

Table D-1.  (continued)

Message   Meaning

& line no.  illegal relative address

An   addressing   mode  specified  is  not  valid  for  the

instruction in the line indicated.  Refer to  the  Motorola

16-Bit  Microprocessor  User's  Manual  for  valid register

modes for the specified instruction.   Rewrite  the  source

code to use a valid mode and reassemble the file.

& line no.  illegal shift count

The instruction in the line  indicated  shifts  a  quantity

more  than 31 times.  Modify the source code to correct the

error and reassemble the source file.

& line no.  illegal size

The  instruction  in the line indicated requires one of the

following three size specifications:  b (byte),  w  (word),

or l (longword).  Supply the correct size specification and

reassemble the source file.

& line no.  illegal string

The line indicated contains an illegal string.  Examine the

line.  Correct the string and reassemble the source file.

& line no.  illegal text delimiter

The  text  delimiter  in the line indicated is in the wrong

format.   Use  single  quotes  ('text')  or  double  quotes

("text")  to  delimit  the  text  and reassemble the source

file.

& line no.  illegal 8-bit displacement

The line indicated illegally contains a displacement larger

than 8-bits.  Modify the code  and  reassemble  the  source

file.

& line no.  illegal 8-bit immediate

The line indicated illegally contains an immediate  operand

larger  than  8-bits.    Use  the 16- or 32-bit form of the

instruction and reassemble the source file.

& line no.  illegal 16-bit displacement

The line indicated illegally contains a displacement larger

than 16-bits.  Modify the code and  reassemble  the  source

file.

& line no.  illegal 16-bit immediate

The line indicated illegally contains an immediate  operand

larger   than   16-bits.    Use  the  32-bit  form  of  the

instruction and reassemble the source file.

Table D-1.  (continued)

Message   Meaning

& line no.  invalid data list

One  or more entries in the data list in the line indicated

is invalid.   Examine  the  line  for  the  invalid  entry.

Replace  it  with  a  valid entry and reassemble the source

file.

& line no.  invalid first operand

The first operand in an expression in the line indicated is

invalid.  Supply a valid operand and reassemble the  source

file.

& line no.  invalid instruction length

The instruction in the line indicated requires one  of  the

following  three  size specifications:  b (byte), w (word),

or l (longword).  Supply the correct size specification and

reassemble the source file.

& line no.  invalid label

A required operand is not present in the line indicated, or

a label reference in the line is not in the correct format.

Supply a valid label and reassemble the source file.

& line no.  invalid opcode

The  opcode  in  the  line  indicated  is  non-existent  or

invalid.  Supply a valid opcode and reassemble  the  source

file.

& line no.  invalid second operand

The second operand in an expression in the  line  indicated

is  invalid.    Supply  a  valid operand and reassemble the

source file.

& line no.  label redefined

This message indicates that a label has been defined twice.

The  second  definition  occurs  in  the  line   indicated.

Rewrite  the source code to specify a unique label for each

definition and reassemble the source file.

& line no.  missing )

An  expression  in  the  line  indicated is missing a right

parenthesis.  Supply the missing parenthesis and reassemble

the source file.

& line no.  no label for operand

An operand in  the  line  indicated  is  missing  a  label.

Supply a label and reassemble the source file.

Table D-1.  (continued)

Message   Meaning

& line no.  opcode redefined

A label in the line indicated has the same mnemonics  as  a

previously  specified  opcode.  Respecify the label so that

it does not have the same spelling as the mnemonic for  the

opcode.  Reassemble the source file.

& line no.  register required

The instruction in the line  indicated  requires  either  a

source  or  destination  register.   Supply the appropriate

register and reassemble the source file.

& line no.  relocation error

An  expression in the line indicated contains more than one

externally defined global symbol.  Rewrite the source code.

Either  make one of the externally defined global symbols a

local symbol, or evaluate the expression within  the  code.

Reassemble the source file.

& line no.  symbol required

A statement  in  the  line  indicated  requires  a  symbol.

Supply a valid symbol and reassemble the source file.

& line no.  undefined symbol in equate

One of the symbols in the  equate  directive  in  the  line

indicated  is  undefined.  Define the symbol and reassemble

the source file.

& line no.  undefined symbol

The  line  indicated  contains an undefined symbol that has

not been declared global.  Either define the symbol  within

the  module  or define it as a global symbol and reassemble

the source file.

D.1.2. User-recoverable Fatal Error Messages

Table D-2 describes fatal error messages for AS68.  When  an  error  occurs

because  the  disk is full, AS68 creates a partial file.  Erase the partial

file to ensure that you do not try to link it.

Table D-2.  AS68 User-recoverable Fatal Error Messages

Message   Meaning

&    cannot create init: AS68SYMB.DAT

AS68 cannot create the initialization file because the path

name is incorrect or the disk to which it was  writing  the

file  is  full.   If you used the -S switch to redirect the

symbol table to another disk, check the path name.   If  it

is  correct, the disk is full.  Erase unnecessary files, if

any, or insert a new disk  before  you  reinitialize  AS68.

Erase the partial file that was created on the full disk to

ensure that you do not try to link it.

&    expr opstk overflow

An  expression  in  the  line  indicated  contains too many

operations  for  the  operations  stack.     Simplify   the

expression before you reassemble the source code.

&    expr tree overflow

The expression tree does not have space for the  number  of

terms  in  one  of the expressions in the indicated line of

source code.  Rewrite the expression  to  use  fewer  terms

before you reassemble the source file.

&    I/O error on loader output file

The disk to which AS68 was writing the loader  output  file

is  full.    AS68  wrote a partial file.  Erase unnecessary

files, if any, or insert a  new  disk  and  reassemble  the

source  file.    Erase the partial file that was created on

the full disk to ensure that you do not try to link it.

&    I/O write error on it file

The  disk  to  which AS68 was writing the intermediate text

file  is  full.    AS68  wrote  a  partial  file.     Erase

unnecessary  files,  if  any,  or  insert  a  new  disk and

reassemble the source file.  Erase the  partial  file  that

was  created on the full disk to ensure that you do not try

to link it.

&    it read error itoffset= no.

The  disk  to  which AS68 was writing the intermediate text

file is full.  AS68 wrote a partial  file.    The  variable

itoffset= no. indicates the first zero-relative byte number

not read.  Erase unnecessary files, if any, or insert a new

disk  and  reassemble  the  source file.  Erase the partial

file that was created on the full disk to ensure  that  you

do not try to link it.

Table D-2.   (continued)

Message   Meaning

&    Object file write error

The disk to which AS68 was writing the object file is full.

AS68  wrote  a  partial  file.  Erase unnecessary files, if

any, or insert a new disk and reassemble the  source  file.

Erase the partial file that was created on the full disk to

ensure that you do not try to link it.

&    overflow of external table

The  source  code  uses  too many externally defined global

symbols  for  the  size  of  the  external  symbol   table.

Eliminate   some  externally  defined  global  symbols  and

reassemble the source file.

&    Read Error On Intermediate File: ASXXXXn

The  disk  to  which AS68 was writing the intermediate text

file ASXXXX is full.  AS68  wrote  a  partial  file.    The

variable  n indicates the drive on which ASXXXX is located.

Erase unnecessary files, if any, or insert a new  disk  and

reassemble  the  source  file.  Erase the partial file that

was created on the full disk to ensure that you do not  try

to link it.

&    symbol table overflow

The program uses too many symbols  for  the  symbol  table.

Eliminate  some  symbols  before  you reassemble the source

code.

&    Unable to open file filename

The  source  filename indicated by the variable filename is

invalid or has an invalid path name.  Check the  path  name

and  the  filename.   Respecify the command line before you

reassemble the source file.

&    Unable to open input file

The  filename  in the command line indicated does not exist

or has an invalid path name. Check the path  name  and  the

filename.  Respecify the command line before you reassemble

the source file.

&    Unable to open temporary file

You used an invalid path name or the disk to which AS68 was

writing is full.  Check the path name.  If it  is  correct,

the  disk  is  full.    Erase unnecessary files, if any, or

insert a new disk before you reassemble the source file.

Table D-2.   (continued)

Message   Meaning

&    Unable to read init file: AS68SYMB.DAT

The  path  name  used to specify the initialization file is

invalid or the assembler has not been initialized.    Check

the  path  name.    Respecify  the  command line before you

reassemble the source file.  If the assembler has not  been

initialized, refer to Section 5 for instructions.

&    Write error on init file: AS68SYMB.DAT

The disk to which AS68 was writing the initialization  file

is  full.    AS68  wrote a partial file.  Erase unnecessary

files, if any, or insert a  new  disk  and  reassemble  the

source  file.    Erase the partial file that was created on

the full disk to ensure that you do not try to link it.

&    write error on it file

The disk to which AS68 was writing the intermediate text is

full.  AS68 wrote a partial file.  Erase unnecessary files,

if  any, or insert a new disk.  Erase the partial file that

was created on the full disk to ensure that you do not  try

to link it.  Reassemble the source file.

D.1.3. AS68 Internal Logic Error Messages

The following are messages indicating fatal errors in the internal logic of

AS68:

&  doitrd:  buffer botch pitix=nnn itbuf=nnn end=nnn

&  doitwr: it buffer botch

&  invalid radix in oconst

&  i.t. overflow

&  it sync error itty=nnn

&  seek error on it file

&  outword: bad rlflg

If you receive one of these messages, contact your vendor  for  assistance.

You should do the following:

*   Indicate which version of the operating system you are using.

*   Describe your system's hardware configuration.

*   Provide  sufficient information to reproduce the error.  Indicate which

program was running at the time the error occurred.  If  possible,  you

should also provide a disk with a copy of the program.

D.2. LO68 ERROR MESSAGES

The  GEM  DOS  Linker,  LO68,  returns  two  types of fatal error messages:

diagnostic and logic.    Both  types  of  fatal  error  messages  have  the

following format:

:  error message text

The colon (:) indicates that the error message comes from LO68.  The "error

message text" describes the error.

D.2.1. Fatal Diagnostic Error Messages

A fatal diagnostic error prevents your program  from  linking.    When  the

error is caused by a full disk, erase the partial file that LO68 created on

the disk that received the error to ensure that you do not  use  the  file.

The LO68 diagnostic errors are listed in Table D-3 in alphabetic order with

explanations and suggested user responses.

Table D-3.  LO68 Fatal Diagnostic Error Messages

Message   Meaning

: duplicate definition in p,filename

The  symbol  indicated  by the variable p is defined twice.

The variable filename  indicates  the  file  in  which  the

second  definition  occurred.    Rewrite  the  source code.

Provide a unique definition for each symbol and  reassemble

or recompile the source code before you relink the file.

: file format error:  filename

The file indicated by the variable filename is  either  not

an object file or the file has been corrupted.  Ensure that

the file is an object file,  output  by  the  assembler  or

compiler.    Reassemble  or  recompile  the file before you

relink it.

: File Format Error:  Invalid symbol flags = flags

LO68  does  not recognize the symbol flags indicated by the

variable flags.  The file LO68 read is either not an object

file  or  has  been  corrupted.  Ensure that the file is an

object  file,  output  by  the   assembler   or   compiler.

Reassemble or recompile the file before you relink it.

: File Format Error:  invalid relocation flag in filename

The contents of the file indicated by the variable filename

are  incorrectly  formatted.    The  file  either is not an

object file or has been corrupted.  Ensure that the file is

an  object  file,  output by the assembler or compiler.  If

the file is an object file but this error occurs, the  file

has  been  corrupted.    Reassemble  or  recompile the file

before you relink it.

Table D-3.   (continued)

Message   Meaning

: File Format Error:  no relocation bits in filename

The  file  indicated by the variable filename either is not

an object file or has been corrupted.  Ensure that the file

is an object file, output by the assembler or compiler.  If

the file is an object file but this error occurs, then  the

file  has been corrupted.  Reassemble or recompile the file

before you relink it.

: Illegal option p

The  option in the command line indicated by the variable p

is invalid.  Supply a valid option and relink.

: Invalid lo68 argument list

This  message indicates format errors or invalid options in

the command line.  Examine the command line to  locate  the

error.  Correct the error and relink.

: output file write error

The  disk  to  which  LO68  is  writing  is  full.    Erase

unnecessary  files, if any, or insert a new disk before you

reenter the LO68 command line.

: read error on file:  filename

The object file indicated by the variable filename does not

have  enough  bytes.    The  file  either  is   incorrectly

formatted  or  has  been corrupted.  This error is commonly

caused when the input to LO68 is a partially  assembled  or

compiled  object  file.    The  assembler,  AS68,  and some

compilers create partial object files when they receive the

disk  full  abort  message  while assembling or compiling a

file.  Ensure that the file  is  a  complete  object  file.

Reassemble or recompile the file before you relink it.

: symbol table overflow

The object code contains too many symbols for the  size  of

the  symbol  table.    Rewrite the source code to use fewer

symbols.  Reassemble or recompile the  source  code  before

you relink the file.

: Unable to create pathname

The output file indicated by pathname has an  invalid  path

name,  or the disk to which LO68 is writing is full.  Check

the path name.  If it is correct, the disk is full.   Erase

unnecessary  files, if any, or insert a new disk before you

reenter the LO68 command line.

Table D-3.   (continued)

Message   Meaning

: unable to open filename

The  filename indicated by the variable filename is invalid

or the file does not exist.  Check the filename before  you

reenter the LO68 command line.

: Unable to open temporary file:  pathname

Either the file, indicated  by  pathname,  has  an  invalid

drive  code in the path name, specified by the f option, or

the disk to which LO68 is writing is full.  Check the  path

name.    If  it  is  correct,  the  disk  is  full.   Erase

unnecessary files, if any, or insert a new disk before  you

reenter the LO68 command line.

: Undefined symbol(s)

The symbol  or  symbols  which  are  listed  one  per  line

following the error message are undefined.  Provide a valid

definition  and  reassemble  the  source  code  before  you

reenter the LO68 command line.

D.2.2. LO68 Internal Logic Error Messages

This  section  lists messages indicating fatal errors in the internal logic

of LO68.  If you receive one of  these  messages,  contact  the  place  you

purchased your system for assistance.  You should do the following:

*   Indicate which version of the operating system you are using.

*   Describe your system's hardware configuration.

*   Provide  sufficient information to reproduce the error.  Indicate which

program was running at the time the error occurred.  If  possible,  you

should also provide a disk with a copy of the program.

Errors:

: asgnext botch

: finalwr: text size error

: relative address overflow at lx in sn

: seek error on file  filename

: short address overflow in  filename

: unable to reopen filename

D.3. AR68 ERROR MESSAGES

The  GEM  DOS  Archive  utility,  AR68,  returns  two  types of fatal error

messages:  diagnostic and logic.  Both types of fatal error  messages  show

at the console as they occur.

D.3.1. Fatal Diagnostic Error Messages

Table  D-4  lists  AR68  fatal  error  messages  in alphabetical order with

explanations and suggested user responses.

Table D-4.  AR68 Fatal Diagnostic Error Messages

Message   Meaning

filename not in archive file

The object module indicated by the variable filename is not

in the library.  Check the filename before you reenter  the

command line.

cannot create filename

The path name  for  the  file  indicated  by  the  variable

filename  is  invalid, or the disk to which AR68 is writing

is full.  Check the path name.  If it is valid, the disk is

full.    Erase  unnecessary  files, if any, or insert a new

disk before you reenter the command line.

cannot open filename

The  file  indicated  by  the  variable  filename cannot be

opened because the filename or the path name is  incorrect.

Check the path name and the filename before you reenter the

command line.

invalid option flag:  x

The symbol, letter, or number in the command line indicated

by the variable x is an invalid option.  Refer Section 3 of

this  manual  for  an  explanation of the AR68 command line

options.  Specify a valid option and  reenter  the  command

line.

not archive format: filename

The file indicated  by  the  variable  filename  is  not  a

library.    Ensure  that you are using the correct filename

before you reenter the command line.

not object file:  filename

The  file  indicated  by  the  variable  filename is not an

object file, and cannot be added to the library.  Any  file

added  to the library must be an object file, output by the

assembler, AS68, or the compiler.  Assemble or compile  the

file before you reenter the AR68 command line.

Table D-4.   (continued)

Message   Meaning

one and only one of DRTWX flags required

The AR68 command line requires one of the D, R, T, W, or  X

commands,  but not more than one.  Reenter the command line

with the correct command.    Refer  to  Section  7  for  an

explanation of the AR68 commands.

filename not in library

The object module indicated by the variable filename is not

in  the  library.    Ensure  that  you  are  requesting the

filename of an existing object module  before  you  reenter

the command line.

Read error on filename

The file indicated by the variable filename cannot be read.

This message means one of three things:  the file listed at

filename is corrupted; a hardware error  has  occurred;  or

when  the file was created, it was not correctly written by

AR68 due to an error in the internal logic of AR68.

Cold start the system and retry  the  operation.    If  you

receive  this  error  message  again,  you  must  erase and

recreate the file.  Use your backup file, if you maintained

one.  If the error reoccurs, check for a hardware error. If

the error persists, contact the the vendor of  your  system

for assistance.  You should do the following:

*   Indicate  which version of the operating system you are

using.

*   Describe your system's hardware configuration.

*   Provide sufficient information to reproduce the  error.

Indicate  which  program  was  running  at the time the

error occurred.  If possible, you should also provide a

disk with a copy of the program.

Table D-4.   (continued)

Message   Meaning

temp file write error

The disk to which AR68 was writing the  temporary  file  is

full.    Erase  unnecessary  files, if any, or insert a new

disk before you reenter the command line.

usage:  AR68  DRTWX[AV][F  D:]   [OPMOD]   ARCHIVE   OBMOD1

[OBMOD2...][>filespec]

This  message indicates a syntax error in the command line.

The correct format for the command line is given, with  the

possible  options  in  brackets.   Refer to Section 7 for a

more detailed explanation of the command line.

Write error on filename

The disk to which AR68 is writing the file indicated by the

variable filename is full.   Erase  unnecessary  files,  if

any,  or  insert  a new disk before you reenter the command

line.

D.3.2. AR68 Internal Logic Error Messages

The following are messages that indicate fatal errors in the internal logic

of AR68:

cannot reopen filename

seek error on library

Seek error on tempname

Unable to recreate--library is in filename

If  you  receive any of these messages except the last, contact your vendor

for assistance. You should do the following:

*   Indicate which version of the operating system you are using.

*   Describe your system's hardware configuration.

*   Provide sufficient information to reproduce the error.  Indicate  which

program  was  running at the time the error occurred.  If possible, you

should also provide a disk with a copy of the program.

For the last error, Unable to recreate--library is in filename, you  should

rename  the  temporary  file indicated by the variable filename.  AR68 used

the library to create the temporary file, then deleted the library in order

to replace it with the updated temporary file.  This error occurred because

AR68 cannot write the temporary file back to the original  location.    The

entire library is in the temporary file.

D.4. DUMP ERROR MESSAGES

DUMP  returns  fatal,  diagnostic error messages at the console.  Table D-5

lists the DUMP error messages in alphabetical order with  explanations  and

suggested user responses.

Table D-5.  DUMP Error Messages

Message   Meaning

Unable to open filename

Either the path name for the input file  indicated  by  the

variable   filename   is  incorrect,  or  the  filename  is

misspelled. Check the filename and  path  name  before  you

reenter the DUMP command line.

Usage:  dump [-shhhhhh] file

The command line syntax is incorrect. The correct syntax is

given  in  the error message.  Specify the DUMP command and

the filename.  If you want to display the contents  of  the

file  from  a  specific address in the file, specify the -S

option followed by the address.  Refer to Section  8.2  for

discussion of the DUMP command line and options.

D.5. SIZE68 ERROR MESSAGES

SIZE68  returns fatal, diagnostic error messages at the console.  Table D-6

lists the SIZE68 error messages in alphabetical order with explanations and

suggested user responses.

Table D-6.  SIZE68 Error Messages

Message   Meaning

File format error:  filename

The file indicated by the variable filename is  neither  an

object  file nor a command file.  SIZE68 requires either an

object file, output by the assembler or the compiler, or  a

command  file,  output by the linker.  Ensure that the file

specified is one of these and reenter  the  SIZE68  command

line.

read error on filename

The file indicated by the variable filename  is  truncated.

Rebuild the file.  Reassemble or recompile, then relink the

source file before you reenter the SIZE68 command line.

unable to open filename

Either the path name is incorrect, or the file indicated by

the variable filename does not exist.  Check the path  name

and filename. Reenter the SIZE68 command line.

D.6. SENDC68 ERROR MESSAGES

SENDC68 returns two types of fatal error messages:  diagnostic and internal

logic error messages.

D.6.1. Diagnostic Error Messages

Table D-7 lists the SENDC68 diagnostic error messages in alphabetical order

with explanations and suggested user responses.

Table D-7.  SENDC68 Diagnostic Error Messages

Message   Meaning

file format error:  filename

The file indicated  by  the  variable  filename  is  not  a

command  file.  The file input to SENDC68 must be a command

file, output by the linker (LO68).  Ensure  that  the  file

specified is a command file.

read error on file:  filename

The file indicated by the variable filename  is  truncated.

Rebuild the file by recompiling or reassembling, and relink

it before you reenter the SENDC68 command line.

unable to create filename

This  message indicates an invalid path name for the output

file indicated by the variable filename.  It can also  mean

that  the  disk to which SENDC68 is writing is full.  Check

the path name.  If it is correct, the disk is  full.  Erase

unnecessary  files, if any, or insert a new disk before you

reenter the SENDC68 command line.

unable to open filename

The  input file indicated by the variable filename does not

exist.  Check the filename and retype the  SENDC68  command

line.

Usage:  sendc68 [-] commandfile [outputfile]

This message  indicates  a  syntax  error  in  the  SENDC68

command  line.    The  correct syntax is given in the error

message.  Retype the command line using the correct syntax.

D.6.2. SENDC68 Internal Logic Error Messages

The following is a message that indicates a fatal  error  in  the  internal

logic of SENDC68:

seek error on file  filename

If  you  receive  this  message,  contact  your  vendor.  You should do the

following:

*   Indicate which version of the operating system you are using.

*   Describe your system's hardware configuration.

*   Provide sufficient information to reproduce the error.  Indicate  which

program  was  running at the time the error occurred.  If possible, you

should also provide a disk with a copy of the program.

D.7. SID68 ERROR MESSAGES

The GEM DOS debugger, SID68, returns two types of error messages:  nonfatal

diagnostic  error  messages,  and  messages  indicating fatal errors in the

internal logic of SID68.

D.7.1. Diagnostic Error Messages

Diagnostic error messages are returned at the console as the errors  occur.

Table  D-8  lists  the  SID68  error  messages  in  alphabetical order with

explanations and suggested user responses.

Table D-8.  SID68 Diagnostic Error Messages

Message   Meaning

Bad or non-existent RAM at HEX no.

This  error occurs in response to a Set (S), Set Word (SW),

or Set Longword (SL) command.  The message indicates one of

two things:

*   The  memory  location  at  HEX no. is Read-Only, an I/O

port, or non-existent.  Use another location.

*   The memory location is damaged.  Check the hardware.

Bad relocation bits

This  message  is  returned  from  the  BDOS  Program  Load

Function (59), and means one of two things:

*   The command file has been corrupted.  Rebuild the file.

Reassemble  or  recompile  the source file, then relink

the file before you reenter the SID68 command line.

*   The file is linked to an absolute  location  in  memory

that  is  already  occupied by SID68.  Link the file to

another location.

SID68 occupies approximately 20K of memory, and resides

at   the   highest  addresses  within  the  TPA.    The

recommended location for linking your file is the  base

address  of  the TPA + 100H.  BDOS Function 63, Get/Set

TPA Limits, returns the high and low boundaries of  the

TPA.

Table D-8.   (continued)

Message   Meaning

Cannot create file

This error occurs during a Write (W) command.  The disk  to

which   SID68  is  writing  has  no  more  directory  space

available; in effect, the  disk  is  full.    If  you  have

another  drive  available, reenter the W command and direct

the file to the disk on that drive.  If  you  do  not  have

another  drive available, you must exit SID68 (and lose the

contents of memory).  Erase unnecessary files, if  any,  or

insert a new disk.

Cannot open file

This error occurs during a R  command.    It  indicates  an

incorrect  path  name or filename.  Check the path name and

filename before you reenter the command line.

Cannot open program file

This  message  occurs  in  response  to  an  E command.  It

indicates an incorrect path name or filename.    Check  the

path name and filename before you reenter the command line.

ERROR, no program or file loaded

This error message occurs in response to a V  command  when

you specify the command but no file is loaded.  Load a file

before you reenter the V command.  The file can  be  loaded

with  an E or R command, or by specifying the filename when

you invoke SID68.

File too big -- read truncated

This  message occurs during a R command when the file being

read is too large to fit in memory.  SID68 reads  only  the

portion  of  the  file  that  can be read into the existing

memory.  To debug this program,  additional  memory  boards

must be added to the system configuration.

File write error

The disk to which SID68 is writing  is  full  or  the  disk

contains  a  bad  sector.  Retry the command.  If the error

persists,  and  you  have  another  disk  drive  available,

redirect  the  output to the disk on that drive.  If you do

not have another drive available, you must exit SID68.  Use

the  STAT command to check the space on the disk.  If it is

full, erase unnecessary files, if  any,  or  insert  a  new

disk.    Because  the  contents of memory are lost when you

exit SID68, you must reload the file in  memory.    If  the

disk was not full, it has a bad sector.  You should replace

the disk.

Table D-8.   (continued)

Message   Meaning

**illegal size field

This  error  occurs during an L command.  The size field of

the instruction being disassembled has  an  illegal  value.

Use a D command to display the location of the error.  This

error could be caused by one of three things:

*   The memory location being disassembled does not contain

an  instruction.    Ensure that the area selected is an

instruction.  If not, reenter  the  L  command  with  a

correct location.

*   The  size  field of the instruction has been corrupted.

Use the debugging commands in  SID68  to  look  for  an

error  that  causes  the  program  to overwrite itself.

Refer to Section 9 for a complete  description  of  the

SID68 commands and options.

*   An invalid instruction was generated by the compiler or

assembler used to create the  program.    Recompile  or

reassemble the source file before you reinvoke SID68.

Insufficient memory or bad file header

This message occurs in response to an E command.  The error

could be caused by one of three things:

*   The  system  you  are using does not have enough memory

available.  Ensure that the program and SID68 fit  into

the  TPA.    Exit  SID68.    Use  the SIZE68 utility to

display the amount of space your  program  occupies  in

memory.    SID68  is approximately 20K bytes.  The BDOS

Get/Set TPA Limits Function (63) returns the  high  and

low  boundaries  of  the  TPA.    If  you  do  not have

sufficient space in the TPA  to  execute  your  command

file and SID68 simultaneously, additional memory boards

must be added to your system configuration.

Table D-8.   (continued)

Message   Meaning

*   The file is not a  command  file  or  has  a  corrupted

header.   If the command file does not run, but you are

sure that your memory space  is  adequate,  use  the  R

command  to look at the file and check the format.  You

might be trying to debug a file that is not  a  command

file.    If it is a command file, the header might have

been corrupted.  Reassemble  or  recompile  the  source

file  before  you  reenter  the E command line.  If the

error persists, it might be caused by an error  in  the

internal  logic  of  SID68.    Contact  your vendor for

assistance.  You should do the following:

--  Indicate which version of the operating system  you

are using.

--  Describe your system's hardware configuration.

--  Provide  sufficient  information  to  reproduce the

error.  Indicate which program was running  at  the

time  the  error occurred.  If possible, you should

also provide a disk with a copy of the program.

*   The command file you are  debugging  is  linked  to  an

absolute location in memory that is already occupied by

SID68.  SID68 is approximately 20K bytes,  and  usually

resides  in  the  highest  addresses  of  the TPA.  The

recommended location for linking your file is the  base

address of the TPA + 100H.  The BDOS Get/Set TPA Limits

Function (63) returns the high and  low  boundaries  of

the TPA.

Table D-8.   (continued)

Message   Meaning

Read error

This message indicates  one  of  three  things.    Try  the

operation  again.    If the error persists, you have one of

the following problems:

*   A write error at the time the file was  created.    You

must  recreate  the file.  If the error reoccurs, or if

you cannot write to the disk, the disk is bad.

*   A bad disk.  Use PIP or COPY to copy the file from  the

bad  disk  to  a  new  disk.   Any files that cannot be

copied must be recreated or replaced from backup files.

Discard the damaged disk.

*   A  hardware  error.   If the error persists, check your

hardware.

unknown opcode

This  error occurs in response to a L command if the memory

location  being  disassembled  does  not  contain  a  valid

instruction.    The  error might have been caused by one of

three things:

*   You gave the L command the wrong address.  Reenter  the

L command with the correct address.

*   The  file  is  not  a command file.  Make sure that the

file you specify is a command file and  reenter  the  L

command.

*   The  command  file  has  been corrupted.  Reassemble or

recompile the source file before  you  reread  it  into

memory  with  a  Load  for  Execution  (E)  or Read (R)

command, as appropriate.  If the problem persists,  use

the debugging commands in SID68 to look for an error in

the program that causes it to overwrite itself.   Refer

to  the  Section  9  for  a complete description of the

SID68 commands and options.

D.7.2. SID68 Internal Logic Error Messages

The following are messages indicating fatal errors in the internal logic of

SID68:

illegal instruction format #

Unknown program load error

If  you  receive one of these messages, contact your vendor for assistance.

You should do the following:

*   Indicate which version of the operating system you are using.

*   Describe your system's hardware configuration.

*   Provide sufficient information to reproduce the error.  Indicate  which

program  was  running at the time the error occurred.  If possible, you

should also provide a disk with a copy of the program.

End of Appendix D

MOTOROLA S-RECORD FORMAT

The Motorola S-record format is a  method  of  representing  binary  memory

images  in  an  ASCII  form.  The  primary use of S-records is to provide a

convenient form for transporting programs  between  computers.  Since  most

computers have a means of reading and writing ASCII information, the format

is widely applicable. The SENDC68 utility provided with GEM DOS can be used

to convert programs into S-record form.

An  S-record file consists of a sequence of S-records of various types. The

entire content of an S-record is ASCII. When a hexadecimal number needs  to

be  represented  in  an S-record, it is represented by the ASCII characters

for the hexadecimal digits comprising the number.  Each  S-record  contains

the five fields shown in Figure E-1. Table E-1 describes each field.

Field:

+---------+---------+---------+---------+---------+---------+

|    S    |  type   | length  | address |   data  |  cksum  |

+---------+---------+---------+---------+---------+---------+

Size*:

1         1         2     2, 4 or 6   varies      2

* in characters

Figure E-1.  S-record Format

Table E-1.  S-record Field Descriptions

Field           Description

S               The ASCII character S. This signals the beginning of the S-

record.

type            A digit between 0 and 9, represented  in  ASCII,  with  the

exceptions  that  4  and 6 are not allowed. The use of each

type value is explained in Table E-2.

length          The number of character pairs in the record, excluding  the

first  three fields. (That is, one half the number of total

characters in the address, data, and  cksum  fields.)  This

field  has  two hexadecimal digits, representing a one-byte

quantity.

address         The address at which the data portion of the record  is  to

reside  in  memory.  The  data  goes  to  this  address and

successively higher numbered addresses. The length of  this

field  is  determined by the record type in the second byte

of the S-record.

data            A variable length field containing the actual  data  to  be

loaded  into memory. Specify each byte of data as a pair of

hexadecimal digits in ASCII.

cksum           A checksum compute  over  the  length,  address,  and  data

fields. Compute the checksum as follows:

*   add  the  values  of the character pairs in the length,

address, and data fields

*   take the one's complement of the sum

*   drop the most significant byte

Enter the least significant byte value in the  cksum  field

as two ASCII hexadecimal digits.

There  are  eight  types  of  S-records.  They  can  be  divided  into  two

categories: records containing actual data and records used to  define  and

delimit  groups  of data-containing records. Types 1, 2, and 3 are reserved

for records in the first category; types 0, 5, 7, 8, and 9 for reserved for

records  in  the  second category. Types 4 and 6 are not allowed. Table E-2

defines the types.

Note: All byte values in Table E-2 are expressed as  two  ASCII  characters

representing the hexadecimal value.

Table E-2.  S-Record Type Definitions

Type            Meaning

0               This  type  is  a  header record used at the beginning of a

group of S-records. The data field can contain any  desired

identifying  information.  The  address  field is two bytes

long and is normally zero.

1               This type of record contains normal data. The address field

is two bytes long.

2               This type is the same as type 1 except the address field is

3 bytes long.

3               This type is the same as type 1 except the address field is

4 bytes long.

5               This  type indicates the number of type 1, 2, and 3 records

in a group of S-records. The count is placed in the address

field. The data field is empty.

7               This record signals the end of a block of type 3 S-records.

If desired, the 4-byte address field can be used to contain

an  address  at  which  to  pass control. The data field is

empty.

8               This type is the same as type 7 except that it ends a block

of type 2 S-records and the address field is 3 bytes long.

9               This type is the same as type 7 except that it ends a block

of type 1 S-records and the address field is 2 bytes  long.

End of Appendix E

Figures

2-1 GEM DOS TPA Format. . . . . . . . . . . . . . . . . . . . . . .   2-1

2-2 Adjusted TPA. . . . . . . . . . . . . . . . . . . . . . . . . .   2-1

3-1 GEM DOS Command File Layout . . . . . . . . . . . . . . . . . .   3-1

3-2 GEM DOS Command File Header . . . . . . . . . . . . . . . . . .   3-2

3-3 Symbol Table Entry. . . . . . . . . . . . . . . . . . . . . . .   3-2

3-4 Location and Format of Relocation Information . . . . . . . . .   3-4

4-1 Function Call Parameter Block Format. . . . . . . . . . . . . .   4-2

4-2 D0 Contents Returned from C_CONIN . . . . . . . . . . . . . . .   4-7

4-3 C_CONRS Occurrence. . . . . . . . . . . . . . . . . . . . . . .  4-16

4-4 D_FREE Information Return Buffer. . . . . . . . . . . . . . . .  4-48

E-1 S-record Format . . . . . . . . . . . . . . . . . . . . . . . .   E-1

Tables

1-1 GEM DOS System Modules. . . . . . . . . . . . . . . . . . . . .   1-1

1-2 GEM DOS Programming Tools . . . . . . . . . . . . . . . . . . .   1-1

1-3 Delimiter Characters. . . . . . . . . . . . . . . . . . . . . .   1-2

1-4 GEM DOS Terminology . . . . . . . . . . . . . . . . . . . . . .   1-3

1-5 GEM DOS Programmer's Guide Conventions. . . . . . . . . . . . .   1-3

2-1 Base Page Format. . . . . . . . . . . . . . . . . . . . . . . .   2-1

3-1 Symbol Table Entry Fields . . . . . . . . . . . . . . . . . . .   3-2

3-2 Relocation Offset Byte Table. . . . . . . . . . . . . . . . . .   3-4

4-1 GEM DOS System Call Categories. . . . . . . . . . . . . . . . .   4-1

4-2 GEM DOS Device Handles. . . . . . . . . . . . . . . . . . . . .   4-2

4-3 BDOS Error Codes. . . . . . . . . . . . . . . . . . . . . . . .   4-2

4-4 GEM DOS System Call Summary . . . . . . . . . . . . . . . . . .   4-3

4-5 Character Device I/O Functions. . . . . . . . . . . . . . . . .   4-6

4-6 File Access Function Calls. . . . . . . . . . . . . . . . . . .  4-23

4-7 F_IOCTL Calling Restrictions. . . . . . . . . . . . . . . . . .  4-34

4-8 DTA Contents from F_SFIRST Call . . . . . . . . . . . . . . . .  4-40

4-9 Drive and Directory Management Functions. . . . . . . . . . . .  4-45

4-10 System and Program Control Function Calls. . . . . . . . . . .  4-53

5-1 Assembler Options . . . . . . . . . . . . . . . . . . . . . . .   5-2

5-2 AS68 Directives . . . . . . . . . . . . . . . . . . . . . . . .   5-3

5-3 Instruction Set Summary . . . . . . . . . . . . . . . . . . . .   5-9

5-4 Variations of Instruction Types . . . . . . . . . . . . . . . .  5-10

6-1 LINK68 Command-line Options . . . . . . . . . . . . . . . . . .   6-1

6-2 LO68 Options. . . . . . . . . . . . . . . . . . . . . . . . . .   6-5

7-1 AR68 Command Line Components. . . . . . . . . . . . . . . . . .   7-2

8-1 DUMP Command Line Components. . . . . . . . . . . . . . . . . .   8-2

9-1 SID68 Command Summary . . . . . . . . . . . . . . . . . . . . .   9-1

A-1 Summary of BIOS Functions . . . . . . . . . . . . . . . . . . .   A-1

B-1 Base Page Format:  Offsets and Contents . . . . . . . . . . . .   B-1

C-1 ASCII Symbols . . . . . . . . . . . . . . . . . . . . . . . . .   C-1

C-2 ASCII Conversion Table. . . . . . . . . . . . . . . . . . . . .   C-2

D-1 AS68 Diagnostic Error Messages. . . . . . . . . . . . . . . . .   D-1

D-2 AS68 User-recoverable Fatal Error Messages. . . . . . . . . . .   D-5

D-3 LO68 Fatal Diagnostic Error Messages. . . . . . . . . . . . . .   D-8

D-4 AR68 Fatal Diagnostic Error Messages. . . . . . . . . . . . . .  D-11

D-5 DUMP Error Messages . . . . . . . . . . . . . . . . . . . . . .  D-15

D-6 SIZE68 Error Messages . . . . . . . . . . . . . . . . . . . . .  D-16

D-7 SENDC68 Diagnostic Error Messages . . . . . . . . . . . . . . .  D-16

D-8 SID68 Diagnostic Error Messages . . . . . . . . . . . . . . . .  D-16

E-1 S-record Field Descriptions . . . . . . . . . . . . . . . . . .   E-1

E-2 S-Record Type Definitions . . . . . . . . . . . . . . . . . . .   E-1