Atari STE Hardware documentation, taken from the developer information, and

altered to correspond more to observed facts than the original documentation.

INTRODUCTION TO THE STE

The Atari STE is an enhanced version of the ST with the following changes:

GRAPHICS

The colour pallete has been expanded to 4096 colours from 512.  Support for

Vertical and Horizontal scrolling has been added to the hardware.

SOUND

Stereo 8-bit DMA sound has been added to the system along with volume and tone

control.

CONTROLLERS

New controllers ports have been added to allow keypad type joysticks, analog

paddies and a light pen.

GENLOCK AND THE STE

The ST (and STE) chip set have the ability to accept external sync.  This is

controlled by bit 0 at FF820A, as documented in the ST Hardware Specification.

This was done to allow the synchronization of the ST video with an external

source (a process usually known as GENLOCK).  However, in order to do this

reliably the system clock must also be phase-locked (or synchonized in some

other way) to the imput sync signals.  No way to do this was provided in the

ST, as a result the only GENLOCKs available are internal modifications (usually

for the MEGA).

The STE allows this to be done without opening the case.  To inject a system

clock ground pin three (GPO) on the monitor connector and then inject the clock

into pin 4 (mono direct).  The internal frequency of this clock is 32.215905 MHz

(NTSC) and 31.922046 MHz (PAL).

Note:  DO NOT SWITCH CLOCK SOURCE WHILE THE SYSTEM IS ACTIVE.

(As a result of this GPO is no longer available).

JOY STICKS

Four new joystick ports are added.  These ports are controlled directly by the

68000.  The current state may be sampled at any time by reading the locations.

Joystick 0 and Joystick 2 direction bits are read/write.  If written to they

will be driven until a read is performed.  Similarly, they will not be driven

after a read until a write is performed.

PADDLES

One pair of paddles can be plugged into Joystick 0 (Paddle 0).  A second set

can be plugged into Joystick 1 (Paddle 1).  The current position of each of the

four paddles is reported at these locations.  The fire buttons are the same as

for the respective joystick.  The triggers for the paddles are read as bits one

and two of FF9202 (JOY 0 Left and Right).

LIGHT GUN / PEN

A light gun or pen can be plugged into Joystick 0.  The current position that

the gun or pen is pointing to is reported by these registers.  The position is

accurate to within (X direction only):

4 Pixels in 320*200 Mode

8 Pixels in 640*200 Mode

16 Pixels in 640*400 Mode

Accurate to 1 pixel in the Y direction in all modes.  Accuracies do not account

for the quality of the light gun or pen.  Note that the X position is given in

pixels for 320*200 only.  In order to get correct results in 640*200 mode this

number needs to be shifted left one bit and in 640*400 mode this number needs to

be shifted left two bits.

HOW TO IMPLEMENT FINE SCROLLING ON THE STE.

Three new registers are provided to implement fine-scrolling and split screen

displays:

1) HSCROLL - This register contains the pixel scroll offset.  If it is zero,

this is the same as an ordinary ST.  If it is non-zero, it indicates which

data bits constitute the first pixel from the first data word(S) of a given

line by this register.

2) LINEWID - This register indicates the number of extra words of data (beyond

that required by an ordinary ST at the same resolution) which represents a

single display line.  If it is zero, this is the same as an ordinary ST.  If

it is non-zero, that many additional words of data will constitute a single

video line (thus allowing virtual screens wider than the displayed screen).

CAUTION: In fact, this register contains the word offset which the display

processor will add to the video display address to point to the next line.

If you are actively scrolling (HSCROLL <> 0), this register should contain

the additional width of a display line minus one data fetch (in low

resolution one data fetch would be four words, one word for monochrome, etc).

3) VBASELO - This register contains the low-order byte of the video display base

address.  It can be altered at any time and will not affect the display until

the next vertical blank interrupt.

The following registers have been changed on the STE to allow greater control

over the display:

1) VIDEO ADDRESS COUNTER - Now read/write.  Allows update of the video refresh

address during the frame.  The effect is immediate, therefore it should be

reloaded carefully (or during blanking) to provide reliable results.

2) COLOUR PALLETE - A fourth bit of resolution is added to each colour.  Note

that the least significant bit is added above the old most significant bit to

remain compatible with the ST.

These registers, when used in combination, can provide several video effects.

In this document we will discuss only fine-scrolling and split-scrolling

displays.

FINE SCROLLING:

Many games use horizontal and vertical scrolling techniques to provide virtual

playfields which are larger than a single screen.  We will first discuss

vertical scrolling (line-wise), then horizontal scrolling (pixel-wise) and

finally the example program "neowall.s" which combines both.

VERTICAL SCROLLING:

To scroll line-wise, we simply alter the video display address by one line each

time we wish to scroll one line.  This is done at vertical blank interrupt time

by writing to the three eight-bit video display address registers to define

a twenty-four bit pointer to memory.  Naturally, additional data must be

available to be displayed.  We might imagine this as a tall, skinny screen

which we are opening onto for the user. The video display address registers

define where this window will start.

HORIZONTAL SCROLLING:

To scroll horizontally we might also adjust the video display address. If that

was all we did, we would find that the screen would jump sideways in sixteen

pixel incements.  To achieve smooth pixel-wise scrolling we must use the HSCROLL

register to select where within each sixteen pixel block we wish to start

displaying data to the screen.  Finally, we must adjust the LINEWID register to

reflect both the fact that each line of video data is wider than a single

display line and any display processor fetch incurred by a non-zero value of

HSCROLL.  All this is done at vertical blank interrupt time.  Naturally,

additional data must be available to be displayed.  We might imagine this is

an extremely wide screen which we are opening a window onto for the user.

These registers define where this window will start.

FOR EXAMPLE:

The program "neowall.s" reads in nine NEOchrome picture files, organizes them

into a three by three grid and allows the user to scroll both horizontally and

vertically over the images.  The heart of this program (the only interesting

thing about it actually) is the vertical blank interrupt server.  This routine

first determines the pixel offset and loads it into HSCROLL.  The LINEWID

register is now set to indicate that each virtual line is three times longer

than the actual display width.  If we are now actively scrolling, this amount

is reduced to reflect the additional four-plane data fetch which will be

caused by the scrolling.  Finally the video display address is computed to

designate a window onto the grid of pictures.  This twenty-four-bit address

determines where the upper left corner of the displayed region begins in

memory.  Thus, every frame an arbitrary portion of the total image is selected

for display.  The speed and resolution of this scrolling technique is limited

only by the dexterity of the user.

SPLIT SCREEN:

In many applications it is desirable to subdivide the screen into several

independent regions.  On the STE you may reload some video registers on a

line-by-line besis (using horizontal blanking interrupts) to split the screen

vertically into multiple independent regions.  A single screen no longer need

be a contiguous block of storage, but could be composed of dozens of strips

which might reside in memory in any other.  The same data could be repeated on

one or more display lines.  Individual regions might each have their own

individual data and scrolling directions.

FOR EXAMPLE:

The program "hscroll.s" reads in a NEOchrome picture file and duplicates each

line of the image.  This, combined with the proper use of LINWID, effectively

places two copies of the same picture side-by-side.  Next, both vertical and

horizontal blanking interrupt vectors are captured and the horizontal blanking

interrupt is enabled in counter mode.  To prevent flicker caused by keyboard

input, the IKBD/MIDI interrupt priority is lowered below that of the HBL

interrupt.  Note that the program 'main loop' doesn't even call the BIOS to

check the keyboard, since the BIOS sets the IPL up causes flicker by locking

out horizontal interrupts - this may cause trouble for programs in the real

world.  The screen is effectively divided into ten regions which scroll

independently of one another.  There are two ten-element arrays which contain

the base address of each region and its current scroll offset.  At vertical

blank interrupt time we compute the final display values for each region in

advance and store them into a third array.  We then initialize the display

processor for the first region and request an interrupt every twenty lines

(actually every twenty horizontal blankings).  During each horizontal interrupt

service, we quickly reload the video display address registers and the HSCROLL

register.  This must be done immediately - before the display processor has

time to start the current line or garbage may result.  Note that horizontal

blank interrupts are triggered by the display processor having finished reading

the previous data line.  You have approximately 144 machine cycles to reload

the HSCROLL and video display registers before they will be used again by the

display processor finishes reading the data for the current display line.  We

then pre-compute the data we will need for the next horizontal interrupt to

shave few more cycles off the critical path and exit.

-------------------------------------------------------------------------------

STE DIGITIZED SOUND OVERVIEW

Sound  is stored in memory as digitized samples.  Each sample is a number,

from -128 to +127, which represents displacement of the speaker from the

"neutral" or middle possition.  During horizontal blanking (transparent

to the processor) the DMA sound chip fetches samples from memory and provides

them to a digital-to-analog canverter (DCA) at one of several constant

rates, programmable as (approximately) 50KHz (kilohertz), 25 Khz, 12.5KHz,

and 6.25KHz.  This rate is called the sample frequency.

The output of the DAC is then filtered to a frequency equal to 40% of the

sample frequency by a four-pole switched low-pass filter.  This program

"anti-aliasing" of the sound data in a sample- frequency-sensitive way.

The signal is further filtered by a two-pole fixed frequency (16KHz)

low-pass filter and provided to a National LMC1992 Volume/Tone Controller.

Finally, the output is available at an RCA-style output jack on the back

of the computer.  This can be fed into an amplifier, and then to speakers,

headphones, or tape recorders.

There are two channels which behave as described above; they are intended

to be used as the left and right channels of a stereo system when using the

audio outputs of the machine.  A monophonic mode is provided which will

send the same sample data to each channel.

The stereo sound output is also mixed onto the standard ST audio output

sent to the monitor's speaker.  The ST's GI sound chip output can be mixed

to the monitor and to both stereo output jacks as well.

DATA FORMAT

Each sample is stored as a signed eight-bit quantity, where - 128 (80 hex)

means full negative displacement of the speaker, and 127 (7F hex) means full

positive displacement.  In some stereo mode, each word represents two

samples: the upper byte is the sample for the left channel, and the lower

byte is the sample for the right channel.  In mono mode each byte is one

sample.  However, the samples are always fetched a word at a time, so only

an even number of mono samples can be played.

A group of samples is called a "frame".  A frame may be played once or can

automatically be repeated forever (until sotpped).  A frame is described by

its start and end addresses.  The end address of a frame is actually the

address of the first byte in memory beyond the frame; a frame starting at

address 21100 which is 10 bytes long has an end address of 21110.

Note:  a zero can be written to the DMA sound control register at any

time to stop playback immediately.

PROGRAMMING CONSIDERATIONS

The simplest way to produce a sound is to assemble a frame in memory,

write the start address of the frame into the Frame Start Address register,

and the end address of the frame into the Frame End address register, set

the Mode register appropriately (set stereo to mono, and the sample

frequency), and write a one into the Sound DMA Control register.  The frame

will play once, then stop.

To produce continuous sound, and link frames together, more elaborate

techniques are required.

The DMA sound chip produces a signal called "DMA sound active" which is one

when the chip is playing sounds, and a zero when it's not.  When a frame ends

in the repeat mode (mode 3), there is a transition from "active" to "idle"

and back again on this signal.  The signal is presented as the external input

to MFP Timer A.  You can put Timer A into Event Count mode and use it to

generate an interrupt, for example when a frame has played a given number

of timees.  Because of the design of the MFP, the active edge for this signal

must be the same as the input on GPIP 14, which is the interrupt line from

the keyboard and MIDI interfaces.  It is, and the Active Edge Register is

already programmed for that, so you need not worry about that if you use

Timer A to count frames.

The DMA Sound Chip's mode 3 (repeat mode) ensures seamless linkage of frames,

because the start and end registers are actually double-buffered.  When you

write to these resisters, what you write really goes into a "holding area".

The contents of the holding area go into the true registers at the end of the

current frame.  (Actually, they go in when the chip is idle, which means right

away if the chip was idle to begin with.)

If you have two frames which you want played in succession, you can write the

start and end address of the first frame into the chip, then set its control

register to 3.  The first frame will begin playing.  You can then immediately

write the start and end addresses of the second frame into the chip: they will

be held in the holding area until the first frame finishes, then they'll be

copied into the true registers and the second frame will play.  The interrupt

between frames will still happen, so you can tell when the first frame has

finished.  Then, for instance, you can write the start and end registers for

the start of the third frame, knowing that it will begin as soon as the second

frame has finished.  You could even write new data into the first frame and

write its start and end address into the chip; this kind of ping-pong

effect is rather like double-buffering of a graphics display.

Here is an example of using Timer A in Event Count mode to play a controlled

series of frames.  Suppose you have three frames, A, B and C, and you want to

play frame A three times, then frame B five times, and finally frame C twice.

The sequence of steps below will accomplish this.  Numbered steps are carried

out by your program; the bracketed descriptions are of things which are

happening as a result.

1.  Set Timer A to event count mode, and its counter to 2 (not 3)

2.  Write Frame A's start and end addresses into the registers.

3.  Write a 3 to the sound DMA control register.  [Play begins.]  Go

do something else until interrupted.

[At the end of the sound repition of Frame A, the timer's

interrupt fires.  At the same time, frame A begins its third

repetition.]

4.  Write Frame B's start and end addresses into the DMA sound chip.

These values will be held until the third repetition of Frame A

finishes.

5.  Set Timer A's count register to 5, then go away until

interrupted.

[When the current repetition finishes, the start and end

registers are loaded fron the holding area, and Frame B will

begin playing.  The end of play signal will cause Timer A to

count from 5 to 4.  At the end of Frame B's fourth repetition,

its fifth will start, the timer will count down from 1 to 0,

and the interrupt will occur.]

6.  Write frame C's start and end addresses into the registers, and

program Timer A to count to 2.  Go away until interrupted.

[When the current repetition (B's fifth) finishes, the start

and end registers are loaded from the holding area, and Frame C

will begin playing.  The end-of-frame signal causes Timer A to

count down from 2 to 1.  When Frame C finishes its first

repetition, Timer A counts down from 1 to 0 and interrupts.]

7.  Write a 1 to the DMA Sound Control Register to play the current

frame, then stop.  Disable TimerA and mask its interrupt.

You're done.

As you can see you program the timer to interrupt after one repetition less

than the number of times you want a frame to play.  That is so you can set up

the next frame while the DMA sound chip is playing the last repetition of the

current frame.  This ensures seamless linkage of frames.

INTERRUPTS WITHOUT TIMER A

Besides going to the external input signal of Timer A, the DMA-sound-active

signal, true high, is exclusive-ORed with the monochrome-detect signal, and

together they form the GPIP 17 input to the M68901 MFP.  The intent of this

is to provide for interrupt-driven sound drivers without using up the last

general-purpose timer in the MFP.  It is a little trickier to use, however.

For one thing, it causes the interrupt at the end of every frame, not after

a specified number of frames.  For another, the "interesting" edge on this

signal depends on what kind of monitor you have.

On an ST, monochrme monitors ground the mono-detect signal, so when you read

the bit in the MFP you get a zero.  Colour monitors do not ground it, so it

reads as a one.  When the DMA sound is idle (0), this is still the case.

However, when the sound is active (1), the mono-detect signal is inverted by

the XOR, so the bit in the MFP reads the opposite way.  (the one place where

the OS reads this bit as at VBLANK time, to see if you've changed monitors.

The ROMs on any machine with DMA sound are approprietely modified, so you

need not worry about this.)

If you want to use the mono-detect / DMA interrupt signal, you have to set up

the active-edge register in the MFP to cause the interrupt at the right time.

The interesting edge on the DMA signal is the falling edge, that is, from

active to idle; this happens when a frame finishes.  If you have a monochrome

monitor, this edge is seen as a transition from 1 to 0 on MFP bit 17.

However, with a colour monitor, the edge will be seen as a transition from

0 to 1.  Therefore, you have to program the MFP's active-edge regisrer

differently depending on which monitor you have.  Make sure the DMA sound is

idle (write a zero to the control register), then check MFP 17: if it's one,

you have a colour monitor, and you need to see the rising edge.  If it's

zreo, you have a monochrome monitor, and you need to see the falling edge.

The DMA sound active signal goes from "active" to "idle" when a frame

finishes.  If it was playing in mode one, it stays "idle" and the control

register reverts to zero.  If it was playing in mode 3 the signal goes

back to "active" as the next frame begins.  In this case, the signal is

actually in the "idle" state for a very short time, but the MFP catches it

and causes the interrupt, so don't worry.

ADDITIONAL CONSIDERATIONS

Regardless of how you manage your interrupts, there is more you should know:

the signal goes from "active" to "idle" when the DMA sound chip has fetched

the last sample in the frame.  There is a four-word FIFO in the chip,

however, so it will be eight sample-times (four in stereo mode) before the

sound actually finishes.  If you are using mode 1, you can use this time to

set up the chip with the start and end addresses of the next frame, so it

will start as soon as the current one ends.  However, if the interrupt

should be postponed for four or eight sample-times, you could miss your

chance to start the sound seamlessly.  Therefore, for seamless linkage,

use the pre-loading technique described above.

MICROWIRE INTERFACE

The MICROWIRE interface provided to talk to the national LMC1992 Computer

Controlled Volume / Tone Control is a general purpose MICROWIRE interface

to allow the future addition of other MICROWIRE devices.  For this reason,

the following description of its use will make no assumptions about the

device being addressed.

The MICROWIRE bus is a three wire serial connection and protocol designed to

allow multiple devices to be individually addresses by the controller. The

lengh of the serial data straem depends on the destination device.  In

general, the stream consists of N bits of address, followed by zero or more

don't care bits, followed by M bits of data.  The hardware interface

provided consists of two 16 bit read/write registers: one data register

which contains the actual bit stream to be shifted out, and one mask register

which indicates which bits are valid.

Let's consider a mythical device which requires two address bits and one data

bit.  For this device the total bit stream is three bits (minimum).  Any

three bits of the register pair may be used.  However, since the most

significant bit is shifted first, the commanmd will be received by the device

soonest if the three most if the three most significant bits are used.  Let's

assume: 01 is the devise's address, D is the data to be written, and X's are

don't cares.  Then all of the following register combinations will provide

the same information to the device.

1110 0000 0000 0000 Mask

01DX XXXX XXXX XXXX Data

0000 0000 0000 0111 Mask

XXXX XXXX XXXX X01D Data

0000 0001 1100 0000 Mask

XXXX XXX0 1DXX XXXX Data

0000 1100 0001 0000 Mask

XXXX 01XX XXXD XXXX Data

1100 0000 0000 0001 Mask

01XX XXXX XXXX XXXD Data

As you can see, the address bits must be contigious, and so must the data

bits, but they don't have to be contiguous with each other.

The mask register must be written before the data register.  Sending

commences when the data register is written and takes approximately 16usec.

Subsequent writes to the data and mask registers are blocked until sending

is complete.  Reading the registers while sending is in progress will

return a snapshot of the shift register shifting the data and mask out.

This means that you know it is safe to send the next command when these

registers (or either one) return to thier original state.  Note that the

mask register does not need to be rewritten if it is already correct.

That is, when sending a series of commands the mask register only needs

to be written once.

VOLUME AND TONE CONTROL

The LMC1992 is used to provide volume and tone control.  Before you go and

find a data sheet for this part, be warned that we do not use all of its

features.  Commands for the features we do use are listed below.

Communication with this device is achieved using the MICROWIRE interface.

See MICROWIRE INTERFACE the section for details.  The device has a two

bit address field, address = 10, and a nine bit data field.  There is no

way to reading the current setting.

VOLUME / TONE CONTROLLER COMMANDS

Device address = 10 (Address precedes command)

Data field

001 DDD DDD Set Master Volume

000 000 -80 dB

010 100 -40 dB

101 XXX  0 dB

101 XDD DDD Set Left Channel Volume

00 000 -40 dB

01 010 -20 dB

10 1XX  0 dB

100 XDD DDD Set Right Channel Volume

00 000 -40 dB

01 010 -20 dB

10 1XX  0 dB

010 XXD DDD Set Treble

0 000 -12 dB

0 100  0 dB (Flat)

1 100 =12 dB

001 XXD DDD Set Base

0 000 -12 dB

0 110  0 dB (Flat)

1 100 +12 dB

000 XXX XDD Set Mix

00 -12 dB

01 Mix GI sound chip output

10 Do not mix GI sound chip output

11 reserved

Note: The volume controls attenuate in 2 dB streps.  The tone controls

attenuate in 2 dB steps at 50 kHz (Note that these frequencies may change).

USING THE MICROWIRE INTERFACE AND THE VOLUME/TONE CONTROL CHIP

The MICROWIRE interface is not hard to use: once you get it right, you'll

never have to figure it out again.

The easiest way to use it is to ignore the flexibility, and just use one

form for all commands.  Since the Volume/Tone chip is the only device, and

it has a total of 11 bits of address and data, your mask should be $07ff.

If you're picky, you can use $ffe0, because the high-order bits are shifted

out first, but it adds conceptual complexity.  With a mask of $07ff, the

lower 9 bits of the data register are used for the data, and the next higher

two bits are for the address:

Mask:   %0000 0111 1111 1111

Data:   %xxxx x10d dddd dddd

Replace the d's with the command code and its data.  For example, this

combination sets the master volume to $14:

Mask:   %0000 0111 1111 1111

Data:   %xxxx x100 1101 0100

The other important concept you must understand is that the bits shift out

of these registers as soon as you write the data, and it takes an appreciable

time (16 usec) to finish.  You can't attempt another write until the first

one is finished.  If you read either register while it's being shifted out,

you will see a "snapshot" of the data being shifted.  You know the shifting

is complete when the mask returns to its original value.  (This theory is

wrong if you use a mask which equals its original value sometime during the

shifting, but $07ff never does.)

Assuming you write $07ff into the mask register ahead of time, the following

routine can be used to write new data from the D0 register to the volume/tone

control chip.

MWMASK  equ $ffff8924

MWDATA  equ $ffff8922

mwwrite:

cmp.w      #$07ff,MWMASK  ; wait for prev to finish

bne.s      mwwrite        ; loop until equal

move.w     d0,MWDATA      ; write the data

rts                       ; and return

The purpose of the loop at the beginning is to wait until previous write

completes.

This loop is at the beginning of the routine, not at the end because waiting

at the end would always force a sixteen usec delay.  Even if it has been longer

than that since the last write.

-------------------------------------------------------------------------------

NEW CONTROLLER PINOUT

Arrangement (Looking towards STE):

5  4  3  2  1

10  9  8  7  6

15 14 13 12 11

Connections (PORT A):

1:  UP 0	5:  PAD 0Y	9:  GND	       13:  LT 2

2:  DN 0	6:  FIRE 0     10:  FIRE 2     14:  RT 2

3:  LT 0        7:  +5v        11:  UP 2       15:  PAD 0X

4:  RT 0        8:  n/c        12:  DN 2

Connections (PORT B):

1:  UP 1	5:  PAD 1Y	9:  GND	       13:  LT 3

2:  DN 1	6:  FIRE 1     10:  FIRE 3     14:  RT 3

3:  LT 1        7:  +5v        11:  UP 3       15:  PAD 1X

4:  RT 1        8:  n/c        12:  DN 3

CONTROLLER MEMORY LOCATIONS

FF9201             ---- 3120  RO  Fire Buttons

FF9202   UDLR UDLR UDLR UDLR  RW  Joystick 3/2/1/0 Directions

FF9211             xxxx xxxx  RO  X-Paddle 0

FF9213             xxxx xxxx  RO  Y-Paddle 0

FF9215             xxxx xxxx  RO  X-Paddle 1

FF9217             xxxx xxxx  RO  Y-Paddle 1

FF9220   ---- --xx xxxx xxxx  RO  Light Gun X-Pos

FF9222   ---- --xx xxxx xxxx  RO  Light Gun Y-Pos

VIDEO REGISTER MODIFICATIONS

FF8205             --xx xxxx  RW  Video Address High

FF8207             xxxx xxxx  RW  Video Address Mid

FF8209             xxxx xxx-  RW  Video Address Low

FF820D             xxxx xxx-  RW  Video Base Address Low

FF820F             xxxx xxxx  RW  Over-Length Line Width

FF8264             ---- xxxx  RW  Undocumented HSCROLL register: no extra fetch

FF8265             ---- xxxx  RW  Documented HSCROLL register

FF8240+  ---- 0321 0321 0321  RW  Red/Green/Blue Colour settings

DMA SOUND REGISTERS

FF8901             ---- --cc  RW  Sound DMA Control

cc:

00   Sound DMA disabled (reset state).

01   Sound DMA enabled, disable at end of frame.

11   Sound DMA enabled, repeat frame forever.

FF8903             --xx xxxx  RW  Frame Base Address (high)

FF8905             xxxx xxxx  RW  Frame Base Address (middle)

FF8907             xxxx xxx-  RW  Frame Base Address (low)

FF8909             --xx xxxx  RO  Frame Address Counter (high)

FF890B             xxxx xxxx  RO  Frame Address Counter (middle)

FF890D             xxxx xxx-  RO  Frame Address Counter (low)

FF890F             --xx xxxx  RW  Frame End Address (high)

FF8911             xxxx xxxx  RW  Frame End Address (middle)

FF8913             xxxx xxx-  RW  Frame End Address (low)

FF8921             m--- --rr  RW  Sound Mode Control

rr:

00   6258 Hz sample rate (reset state)

01  12517 Hz sample rate

10  25033 Hz sample rate

11  50066 Hz sample rate

m:

0    Stereo Mode (reset state)

1    Mono Mode

FF8922  xxxx xxxx xxxx xxxx  RW  MICROWIRE Data register

FF8924  xxxx xxxx xxxx xxxx  RW  MICROWIRE Mask register