KORGM1.DK help file.

Loading in the file KorgM1.DK will set up Band-in-a-Box for the KorgM1

synthesizer. It will set up :

1. Drum #'s for Korg M1 Kit 1 (Patch 9 )

2. Patch Map (assigned to factory settings of Korg M1)

3. Startup Patches and Channels in Midi window.

If you have modified the Patches in your M1 from the factory settings

you may have to modify the Patch Map in BB (from the MIDI pull down menu).

The only remaining step for you is to setup the Korg M1 in Multi-Timbral

mode to receive multiple instruments at once. In "Program mode" the M1

will only play 1 instrument at a time. In Combination mode it will play

up to 8 at once- so you want to be in combination mode and setup a

combination.

To Setup the M1 , from the M1 front panel:

1. Go to Combination Mode "COMB". Select combo #0 by pressing 00 on the

# pad.

2. Then press the EDIT COMBO button to EDIT The Combination

3. You will then be using the PAGE + and PAGE - keys to PAGE

through the Combination setting up each of the 8 instruments

each of the A-H keys represents a different instrument.

Setup your Combination as follows:

A    B     C      D     E      F      G      H

Bass Piano Drums Guitar Horns Strings Melody Thru

Page 0 Combination type MULTI

Page 1 Program #  66   42     9     4      22     27     22     4

Page 2 Channel    2     3    10     7      8      6      4      5

Page 3 Key Window Type ALL= G9

Page 4 Output    90    90    90    90     90     90     90     90

Page 5 Transpose  0    0      0     0      0      0      0      0

Page 6 Pan Pot    5:5 is in the middle - can separate as you like

Page 7 Enable Prg All should be Enabled

Page 8 Effect 1= "4:Room Reverb" :ON

0.7 D022 E76 HD10    L:0 H:0 80:20

Effect 2 = "12:Chorus 1  ":ON

M60 S0.30 D010 TRI   L:0 H:0 86:14

Then Go to PAGE 9 and SAVE the Combination by pressing the 'G' key

twice.

!!!

IT IS ESSENTIAL THAT YOU SET THE 'GLOBAL CHANNEL' ON THE M1 TO ANU UNUSED

CHANNEL - EG. CHANNEL 1 . DO THIS FROM THE 'GLOBAL' BUTTON THEN KEY#5.

OTHERWISE A PATCH CHANGE SENT ON THE GLOBAL CHANNEL WILL CHANGE THE

COMBINATION FROM THE "BAND-IN-A-BOX COMBO" TO A DIFFERENT COMBO.

!!!

Once you have done this , Band-in-a-Box is ready to Play your M1

just by you selecting COMBO 0 from the M1 panel ( as long as you

have loaded in the KorgM1.DK file into BB and then saved it as MYSETUP.DK)

YOU WILL NOW BE READY TO GO WITH Band-in-a-Box AND THE M1!.

INFO BELOW IS OPTIONAL/ADDITIONAL.

You may want to fine tune the Combination by adjusting

Page 4 Output levels : If you find 1 instrument always too loud/soft

EDIT/Save the Combination to adjust

Page 6 Pan Pot : adjust for stereo placemant of instruments

Page 8 Effects , adjust the Reverb/Chorus if too strong/weak

OPTIONAL- Edit the Volume/Octave of an M1 sound (program)

Most of the BB styles were made on a Roland Sound Canvas. The Korg M1

patches are sometimes louder or softer than the Sound Canvas. The best

way to correct this is to Edit the M1 program to make the sound quieter

or louder. This can be done by selecting a Program then choosing EDIT

program. PAGE + once to get to the screen that says OSC1 .

The D key then gets you to the setting the Level (volume) of the sound.

The E key sets the Octave of the sound 4' 8' 16' (feet).

Some sounds use two oscillators so Page + and repeat if present for OSC2.

Have Fun!