Hej du! Hr fr du din lista av VT100 - kommandon. Har snabbt

skrivit av mina papper varfr det kan ha insmugit sig ett och

annat stavfel. Koderna ska skickas till terminalen utan ngra

blankslag. Pn str fr ett numeriskt argument vilket skickas

i ASCII-format, dvs menar du 32 skicka inte det binra vrdet

fr 32 (tolkas som '0'). I C skulle det se ut s hr om du

vill placera cursorn:

printf("\x1b[%d;%dH", line, col);

Mnga koder kan inte tolkas av flera VT100 emulatorer, tex

de fr dubbel bredd och hjd eller fr att tnda lysdioderna.

Du fr labba lite.

Control sequences in ANSI (VT100) mode

======================================

Function				Control Sequence

Cursor movement commands

------------------------

Cursor up				ESC [ Pn A

Cursor down				ESC [ Pn B

Cursor Forward				ESC [ Pn C

Cursor Backward				ESC [ Pn D

Cursor Position				ESC [ line ; col H

or ESC [ line ; col f

Index					ESC D

New Line				ESC E

Reverse Index				ESC M

Save cursor and attributes		ESC 7

Restore cursor and attributes		ESC 8

Double height and widht commands

--------------------------------

Double height top half			ESC #3

Double height height bottom half	ESC #4

Single widht line			ESC #5

Double width line			ESC #6

Erasing commands

----------------

From cursor to end of line		ESC [ K

From start of line to cursor		ESC [ 1 K

Entire line				ESC [ 2 K

From cursor to end of screen		ESC [ J

From start of screen to cursor		ESC [ 1 J

Entire screen				ESC [ 2 J

Character attribute commands

----------------------------

Video attributes off			ESC [ m

High intensity on			ESC [ 1 m

Underline on				ESC [ 4 m

Blink					ESC [ 5 m

Reverse video on			ESC [ 7 m

Character set commands

----------------------

U.K. set is G0				ESC ( A

U.K. set is G1				ESC ) A

U.S. set is G0				ESC ) B

U.S. set is G1				ESC ) B

Special graphics is G0			ESC ( C

Special graphics is G1			ESC ) C

Mode setting commands

---------------------

Set new line mode			ESC [ 20 h

Reset New line mode			ESC [ 20 l

Set cursor key mode			ESC [ ? 1 h

Reset cursor key mode			ESC [ ? 1 l

Set VT52 mode				ESC [ ? 2 l

Set slow scroll				ESC [ ? 4 h

Reset slow scroll			ESC [ ? 4 l

Set reverse video			ESC [ ? 5 h

Reset reverse video			ESC [ ? 5 l

Set origin mode				ESC [ ? 6 h

Reset origin mode			ESC [ ? 6 l

Set autowrap mode			ESC [ ? 7 h

Reset autowrap mode			ESC [ ? 7 l

Set application keypad mode		ESC =

Reset application keypad mode		ESC >

Scrolling region and tab stop commands

--------------------------------------

Define scrolling region			ESC [ top ; bottom r

Set tab at current column		ESC H

Clear tab at current column		ESC [ g

or ESC [ 0 g

Clear all tabs				ESC [ 3 g

Reporting commands

------------------

Cursor position request			ESC [ 6 n

Cursor position report			ESC [ line ; col R

Status report request			ESC [ 5 n

Status report (terminal OK)		ESC 0 n

DA request				ESC [ c

or ESC [ 0 c

DA response

with advanced video			ESC [ ? 1 ; 2 c

with no options				ESC [ ? 1 ; 0 c

with printer port			ESC [ ? 1 ; 11 c

Logging commands

----------------

Print screen				ESC [ i

Enter auto print mode			ESC [ ? 5 i

Exit auto print mode			ESC [ ? 4 i

Enter printer controller mode		ESC [ 5 i

Exit printer controller mode		ESC [ 4 i

Programmable LED commands

-------------------------

Turn all off or one on			ESC [ Ps ; ...Ps q

Ps 0 or none = all off

Ps 1 turn on L1

Ps 2 turn on L2

Ps 3 turn on L3

Ps 4 turn on L4

Control sequences in VT52 mode

==============================

Function				Control sequence

Cursor up				ESC A

Cursor down				ESC B

Cursor right				ESC C

Cursor left				ESC D

Enter graphics				ESC F

Exit graphics				ESC G

Curosr home				ESC H

Reverse line feed			ESC I

Erase to end of page			ESC J

Erase to end of line			ESC K

Cursor addressing			ESC Y line col

(line and column relative to decimal 32)

DA request				ESC Z

Enter alternate keypad mode		ESC =

Exit alternate keypad mode		ESC >

Print page				ESC ]

Enter auto print mode			ESC ^

Exit auto print mode			ESC _

Enter printer controller mode		ESC W

Exit printer controller mode		ESC X

Enter ANSI mode				ESC <