Abstract

The Atari ST's 128 Kbyte read-only cartridge port can be

transformed into a 64 Kbyte read/write port by using this cir-

cuit. Writing to this port is slower than reading it by a factor

of two, approximately. It can be written to at a speed of

76 Kilowords per second.

This interface can be built by using only three chips, two

AS-TTLs and a PAL -- a total parts cost of less than $15.

How_It_works

1.  Background_Information

The ST brings out 15 address lines (A1-A15),  16 data lines (d0-

D15) /LDS,[*] /UDS, /AS, /ROM4 and /ROM3.

Of these signals, /UDS, /LDS, and /AS have the same meaning

as they as they do for a 68000 CPU. A1-A15 are the least signifi-

cant 15 address lines. D0-D15 are connected internally to the

ST's data-bus. /ROM3 and /ROM4 are generated by the MMU and are

output signals which tell the external hardware which of the two

banks is being addressed. /ROM4 is activated when the 68000 tries

to read address 0xfa0000-0xfaffff. /ROM3 is activated when the

68000 tries to read address 0xfb0000-0xfbffff. (Note that the

names of these signals are counterintuitive; ie. they have been

correctly written above.)

2.  Caveats

Any and all hardware connected to this modified port must be

fast -- fast enough to run with no wait-states and fast enough to

tolerate the decoding delays introduced by this circuit. I've

used 200ns static RAM chips without any trouble.

It is advisable to use /ROM4 to time any external hardware

you build (in preference to /AS), since /ROM4 contains the neces-

sary delay to ensure the addresses are stable when it is

activated. (At least this is what I assumed, and it worked.)

The data for write operations is produced by the two AS-TTL

latches[*] used in the circuit (see below.)

________

[*] A preceeding slash indicates an active-low signal

Cartridge Port Interface        1                  April 14, 1987

3.  Operating_Principle

i.  The PAL contains an asynchronous finite state machine. When

the upper bank (address 0xfbxxxx) is read, the PAL manufac-

tures an A0 (based on the state of /UDS and /LDS) and sup-

plies this to U1. At the end of the read cycle, U1 and U2

are clocked to latch the least significant 16 address bits

into the PAL.

ii.  Subsequently, when the lower bank is read (address

0xfaxxxx), the latched address (which was latched in step 1)

is supplied as data and /PWE (the write-enable signal) is

activated (it goes low). /ROM4 active should tell the exter-

nal hardware that it is being addressed, and /PWE active

should tell it that this is a write-cycle.  As an interest-

ing  side-effect of this design, the data word written out

is read back by the 68000 (you can use this fact to debug

the interface!)

iii. Reads to the upper bank will return garbage unless some ROM

is present there. Multiple successive reads to this bank

will not be harmful (ie. they will not confuse the state-

machine) and the latest address will be latched preparation

for a read to the lower bank.

Note that if you put RAM in the upper-bank you will not

be able to write to it. Connecting the /PWE line to such RAM

will have no effect. Putting a ROM in this address space

should not create any problems (although I haven't tried

this).

Example:

To write data 0xcd01 to address 0x3bc0 in the lower bank,

the following steps are performed.

step1.  read byte at address 0xfbcd01   (0xfb0000+data)

step2.  read word to address 0xfa3bc0   (0xfa0000+addr)

_________________________________________________________________

[*] I used AS374s, you can use AS or FAST. High speed chips are

necessary so that delays from /OE (latch output-enable) to

valid data are not critical. You could possibly get away with

ALSTTL, but I haven't tried this.

Cartridge Port Interface        2                  April 14, 1987

Construction

The schematic should help in this department.  Constructing this

gadget is easy and cheap. The hardest part might be trying to

find a connector for this port. You can get one from

Douglas Electronics,

718 Marina Blvd.,

San Leandro, Calif

94577

1-415-483-8770

(part number 33-DE-40. $10.00 per piece)[*]

Another problem you might face is getting the PAL programmed.  If

you don't know of a way to get the PAL programmed, you could use

discrete logic chips to build the state machine.

4.  Building_it

The pin-outs given in the User's guide are CORRECT. The

pin-outs given in the "Internals" book are WRONG.  Follow the

schematic. You can get the pin-outs for the '374s from an ASTTL

or FAST databook. The PAL16L8's pin-outs are given below.

________

[*] You guessed it -- I'm not connected with D.E. although I and

a number of others have purchased these connectors from them.

Cartridge Port Interface        3                  April 14, 1987

PAL PINOUTS

Legend:

A preceding slash implies an active-low signal.

NC implies no-contact (nothing should be connected to these pins.)

(i) indicates that the signal is an input.

(o) indicates that the signal is an output.

(A) implies that the signal comes from/goes to the Atari cart. port.

(u1) implies that the signal goes to U1, the least-significant

latch.

(u2) implies that the signal goes to U2, the most-significant

latch.

(ex) implies that the signal goes to any external hardware connected

such as a ram-disk.

pin #           signal                  pin #           signal

-----           ------                  -----           ------

1               /UDS (i, A)             11              NC (no contact)

2               /LDS (i, A)             12              /OE (o, U1, U2)

3               /ROM4 (i, A)            13              A0 (o, U1)

4               /ROM3 (i, A)            14              /PWE (o, ex)

5               NC                      15              NC

6               NC                      16              /dclk (o, U1,U2)

7               NC                      17              NC

8               NC                      18              NC

9               NC                      19              NC

10              gnd (A)                 20              Vcc (A)

Note that the line called "help" is shown as

"NC" since it is to be left unconnected.

ALL THE BEST!!

Anees Munshi. April 14, 1986.

ARPA        asm%csri.toronto.edu@csnet-relay.arpa

BitNet      asm@utcsri.UTORONTO

CSNet       asm@csri.toronto.edu

UUCP        {allegra,ihnp4,decvax,pyramid}!utzoo!utcsri!asm

58 York Road, Weston, Ontario M9R 3E6. Canada. (416) 241-2166

Cartridge Port Interface        4                  April 14, 1987

##############################################################################################