;****************************************************************************

;*

;*  Welcome to ASPI.  The following code is provided to assist you with

;*  writing code to the Advanced SCSI Programming Interface (ASPI).  Run the

;*  program ASPIPROG.EXE to see what the following program does when compiled.

;*

;*  The only support provided for ASPI developers by Adaptec is through this

;*  sample code and the ASPI specification, which can be ordered through

;*  Literature at (408) 945-8600 x2579.  The ASPI specification is available

;*  for DOS, OS/2, and NetWare 386.  Please do not call the Adaptec technical

;*  support line with ASPI programming questions.

;*

;*  This sample code provides examples of how to use the ASPI	programming

;*  interface.  The main program obtains the ASPI entry point	and calls

;*  subroutines for the different ASPI functions.

;*

;*	1. Obtains ASPI Entry Point

;*	2. Executes ASPI Host Adapter Inquiry Command

;*	3. Executes ASPI Get Device Type Command

;*	4. Executes ASPI Execute SCSI I/O Command

;*	5. Executes ASPI Reset SCSI Device Command using Posting

;*	6. Executes (3) and then executes ASPI Abort SCSI I/O command

;*

;*

;****************************************************************************

.286

.xlist

include aspidefs.inc	;include file for ASPI definitions

include scsidefs.inc	;include file for SCSI definitions

.list

.MODEL Small

.STACK 300h

;****************************************************************************

;	DATA SEGMENT

;****************************************************************************

.DATA

SCSIMgrString	db	"SCSIMGR$"      ;SCSI Mgr String (need to get entry)

dw	0		;NULL-Terminate String

ASPI_Entry	dd	0		;Entry Address stored by GetASPIEntry

SRB		db	64+10+14 dup (0) ;SCSI Request Block

SRB2		db	64+10+14 dup (0) ;SCSI Request Block 2

Data_Buffer	db	50 dup (?)	;Data Buffer for Inquiry command

Flag		db	0		;flag used by post routine

;****************************************************************************

;	DISPLAY STRINGS

;****************************************************************************

Str_InquiryFail db  "Error issuing SCSI inquiry command",CR,LF,'$'

Str_NoASPI	db  "Unable to open ASPI Manager!",CR,LF,'$'

Str_ASPI_Stat	db  "  ASPI Status (HEX)               : %",CR,LF,'$'

Str_HA_Inquiry	db  CR,LF,"ASPI HOST ADAPTER INQUIRY COMMAND",CR,LF,'$'

Str_Num_HA	db  "  Number of Host Adapters         : %",CR,LF,'$'

Str_TID_HA	db  "  SCSI ID of Host Adapter 0       : %",CR,LF,'$'

Str_Mgr_ID	db  "  SCSI Manager ID                 : %%%%%%%%%%%%%%%%",CR,LF,'$'

Str_HA_ID	db  "  Host Adapter ID                 : %%%%%%%%%%%%%%%%",CR,LF,'$'

Str_Get_DevTyp	db  CR,LF,"ASPI GET DEVICE TYPE COMMAND",CR,LF,'$'

Str_Dev_Typ	db  "  SCSI Device Type                : %",CR,LF,'$'

Str_SCSI_IO	db  CR,LF,"ASPI SCSI I/O COMMAND (Inquiry Command)",CR,LF,'$'

Str_PQualTyp	db  "  Qualifier / Device Type (HEX)   : %",CR,LF,'$'

Str_Versions	db  "  ISO / ECMA / ANSI Version (HEX) : %",CR,LF,'$'

Str_Inq_Dat	db  "  Inquiry Data                    : %%%%%%%%%%%%%%%%%%%%%%%%%%%%",CR,LF,'$'

Str_Dev_Rst1	db  CR,LF,"ASPI RESET DEVICE COMMAND (with Posting)",'$'

Str_Wait4Post	db  ".",'$'

Str_HA_Inq2	db  CR,LF,"ASPI HOST ADAPTER INQUIRY COMMAND (Aborted)",'$'

Str_CRLF	db  CR,LF,'$'

;****************************************************************************

;	CODE SEGMENT

;****************************************************************************

.CODE

start:		mov	ax,@DATA		;initialize ds and es

mov	ds,ax

mov	es,ax

;*	1. Obtains ASPI Entry Point

call	GetASPIEntry									        ;Get ASPI entry point

jc	ASPI_Done		                    ;Jump if no manager

;*	2. Executes ASPI Host Adapter Inquiry Command

lea	si,SRB			                     ;point to SRB

call	HAInquiry		                  ;Send ASPI HA Inquiry

call	Poll_Status		                ;wait for status

call	DispSRBInq		                 ;Display Inquiry Info

;*	3. Executes ASPI Get Device Type Command

lea	si,SRB			                     ;point to SRB

call	GetDevType	                 	;Send ASPI Get Device Type

call	Poll_Status                		;wait for status

call	DispDevTyp                 		;Display Device Type Info

;*	4. Executes ASPI Execute SCSI I/O Command

lea	si,SRB                     			;point to SRB

call	Load_Inquiry		               ;Load SCSI Inquiry into SRB

call	XSCSI_IO		                   ;Execute SCSI I/O

call	Poll_Status                		;wait for status

call	DispInquiry		                ;Display I/O results

;*	5. Executes ASPI Reset SCSI Device Command using Posting

lea	dx,Str_Dev_Rst1              	;load the string for outtext

call	outtext 		                   ;output the text

lea	si,SRB                     			;point to SRB

mov	Flag,0		                    		;reset the post flag

call	RST_SDevice	               		;Resets SCSI ID = 2

lea	dx,Str_Wait4Post           			;point to Wait for Post String

PostLp1:

mov	al,Flag 	                    	;get the post flag

cmp	al,1

call	outtext 	                   	;output the text

jnz	PostLp1 	                    	;jump if not 1

call	DispStat                   		;Display status

;*	6. Executes (3) and then executes ASPI Abort SCSI I/O command

lea	dx,Str_HA_Inq2	             	;load the string for outtext

call	outtext 		                  ;output the text

lea	si,SRB                    			;point to SRB

call	Load_Inquiry		              ;Load SCSI Inquiry into SRB

call	XSCSI_IO                  		;Execute SCSI I/O

call	Abort_SIO                 		;call abort SCSI IO

call	Poll_Status               		;wait for status

call	DispStat		                  ;Display status

ASPI_Done:	mov	ax,ASPI_DOSExit

int	21h

ret	                          			;Exit to DOS

;****************************************************************************

;

;		  %%% ASPIDISK SUPPORT ROUTINES %%%

;

;****************************************************************************

; Subroutine Name:  GetASPIEntry

;

; Description:	This gets the ASPI manager's entry point and stores it in

;		'ASPI_Entry.'

;		If no ASPI manager is found, an appropriate error message

;		is displayed.

;

; Entry:  Nothing

;

; Exit:   The memory location ASPI_Entry is loaded with entry point

;	  CF set if no ASPI manager found, else reset

;****************************************************************************

GetASPIEntry	proc	near

mov	ax,ASPI_Open

lea	dx,SCSIMgrString

int	21h			               ;Open ASPI Manager

jc	NoASPIManager		       ;Branch if none found

push	ax			               ;Save ASPI File Handle

mov	bx,ax			             ;BX = File Handle

mov	ax,ASPI_Get_Entry

lea	dx,ASPI_Entry		      ;Store entry point here

mov	cx,4			              ;Four bytes to transfer

int	21h			               ;Get ASPI entry point

mov	ax,ASPI_Close

pop	bx																			;BX = ASPI File Handle

;BX = ASPI File Handle

int	21h			;Close ASPI Manager

clc				;Return with carry clear

ret

NoASPIManager:	lea	dx,Str_NoASPI		;Display error message, no

call	outtext 		;  ASPI manager found

stc				;Return with carry set

ret

GetASPIEntry	endp

;****************************************************************************

; Subroutine Name:  Poll_Status

;

; Description:	This subroutine polls the ASPI Status Byte and waits for

;		non-zero status.  When status goes non-zero we return.

;		Although this sample code will hange if status never goes

;		non-zero, an actual program should time out.

;

; Entry:  Assumes that si is pointing to an SRB that has just been issued

;

;****************************************************************************

Poll_Status	proc	near

poll_loop:	mov	al,[si]._SRStat ;get the status byte of SRB

cmp	al,0		;compare to zero

jz	poll_loop	;jump if still zero

ret

Poll_Status	endp

;****************************************************************************

; Subroutine Name:  HAInquiry

;

; Description:	This subroutine sends the Host Adapter Inquiry Command to the

;		ASPI Manager.  First the SRB is totally cleared.  Then the

;		opcode and other fields of the SRB are set for HOST ADAPTER

;		INQUIRY command.

;

; Entry:  si points to the SRB

;

; Exit:   SRB is full of returned data

;****************************************************************************

HAInquiry	proc	near

call	ClearSRB		;Zero out our SRB

mov	[si]._SRCmd,HA_INQ	;ASPI opcode = HA inquiry

mov	[si]._SRHAid,0		;Host Adapter 0

mov	[si]._SRFlags,0 	;0=no length checking

mov	[si]._SRTarget,0	;Target 0

mov	[si]._SRLun,0		;Lun 0

;CALLING ASPI !!

push	ds			;push segment of SRB

lea	bx,SRB

push	bx			;push offset of SRB

call	[ASPI_Entry]		;Call ASPI

add	sp,4			;Restore stack

ret

HAInquiry	endp

;****************************************************************************

; Subroutine Name:  GetDevType

;

; Description:	This subroutine sends the Get Device Type Command to the

;		ASPI Manager.  First the SRB is totally cleared then the SRB

;		is loaded with opcode and other parameters for the GET DEVICE

;		TYPE command.

;

; Entry:  si points to the SRB

;

; Exit:   SRB is full of returned data

;****************************************************************************

GetDevType	proc	near

call	ClearSRB		;Zero out our SRB

mov	[si]._SRCmd,GET_TYPE	;ASPI opcode = Get Device Type

mov	[si]._SRHAid,0		;Host Adapter 0

mov	[si]._SRFlags,0 	;0=no length checking

mov	[si]._SRTarget,0	;Target 0

mov	[si]._SRLun,0		;Lun 0

;CALLING ASPI !!

push	ds			;push segment of SRB

lea	bx,SRB

push	bx			;push offset of SRB

call	[ASPI_Entry]		;Call ASPI

add	sp,4			;Restore stack

ret

GetDevType	endp

;****************************************************************************

; Subroutine Name:  XSCSI_IO

;

; Description:	This subroutine sends the Execute SCSI I/O Command to the

;		ASPI Manager.  It is assumed that the SCSI command has already

;		been loaded into the SRB.  Here the EXEC_SIO is loaded into

;		opcode of the SRB along with setting the other fields for a

;		24 byte read of SCSI INQUIRY data.

;

; Entry:  The SCSI Command Block is setup upon entry, si points to SRB

;

; Exit:   SRB is full of returned data

;****************************************************************************

XSCSI_IO	proc	near

mov	[si]._SRCmd,EXEC_SIO	;ASPI opcode = execute SCSI IO

mov	[si]._SRHaId,0		;Host Adapter 0

mov	[si]._SRFlags,SRB_DIR_IN ;Set flags to direction in

mov	[si]._SRTarget,0	;Target 0

mov	[si]._SRLun,0		;Lun 0

mov	[si]._SRBuflenLo,24	;Buffer xfer length LSB

mov	[si]._SRBuflenHi,0	;Buffer xfer length MSB

mov	[si]._SRSnsLen,20	;Sense Buffer length 20

lea	ax,Data_Buffer

mov	[si]._SRBufPtrLo,ax	;load Data buffer offset

mov	[si]._SRBufPtrHi,ds	;load Data buffer segment

mov	[si]._SRCDBLen,6	;SCSI CDB Length

;CALLING ASPI !!

push	ds			;push segment of SRB

lea	bx,SRB

push	bx			;push offset of SRB

call	[ASPI_Entry]		;Call ASPI

add	sp,4			;Restore stack

ret

XSCSI_IO	endp

;****************************************************************************

; Subroutine Name:  RST_SDevice

;

; Description:	This subroutine sends the RESET SCSI DEVICE Command to the

;		ASPI Manager.  It attempts to reset SCSI ID 2 instead of

;		ID 0 in case ID 0 is the boot device of your computer.

;		Posting is used by this routine.

;

; Entry:  si points to the SRB

;

;****************************************************************************

RST_SDevice	proc	near

call	ClearSRB		;Zero out our SRB

mov	[si]._SRCmd,RESET_DEV	;ASPI opcode = Reset Device

mov	[si]._SRHaId,0		;Host Adapter 0

mov	[si]._SRFlags,SRB_POST	;set post bit

mov	[si]._SRTarget,0	;Target 0

mov	[si]._SRLun,2		;Lun 0

mov	[si]._SRBuflenLo,0	;Buffer xfer length LSB

mov	[si]._SRBuflenHi,0	;Buffer xfer length MSB

mov	[si]._SRSnsLen,20	;Sense Buffer length 20

mov	ax,OFFSET ASPI_Post

mov	[si]._SRPostOff,ax	;load post routine offset

mov	[si]._SRPostCS,cs	;load post routine segment

;CALLING ASPI !!

push	ds			;push segment of SRB

lea	bx,SRB

push	bx			;push offset of SRB

call	[ASPI_Entry]		;Call ASPI

add	sp,4			;Restore stack

ret

RST_SDevice	endp

;****************************************************************************

; Subroutine Name:  Abort_SIO

;

; Description:	This subroutine sends the Abort SCSI I/O Request command to the

;		ASPI Manager.  First SRB2 is totally cleared then the SRB2

;		is loaded with opcode and other parameters for the ABORT SCSI

;		I/O COMMAND.

;

; Entry:  si points to the SRB to be annihilated

;

;****************************************************************************

Abort_SIO	proc	near

push	si			;save pointer to SRB 2b killed

lea	si,SRB2 		;point to another SRB

call	ClearSRB		;Zero out SRB2

pop	si			;restore pointer to SRB

mov	SRB2._SRCmd,ABORT_SRB	;ASPI opcode = Abort SRB

mov	SRB2._SRHAid,0		;Host Adapter 0

mov	SRB2._SRFlags,0 	;0=no length checking

mov	SRB2._AbortPtrLo,si	;load offset of SRB 2b killed

mov	SRB2._AbortPtrHi,ds	;load segment of SRB 2b killed

;CALLING ASPI !!

push	ds			;push segment of SRB2

lea	bx,SRB2

push	bx			;push offset of SRB2

call	[ASPI_Entry]		;Call ASPI

add	sp,4			;Restore stack

ret

Abort_SIO	endp

;****************************************************************************

; Subroutine Name:  Load_Inquiry

;

; Description:	This routine loads the SCSI inquiry command into the

;		SRB.

;

; Entry:  si points to the SRB

;

; Exit:   All registers preserved

;****************************************************************************

Load_Inquiry	proc	near

call	ClearSRB		;Zero out our SRB

mov	[si]._OpCode,SCSI_Inquiry ;load opcode

mov	[si]._LUNumber,0	;set LUN = 0

mov	[si]._LBAMiddl,0	;set byte 2 = 0

mov	[si]._LBALeast,0	;set byte 3 = 0

mov	[si]._Length,24 	;set length = 24

mov	[si]._Control,0 	;set control = 0

ret

Load_Inquiry	endp

;****************************************************************************

; Subroutine Name:  ClearSRB

;

; Description:	This routine zeros out the given ASPI SRB.

;

; Entry:  SI - ASPI SRB

;

; Exit:   All registers preserved

;****************************************************************************

ClearSRB	proc	near

push	es			;Save registers

push	ax

push	cx

push	di

mov	cx,(64+10)/2		;Zero out 74 bytes

mov	ax,ds

mov	es,ax			;ES=DS

mov	di,si			;ES:DI points to SRB

xor	ax,ax			;Store zeros

cld

rep	stosw			;Zero out the SRB

pop	di

pop	cx

pop	ax

pop	es			;Restore registers and

ret				;return to caller

ClearSRB	endp

;****************************************************************************

; Subroutine Name:  ASPI_Post

;

; Description:	This routine is executed when the Reset Device command

;		completes.  It simply flips a flag so that execution in

;		the RST_SDEV routine can continue.  This flag method is

;		used strictly for example purposes.

;

;

; Exit:   All registers preserved

;****************************************************************************

ASPI_Post	proc	far

push	bp

mov	bp,sp

pusha

push	ds

push	es			;Save registers

push	dx

;		mov	bx,[bp+6]		;BX = SRB's offset

;		mov	ds,[bp+8]		;ES = SRB's segment

; CODE TO DETERMINE WHICH SRB JUST COMPLETED

mov	ds,[bp+8]		;set ds to our data segment

mov	Flag,1			;set the flag

pop	dx

pop	es			;Restore registers and

pop	ds

popa

pop	bp

retf				;return to caller

ASPI_Post	endp

;***************************************************************************

;

;			 %%% DISPLAY ROUTINES %%%

;

;***************************************************************************

; Subroutine Name:  outtext

;

; Description:	Displays a message on the screen.

;

; Entry:  DS:DX - Pointer to '$' terminated character string

;

; Exit:   Nothing

;***************************************************************************

outtext 	proc	near

push	ax		; save ax

mov	ah,09h		; Do DOS Function Print String

int	21h		; call interrupt routine

pop	ax		; restore ax

ret

outtext 	endp

;****************************************************************************

; Subroutine Name:  itoa

;

; Description:	Converts the digits of the given value to a '$'-terminated

;		character string.  Appends a carriage return and linefeed

;		to the end of the string as well if requested.

;

; Entry:  AX	- Number to be converted

;	  DS:DX - Pointer to string result

;	  CX	- Base of value (ie. 16 for hex, 10 for dec.)

;

; Exit:   All registers preserved

;****************************************************************************

itoa		proc	near

pusha

mov	bx,dx

or	ax,ax			;Is input value zero?

jne	NotZero 		;Branch if it's not

mov	BYTE PTR [bx],'0'       ;  else fill string with

inc	bx			;  a single '0' and exit

jmp	SHORT itoa_exit

NotZero:	xor	di,di

NextDigit:	or	ax,ax			;Finished pushing remainders

je	itoa_pop		;  onto the stack?

xor	dx,dx

div	cx			;Let's get the remainder

push	dx			;Push the remainder

inc	di			;Increment digit count

jmp	NextDigit

itoa_pop:	or	di,di			;Finished popping digits?

je	itoa_exit		;Branch and exit if we are

pop	dx			;Pop a digit

cmp	dl,10			;Is it a 0-9 digit?

jb	DecDigit		;If it is convert to ASCII

add	dl,'A'-10               ;  else convert to HEX

jmp	SHORT PlaceChar 	;  alphabet

DecDigit:	add	dl,'0'

PlaceChar:	mov	[bx],dl 		;Store character in string

inc	bx			;Move to next place in string

dec	di			;Decrement pop count

jmp	itoa_pop		;Pop next value

itoa_exit:

mov	BYTE PTR [bx],CR	;Place Carriage Return &

mov	BYTE PTR [bx+1],LF	;  and Line Feed and '$'

mov	BYTE PTR [bx+2],'$'     ;  char at end of string

jmp	SHORT itoa_exit1

mov	BYTE PTR [bx],'$'

itoa_exit1:	popa				;Restore all registers

ret

itoa		endp

;****************************************************************************

; Subroutine Name:  DispSRBInq

;

; Description:	This subroutine displays the information returned in the SRB

;		for the Host Adapter Inquiry Command.

;

; Entry:  Expects The SRB to be full of Host Adapter Inquiry Data

;

; Exit:

;****************************************************************************

DispSRBInq	proc	near

lea	dx,Str_HA_Inquiry	;load the string for outtext

call	outtext 		;output the text

mov	cx,16			;set base= hex for iota

lea	dx,Str_ASPI_STAT+36	;point to loc in string

mov	al,SRB._SRStat		;load value to convert

mov	ah,0			;zero out ah

call	itoa			;convert to ASCII in string loc

lea	dx,Str_ASPI_STAT	;load the string for outtext

call	outtext 		;output the text

lea	bx,Str_Num_HA		;Get Number of HA string loc

mov	al,SRB._HACount 	;Get # of ASPI host adapters

add	al,'0'                  ;AL = ASCII Host Adapter #

mov	BYTE PTR [bx+36],al	;Place HA # in string

mov	dx,bx			;load the string for outtext

call	outtext 		;output the text

lea	bx,Str_TID_HA		;Get Target ID of HA string loc

mov	al,SRB._HAIdNum 	;Get ID

add	al,'0'                  ;AL = ASCII ID

mov	BYTE PTR [bx+36],al	;Place HA # in string

mov	dx,bx			;load the string for outtext

call	outtext 		;output the text

mov	cx,16			;ID has 16 bytes (for movsb)

lea	si,SRB._SCSIMgrId	;load source of string mov

lea	di,Str_Mgr_ID+36	;load destination of string mov

rep	movsb			;move string

lea	dx,Str_Mgr_ID		;Get Manager ID string loc

call	outtext 		;output the text

mov	cx,16			;ID has 16 bytes (for movsb)

lea	si,OFFSET SRB._HAIdString ;load source of string mov

lea	di,Str_HA_ID+36 	;load destination of string mov

rep	movsb			;move string

lea	dx,Str_HA_ID		;Get Manager ID string loc

call	outtext 		;output the text

ret

DispSRBInq	endp

;****************************************************************************

; Subroutine Name:  DispDevType

;

; Description:	This subroutine displays the information returned in the SRB

;		for the Get Device Type Command.

;

; Entry:  Expects The SRB to be full of Host Adapter Inquiry Data

;

; Exit:

;****************************************************************************

DispDevTyp	proc	near

lea	dx,Str_Get_DevTyp	;point to Get Dev Type string

call	outtext 		;output the text

lea	bx,Str_ASPI_STAT	;point to ASPI Status string

mov	al,SRB._SRStat		;Get ASPI status

add	al,'0'                  ;convert to ASCII

mov	BYTE PTR [bx+36],al	;place Status in string

mov	dx,bx			;load the string for outtext

call	outtext 		;output the text

lea	bx,Str_Dev_Typ		;Get SCSI Dev Type string loc

mov	al,SRB._ScsiDevType	;Get # of ASPI host adapters

add	al,'0'                  ;AL = ASCII Host Adapter #

mov	BYTE PTR [bx+36],al	;Place HA # in string

mov	dx,bx			;load the string for outtext

call	outtext 		;output the text

ret

DispDevTyp	endp

;****************************************************************************

; Subroutine Name:  DispInquiry

;

; Description:	This subroutine displays the information returned in the SRB

;		for the Get Device Type Command.

;

; Entry:  Expects The SRB to be full of Host Adapter Inquiry Data

;

; Exit:

;****************************************************************************

DispInquiry	proc	near

lea	dx,Str_SCSI_IO		;load the string for outtext

call	outtext 		;output the text

mov	cx,16			;set for HEX base (iota call)

lea	dx,Str_ASPI_STAT+36	;point to string location

mov	al,SRB._SRStat		;load value to convert

mov	ah,0			;zero out ah

call	itoa			;convert to ASCII in string

lea	dx,Str_ASPI_STAT	;load the string for outtext

call	outtext 		;output the text

lea	dx,Str_PQualTyp+36	;point to string location

mov	al,Data_Buffer.PerQualTyp ;load value to convert

mov	ah,0			;zero out ah

call	itoa			;convert to ASCII in string

lea	dx,Str_PQualTyp 	;Get Peripheral Qual Dev string

call	outtext 		;output the text

lea	dx,Str_Versions+36	;Get Peripheral Qual Dev string

mov	al,Data_Buffer.Versions ;Get Byte from Data Buffer

mov	ah,0			;zero out ah

call	itoa			;convert to ASCII in string

lea	dx,Str_Versions 	;load the string for outtext

call	outtext 		;output the text

push	si			;save pointer to SRB

mov	cx,28			;load number of bytes to mov

lea	si,OFFSET Data_Buffer.VendorID ;load source address

lea	di,Str_Inq_Dat+36	;load destination address

rep	movsb			;move the string

lea	dx,Str_Inq_Dat		;load the string for outtext

call	outtext 		;output the text

pop	si			;restore pointer to SRB

ret

DispInquiry	endp

;****************************************************************************

; Subroutine Name:  DispStat

;

; Description:	This subroutine displays the ASPI status byte

;

;

;****************************************************************************

DispStat	proc	near

lea	dx,Str_CRLF

call	outtext 		;output the text

lea	bx,Str_ASPI_STAT	;point to ASPI Status string

mov	al,SRB._SRStat		;Get ASPI status

add	al,'0'                  ;convert to ASCII

mov	BYTE PTR [bx+36],al	;place Status in string

mov	dx,bx			;load the string for outtext

call	outtext 		;output the text

ret

DispStat	endp

;****************************************************************************

END	START