___________________________________________________________________________

ICD CD-ROM DRIVE SOFTWARE

___________________________________________________________________________

ICD, Incorporated

1220 Rock Street

Rockford, IL  61101

U.S.A.

Technical Support: (815) 968-2228 ext 235

FAX:               (815) 968-6888

___________________________________________________________________________

Notice:  Advantage ST, AdSCSI ST, and The Link are trademarks of ICD, Inc.

Other brand and product names are registered trademarks or trademarks of

their respective holder.

This documentation and the ICD XFS CD-ROM driver are copyright 1994 by ICD,

Inc.  All Rights Reserved.  Reproduction or translation of any part of this

work (beyond that permitted by sections 107 and 108 of the United States

Copyright Act) without permission of the copyright owner is unlawful.

In no event will ICD, Inc. be liable for any damages whatsoever (including

without limitation, damages for loss of business profits, business

interruption, loss of business information or the like) arising out of the

use of, interruption in the use of, or inability to use this software, even

if ICD, Inc. has been advised of any possibility or likelihood of such

damages.

MinT and its utilities are included with the permission of Atari Corp.

___________________________________________________________________________

INTRODUCTION

If included as part of the ICD LINK software, this CD-ROM drive software

allows the use of standard SCSI-II-compatible CD-ROM drives connected to an

ICD Advantage ST, AdSCSI ST, or Link host adapter.

If included as part of the ICD PRO software, this CD-ROM drive software

allows the use of standard SCSI-II-compatible CD-ROM drives connected to the

Atari TT 030 and Falcon 030 SCSI ports as well as the above ICD host adapters.

This CD-ROM software also requires the presence of ICDBOOT with a minimum

version of 6.5.2.

NOTE:  some older CD-ROM drives are not SCSI-II and will not work with this

software.

ICD's XFS and MinT will access both High Sierra and ISO-9660 format CD-ROMs.

INSTALLATION

Installing the CD-ROM software is fairly straightforward.

Make a MINT folder on one of your hard drive partitions.  Copy the file

MINT.TOS into it and double-click on it.  It is a self-extracting ZIP file

and will extract all the files.  You should probably do this on a partition

other than drive C as space on C may be limited.

Copy MINT.PRG to your present AUTO folder.  This is usually on drive C.

Note that MINT.PRG must be physically the last program in your AUTO folder.

(Remember that many versions of TOS display only sorted directories).  If it

is not the last program, use a utility such as AUTO_ORG.PRG (from Codehead

Software) to reorganize your AUTO folder.  Or, make a JUNK folder, copy all

files from your AUTO folder into JUNK, erase all files in AUTO, then copy

files from JUNK back into AUTO one by one, ending with MINT.PRG.

Make a folder on the boot drive called MINT.  Copy the files GEM.SYS and

GEM.CNF into this folder.  Copy the file MINT.CNF into the root directory of

the drive.

Copy the file ICDCDZ2.XFS into the root or main directory of the same drive

that your AUTO folder is on.  The name of this file may have to be changed;

the characters after "ICDCD" indicate the logical drive that your CD-ROM

files will be under and the logical SCSI ID of your CD-ROM drive.

ICDCDdi.XFS

^^

|\

| \

|  ---- is the logical SCSI ID of your drive unit in decimal

\       (1 through 7, 9 through 15).  SEE TABLE BELOW.

\

\

---- is the logical drive you will use.

examples:

ICDCDZ2.XFS

^^

||

|\

| \

|  ---- indicates physical SCSI ID of 2 and a logical SCSI ID

\       of 2, connected to DMA port of ST or TT using an ICD

\      host adapter.

\

---- indicates drive Z:.

ICDCDM10.XFS

^^

||

|\

| \

|  ---- indicates physical SCSI ID of 2 and a logical SCSI ID

\       of 10, connected to SCSI port on TT or Falcon.

\

\

---- indicates drive M:.

The following table shows how to figure out the logical SCSI ID of your

CD-ROM drive.  The physical SCSI ID is the one set on your drive through

jumpers or by a switch.  You must translate the physical ID to a logical

ID as follows:

Port         Physical SCSI ID   Logical SCSI ID

-----------------------------------------------

DMA port (on        1                 1

ST and TT)          2                 2

3                 3

4                 4

5                 5

6                 6

7                 7

SCSI port           1                 9

(on TT and Falcon)  2                10

3                11

4                12

5                13

6                14

7                15

If you do not know the physical ID of your CD-ROM drive, hook it up and run

the IDCHECK program.

Note that IDs 0 and 8 are NOT ALLOWED.  In addition, the ID 15 is generally

used by the TT or Falcon and is NOT ALLOWED unless you have changed the

initiator ID of your computer, which is possible only to the sophisticated

user.  Contact ICD if you want to do this.

If necessary, do a 'Show Info' and rename ICDCDZ2.XFS as needed.

Now, reboot your computer.  Your bootup will proceed as normally, and MinT

should install at the end of your AUTO folder.  MinT will load your XFS

file and show the name, logical ID, and logical drive of your CD-ROM unit.

If an error is encountered, a message will be printed and you must then

press any key to continue, but the XFS driver will not have been loaded.

Next, you will see the MinT signon dialog, followed by the loading of your

accessories and then the MinT desktop.  At this point you can install an

icon for drive Z: (or whatever drive you select) and double-click on it

to display the contents of the CD-ROM in the drive.

SUPPORT

ICD will provide technical support on its XFS driver and the installation

of MinT and the drive in regard to access of CD-ROMs only.  Due to constraints

in time and personnel, the rest of the MinT package cannot be supported by

ICD.  Phone support is normally available 10 AM to 2 PM Central Time Monday,

Wednesday, and Friday, or send a FAX or letter and it will be answered in

like fashion as soon as possible.

ADVICE

If you do not want to run MinT all the time, set up SUPERBT or DESKMGR to

allow you to have separate bootup schemes for normal and MinT sessions.

SUPERBT is a shareware program available on the ICD BBS.  DESKMGR is

available from Codehead Software.  Remember to support shareware authors!