#-----------------------------------------------------------------------------

# File name:	HISTORY.DOC			Revision date:	1999.01.28

# Created by:	Ulf Ronald Andersson		Creation date:	1999.01.28

#-----------------------------------------------------------------------------

# Purpose:	Version history of B030 compatibility boot package

#-----------------------------------------------------------------------------

The information about updates for each released B030 version follow below,

in reverse chronological order. (Latest version at start of text that is.)

------------------------------------------------------------------------------

Version 1.1 released Jan 28 1999:

---------------------------------

This was the second release of B030, and it mainly updated five things.

1:  Source was translated to DevPac dialect from PASM dialect, to avoid the

risk of a newly discovered PASM bug harming future releases.  The bug

was discovered during work on B030 1.0, and a workaround was found to

avoid problems temporarily.  Shifting to DevPac however, eliminates the

risk entirely for the future.

2:  The PMMU mapping method was changed to avoid multilevel translation tree

access.  All PMMU pages are now resolved by page descriptors at the first

translation tree level.  This increased LONGBOOT.BIN by appx 1 kilobyte,

but was necessary to achieve perfect emulation of hardware access timing.

3:  The new PMMU mapping method mentioned above also allowed improved mapping

of the various address areas not normally used, such as the cartridge ROM

area and the reserved I/O areas that surround it.  These are now properly

mapped to $FFxxxxxx for I/O areas, and $00xxxxxx for all others.

NB: By specs this should not matter, but tests show that it does matter.

That could be a quirk of TOS 4.04, but we must adapt to it even so.

4:  There was a bug in the screen allocation method used previously, which

blocked desktop from making resolution changes when booted by B030. This

has been fixed in the new version.

5:  The default for TPA clearing has been changed from 'ON' to 'OFF', to avoid

wasting time.  The clearing is not really needed by any known program, but

was added to make it possible to boot with no trace of B030 left in TPA as

the 'real' boot sector is executed.  This has now become an option rather

than the default mode.  (Clearing up to 13.5 MB takes time...)

NB: Points 2 and 3 above have significant impact on hardware compatibility.

This now makes B030 compatible with timing dependent ST cartridges when

running on a Falcon. (Tested with EP16/512 EPROM programmer cartridge.)

Operation on a TT is not equally compatible, since that computer does not

have any 8MHz CPU clock mode, which is required for ST-identical timing.

------------------------------------------------------------------------------

Version 1.0 released Aug 5 1998:

--------------------------------

This was the initial release of B030.

#-----------------------------------------------------------------------------

# End of file:	HISTORY.DOC

#-----------------------------------------------------------------------------