STORM BLITTER EXTENSION      V0.1 BETA

TEMPORARY MANUAL

Please read this document

in Medium resolution

Introduction

~~~~~~~~~~~~

Welcome to the STORM Blitter extension  v0.1 beta. As you will

have noticed, this manual is  not  finished,  neither is the extension

actually. You will be reading this document if you have registered for

the STORM/GBP extension.   The  final  versions  of  the extension are

currently  being  put  together,  and  will  be  shipped  to  all  GBP

registered users with a week of  being completed.  For the moment, you

will have to make do with this manual.

The extension is fully operational, but does not contain short

cut routines for copying to  the  screen  etc.. Therefer you will have

have a play with the blitter routines, and see how they work (I myself

am only just getting to grips with them).

Commands

~~~~~~~~

Command:        blitsinc x,y

Description:

This command sets the  source  increments  to the next

word of data for the x  and  y  axis.   The  blitter  chip is a 1 word

copier (1 word = 2 bytes), we have  to  tell it the offset to the next

peice of data that we want  to  copy.  The increments allow us to copy

all data, or we culd miss some out (if so desired).

For example if we wanted to  copy  all  words, we would se the

increment to 2, as the next piece of data we want to copy lies 2 bytes

ahead of our current location.  If  we wanted to copy alternate words,

we could set the increment to 4.... Have a look at this..

Data is stored in memory sequentially  (think  of it as a long

string of information). It may  take  the  form as this (the following

values are 16 bits, and therefore take 2 bytes each):

100,255,120,123,452,542,545,245,2401,403,230

by setting the inc value to 2  our copy would look exactly the

same, but by setting the inc  value  to  4  it would produce an output

something like this..

100,120,452,545,2401,230

Command:        blitdinc x,y

Description:

Exactly the same  as  above,  but  the  increments for

storing the data back into memory.

Command:        blitaddress source,dest

Description:

This command sets the  source  address and destination

of the data.

Command:        blitmask left,middle,right

Description:

This command sets the left,middle  and right masks for

the data that is being  copied.  Whenever  a  bit  is set in the value

passed the information will be copied, but  when there is a 0, no data

will be copied... Ie

a mask of %1110000000000  would  copy  3 pixels of information

where as a mask of  %1111111111111111  would copy all information. All

mask values are 16 bits long (0-65535).

Command:        blitcount x,y

Description:

This command sets the x  and  y count for the graphics

processing. X is the number of words to be copied, and Y is the number

of lines to be copied. To copy an entier screen your x & y count would

be..   blitcount 80,200

Command:        blithop n

Description:

This  command  sets  the   logical  operation  of  the

Halftone copy mode. The variable n can take the following forms..

0       all 1 bits

1       use data from halftone ram

2       copy source

3       source AND halftone ram

During the writing of this manual I have noticed that halftone

support is missing from the extension,  please  be assured that I will

program support for halftone programming  for  the next release of the

extension.

Command:        blitop n

Description:

This  command  sets  the  logical  operation  for  the

blitter call itself. The variable n can take the following forms

0               all 0 bits

1               source AND destination

2               source AND NOT destination

3               source

4               NOT source AND destination

5               destination

6               source XOR destination

7               source OR destination

8               NOT source AND NOT destination

9               NOT source XOR destination

10              NOT destination

11              source OR NOT destination

12              NOT source

13              NOT source OR destination

14              NOT source OR NOT destination

15              all 1 bits

Command:        blitskew n

Description:

Sets the number of  bits  to  be  shifted to the right

before copying. Variable n can take the for 0-15

Command:        blitnfsr n

Description:

Used for "No Final Source Read"  mode,  pass n as 1 to

set the mode, 0 to release.

Command:        blitfxsr n

Description:

Used for "Force Extra Source  Read"  mode, pass n as 1

to set the mode, 0 to release.

Command:        blitline n

Description:

Sets the line number to  start the halftone operations

on, n can take the form 0-15

Command:        blitsmudge n

Description:

When n is passed as 1,  it  forces the blitter chip to

use the data from BLIT SKEW to  be used as the halftone patterns, pass

n as 0 to cancel.

Command:        blithog n

Description:

Pass n to 1 so  the  CPU  will  wait until the blitter

chip is finished before return control  to the user. Otherwise control

is return straight away, and the  blitter  chip  will copy the data in

the background.  NOTE.. Using the blitter  chip for 2 operations while

the blithog is set to 0 can  crash  you programs.. I's main use if for

copy data while your program  calculates  other things. A command will

be added shortly to test if the blitter is working or not.. More later

in the file..

Command:        blitit

Description:

Causes the blitter chip to begin processing it's data.

Well that's it for the  moment,  I  am  sorry that this manual

does not go into depth, but that  will  be sorted out very shortly. WE

are still working on this extension,  so upgrades might be very often,

on the other hand we  may  complete  the  extension before sending the

upgrades out. Drop me a line an tell me what you think..

The future of the extension

~~~~~~~~~~~~~~~~~~~~~~~~~~~

Future upgrades for the extension...

(1).    Blitter halftone programming

(2).    Blitter busy routines

(3).    Full screen copying            ** FINISHED ROUTINES **

(4).    Screen clearing                ** FINISHED ROUTINES **

(5).    Bit plane copying

(6).    Bit plane clearing

(7).    Blitter sprites                ** UNDER PROGRESS    **

(?).    Any suggestions??

If you have any suggestions  for  the extension, please do not

hesitate to conatct me, my address  can  be found in the GBP Extension

manual.

Ta Ta for now