CDBENCH v1.1 - a CD-ROM benchmark program

-----------------------------------------

CDBENCH version 1.1 is Freeware by Roger Burrows, Anodyne Software.

It is hereby placed in the public domain, and may be freely copied

and distributed.  Please keep the program executable and this

documentation together - thanks.

Description

-----------

CDBENCH is a program that measures the following performance

characteristics of a CD-ROM drive:

Random access time

This is measured by performing multiple random accesses and

reads of data blocks (sectors) over the whole area of the

CD-ROM media, recorded up to a position of 60:02:00 (equivalent

to 60mins 02secs on an audio CD).  The time is reported in

milliseconds.

Full stroke access time

This is measured by performing multiple maximum travel accesses

from 00:02:00 to 60:02:00.  The time is reported in milliseconds.

Data transfer rate

This is measured by performing reads of multiple adjacent data

sectors.  The rate is reported in kilobytes per second.  In order

to properly evaluate constant-angular-velocity drives (otherwise

known as variable-speed drives), the transfer rate is measured at

both the start and the end of the CD-ROM medium (i.e. at 00:02:00

and 60:02:00).

CDBENCH requires the support for CD-ROM physical I/O that is provided

by ExtenDOS Pro or ExtenDOS Gold.

How to use CDBENCH

------------------

Double-click on CDBENCH.TTP.  When the dialog box is displayed, just

press enter.  You will be prompted for the _PHYSICAL_ drive letter

corresponding to your CD-ROM drive (see your ExtenDOS Pro manual if

you are not sure how to figure out what is the physical drive).  The

tests described above will be run, the results will be displayed, and

CDBENCH will wait for you to press enter before returning to the

desktop.

If you are running from a command-line shell such as Gulam, you may

enter the drive letter as an argument to CDBENCH; in this case, there

will be no prompt, and no pause at the end of processing.

Notes

-----

1. Since the program accesses the CD-ROM area up to 60:02:00, you

must ensure that a fairly 'full' CD-ROM is in the drive; if there

is less than about 528MB of data on the drive, the tests will not

run.

2. It is normal to see slight variances in the results for the same

drive from different runs of CDBENCH, particularly in random access

times.

Sample CD-ROM Performance Metrics

---------------------------------

The following results were obtained for selected CD-ROM drives,

using CDBENCH v1.1 on a TT030:

Random     Full     Transfer (KB/s)

Drive                   access    stroke      start / end

-----                  -------    ------    ---------------

Toshiba 3301            350 ms    455 ms       150 / 150

Toshiba 3401            220 ms    290 ms       330 / 330

Toshiba 5301            200 ms    310 ms       599 / 601

The following values were measured with CDBENCH v1.0, which only

measures one transfer rate, and that somewhat less accurately

than v1.1.  Note that access times are comparable with those

measured by v1.1, as the algorithm has not changed:

Random     Full     Transfer rate

Drive                   access    stroke       (KB/sec)

-----                  -------    ------    -------------

AppleCD SC Plus         385 ms    550 ms         147

Atari CDAR-504         1844 ms   3407 ms          51

NEC CDR25               982 ms   2004 ms         151

NEC 3Xp                 344 ms    548 ms         436

Panasonic CR-501        450 ms    770 ms         147

Sony CDU-561            298 ms    488 ms         294

About ExtenDOS Gold

-------------------

ExtenDOS Gold is an extension to Atari operating systems, including

TOS, MultiTOS, MagiC, and Geneva.  It supports a wide range of

CD-ROM and CD-R(W) drives on all Atari TOS-based systems, including

the ST, STe, TT030, and Falcon030.

It allows a CD-ROM to be treated as a very large, removable hard disk,

and supports the photoCD (XA) format on most CD-ROM drives.  It includes

a program and desk accessory that allow you to play audio CDs on most

CD-ROM drives.  With selected drives, direct copying of audio tracks

to a hard drive is also supported.  Audio and data CDs can be created

using ExtenDOS Gold in conjunction with CD Writer Plus.

ExtenDOS Gold and CD Writer Plus are available from your Atari dealer,

or direct from Anodyne Software at the following address:

Anodyne Software

6 Cobbler Court

Ottawa

Ontario K1V 0B8

CANADA.

If you have Internet access, visit our web site at:

http://www.cyberus.ca/~anodyne

or contact us by email at:

anodyne@cyberus.ca

Roger Burrows

18/January/1999