;***********************************************************

;***********************************************************

;

; FILES REQUIRED:     C32.EXE

;                 C32DEMO.BAT

;                 C32DEMO.DOC

;                 C32DEMO.TBL

;

;         TO RUN: Copy all files to a directory

;                 C32DEMO <CR>

;

;***********************************************************

;

;       Cross-32 Meta-Assembler Version 2.0

;

;       The Cross-32 Meta-Assembler from Universal Cross-

;       Assemblers is a table based macro cross-assembler

;       that compiles programs for numerous different

;       target processors on any MS-DOS computer.  For

;       the purposes of this demo, only a table for the

;       Intel 8051 processor is provided.  The assembler

;       will write the value 0DDH in all data locations

;       of the hex output file, but is otherwise fully

;       functional.

;

;       By using a flexible instruction table structure,

;       Cross-32 assembles source code for many microprocessors,

;       microcontrollers and digital signal processors,

;       written in the original manufacturer's mnemonics.

;       Cross-32 reads the assembly language source file

;       and a corresponding assembler instruction table,

;       and writes a list file and an absolute hexadecimal

;       machine file in the binary, Intel or Motorola

;       formats.  This hexadecimal file can then be down-

;       loaded to most EPROM programmers, EPROM emulators

;       and in circuit emulators.

;

;       Cross-32 saves the cost of purchasing many

;       individual assemblers, and reduces the time needed

;       to learn numerous different products.  It can be

;       difficult enough to grasp the intricacies of

;       different processors, without having to master new

;       tools.  To further simplify the learning process,

;       Cross-32 uses features of the C programming

;       language wherever possible.

;

;       The Cross-32 User's Manual includes full

;       directions for writing new processor tables, and

;       modifying those supplied.  Since many new

;       processor's assembly language instruction sets are

;       merely supersets of one of the processors included

;       with Cross-32, this can be as simple as adding

;       several lines to an existing table.  Therefore, as

;       new processors are introduced, the assembler does

;       not become obsolete, and your investment is

;       protected.

;

;       Processor Families Included:

;            1802/5/6

;            37700

;            3870/F8

;            4004/40

;            50740/37450

;            64180

;            6502/C02

;            65816

;            6800/1/2/3/8/6301/3

;            6805/HC05

;            6809

;            68HC11

;            68HC16

;            68000/8/10/302

;            8041

;            8048/49

;            8051/52

;            8085/80

;            8086/88/186/188

;            8096/C196KC

;            80166

;            COP400

;            COP800

;            H8/300

;            H8/500

;            NEC7500

;            NEC781x

;            PIC165x

;            PIC1742

;            SUPER8

;            TMS3201x

;            TMS3202x

;            TMS34010

;            TMS370

;            TMS7000

;            TMS9900/95

;            Z8

;            Z80

;            Z180

;            Z280

;            Z8002/4

;            Call regarding others...

;

;       Assembler Directives

;            ;         Comment

;            CPU       Processor

;            DFB       Define byte or string

;            DFS       Define data storage

;            DWM       Define word

;            DWL       Define inverted word

;            DFL       Define long word

;            ELSE      Alternate conditional

;            END       End of assembly

;            ENDI      End of IF block

;            ENDM      End of MACRO block

;            EQU       Equate label to value

;            HEX       Switch hex file on or off

;            HOF       Select hexadecimal format

;            IF        Conditional assembly

;            INCL      Include file into source

;            LIST      Switch list file on or off

;            MACRO     Define macro

;            ORG       Program counter origin

;            PAGE      Listing Page length and eject

;            SETL      Alterable equate label

;            TITL      Title starting listing pages

;            WDLN      Size of processor word

;

;       Labels

;            -unlimited length

;            -all characters significant

;            -must start with A-Z,_,., or ?

;            character

;            -may include A-Z, 0-9,_.,

;            and ? characters

;            -must end with colon :

;

;       Numeric Constants

;            32-bit signed integers

;            ANSI C: 0xnn, 0nn, nn

;            Trailing character:

;            nnB,nnO,nnQ,nnD,nnH

;            $ sign: $nn

;            Decimal is default base

;            Range -2,147,483,648 to

;            2,147,483,647

;

;       Operators

;            Arithmetic and logical

;            Similar to ANSI C

;            {}   script parentheses

;            !    logical complement

;            ~    arithmetic complement

;            +    unary addition

;            -    unary subtraction

;            INV  invert byte order

;            *    multiplication

;            /    division

;            %    modulus

;            +    addition

;            -    subtraction

;            <<   left shift

;            >>   right shift

;            <    less than

;            <=   less than or equal to

;            >    greater than

;            >=   greater than or equal to

;            ==   equal to

;            !=   not equal to

;            &    arithmetic AND

;            ^    arithmetic XOR

;            |    arithmetic OR

;            &&   logical AND

;            ||   logical    OR

;            ,    multiple expressions

;

;       Other Special Symbols

;            $    value of program counter

;            "    character "string"

;

;       Editor

;            Use your favorite ASCII editor

;            or word-processor in non-document

;            mode

;            Error output compatible with many

;            programming editors

;

;       Error Output

;            file(row,column): message

;            sent to screen and listing

;            compatible with many programming

;            text editors giving an integrated

;            programming environment

;

;       Command Line

;            C32 filename -L listname -H hexname

;

;       Hexadecimal Output

;            User chooses format using

;            HOF directive from:

;                 Binary

;                 Intel Hex and Extended Hex

;                 Motorola S records:

;                      S19, S28, or S37

;

;       Processor Instruction Tables

;            Tables for ALL processors listed above are

;            included.

;            Tables are ASCII files that may be printed

;            and edited.

;            Tables have a four part structure defining

;            registers; operands and their ranges; addressing

;            modes; and mnemonics.

;            Creating a new table requires 3 to 30 hours

;            of work, depending on the processor's complexity.

;

;       Benchmark

;            Assembles 6000 lines per minute of 6801

;            source code on a 12 MHz 80286 computer.

;

;       Documentation

;            User's manual

;            Table of contents

;            Index

;            Numerous examples

;            Instructions for creating processor tables

;            Example source file for each processor on disk

;

;       Support

;            Call the above number anytime

;            Will talk to registered users for as long as

;            they wish

;

;       System Requirements

;            MS-DOS 2.0 or later

;            256 kilobytes RAM

;            3.5" 720k or 5.25" 1.2M floppy drive

;

;       License

;            Each unit of Cross-32 may be used on only one

;            keyboard of one computer at any given time.

;            Unauthorized use, duplication or distribution

;            is strictly prohibited.

;

;       Other Features

;            Case insensitive

;            Two pass assembler with third pass if a phase

;            error occurs.

;            Program counter range: 0 to 4,294,967,295

;            Binary checksum displayed on screen

;            Not copy protected.

;

;       Available from:

;            Universal Cross-Assemblers

;            P.O. Box 6158

;            Saint John, NB, Canada

;            E2L 4R6

;            Tel/Fax: (506)847-0681

;

;       For only US$199.00!!!

;            American Express, Mastercard and VISA accepted!

;            Free planet wide airmail shipping!

;******************************************************

;	8051 TEST FILE

CPU	"C32DEMO.TBL"

HOF	"INT8"

PAGE    56           ;PAGE LENGTH

TITL    "8051 DEMO"  ;TITLE

PAGE                 ;PAGE EJECT

;******************************************************

;MCS-51 INTERNAL REGISTERS

;

B:        EQU       0F0H      ;B REGISTER

ACC:      EQU       0E0H      ;ACCUMULATOR

PSW:      EQU       0D0H      ;PROGRAM STATUS WORD

IPC:      EQU       0B8H      ;INTERRUPT PRIORITY

P3:       EQU       0B0H      ;PORT 3

IEC:      EQU       0A8H      ;INTERRUPT ENABLE

P2:       EQU       0A0H      ;PORT 2

SBUF:     EQU       99H       ;SEND BUFFER

SCON:     EQU       98H       ;SERIAL CONTROL

P1:       EQU       90H       ;PORT 1

TH1:      EQU       8DH       ;TIMER 1 HIGH

TH0:      EQU       8CH       ;TIMER 0 HIGH

TL1:      EQU       8BH       ;TIMER 1 LOW

TL0:      EQU       8AH       ;TIMER 0 LOW

TMOD:     EQU       89H       ;TIMER MODE

TCON:     EQU       88H       ;TIMER CONTROL

PCON:     EQU       87H       ;POWER CONTROL REGISTER

DPH:      EQU       83H       ;DATA POINTER HIGH

DPL:      EQU       82H       ;DATA POINTER LOW

SP:       EQU       81H       ;STACK POINTER

P0:       EQU       80H       ;PORT 0

;

;MCS-51 INTERNAL BIT ADDRESSES

;

CY:       EQU       0D7H      ;CARRY FLAG

AC:       EQU       0D6H      ;AUXILIARY-CARRY FLAG

F0:       EQU       0D5H      ;USER FLAG 0

RS1:      EQU       0D4H      ;REGISTER SELECT MSB

RS0:      EQU       0D3H      ;REGISTER SELECT LSB

OV:       EQU       0D2H      ;OVERFLOW FLAG

P:        EQU       0D0H      ;PARITY FLAG

PS:       EQU       0BCH      ;PRIORITY SERIAL PORT

PT1:      EQU       0BBH      ;PRIORITY TIMER 1

PX1:      EQU       0BAH      ;PRIORITY EXTERNAL 1

PT0:      EQU       0B9H      ;PRIORITY TIMER 0

PX0:      EQU       0B8H      ;PRIORITY EXTERNAL 0

EA:       EQU       0AFH      ;ENABLE ALL INTERRUPT

ES:       EQU       0ACH      ;ENABLE SERIAL INTERRUPT

ET1:      EQU       0ABH      ;ENABLE TIMER 1 INTERRUPT

EX1:      EQU       0AAH      ;ENABLE EXTERNAL 1 INTERR

ET0:      EQU       0A9H      ;ENABLE TIMER 0 INTERRUPT

EX0:      EQU       0A8H      ;ENABLE EXTERNAL 0 INTERR

SM0:      EQU       09FH      ;SERIAL MODE 0

SM1:      EQU       09EH      ;SERIAL MODE 1

SM2:      EQU       09DH      ;SERIAL MODE 2

REN:      EQU       09CH      ;SERIAL RECEPTION ENABLE

TB8:      EQU       09BH      ;TRANSMITT BIT 8

RB8:      EQU       09AH      ;RECEIVE BIT 8

TI:       EQU       099H      ;TRANSMIT INTERRUPT FLAG

RI:       EQU       098H      ;RECEIVE INTERRUPT FLAG

TF1:      EQU       08FH      ;TIMER 1 OVERFLOW FLAG

TR1:      EQU       08EH      ;TIMER 1 RUN CONTROL BIT

TF0:      EQU       08DH      ;TIMER 0 OVERFLOW FLAG

TR0:      EQU       08CH      ;TIMER 0 RUN CONTROL BIT

IE1:      EQU       08BH      ;EXT INTERR. 1 EDGE FLAG

IT1:      EQU       08AH      ;EXT INTERR. 1 TYPE FLAG

IE0:      EQU       089H      ;EXT INTERR. 0 EDGE FLAG

IT0:      EQU       088H      ;EXT INTERR. 0 TYPE FLAG

ORG	0200H

;

ACALL	ABSOL		;B180

LADD:	ADD	A,R0		;28

ADD	A,R1		;29

ADD	A,R2		;2A

ADD	A,R3		;2B

ADD	A,R4		;2C

ADD	A,R5		;2D

L0:	ADD	A,R6		;2E

L2:	ADD	A,DIRECT	;2580

L1:	ADD	A,R7		;2F

;L3:	ADD	A,@R0		;26

;L4:	ADD	A,@R1		;27

;L5:	ADD	A,#IMM8		;24F0

LADDC:	ADDC	A,R0		;38

ADDC	A,R1		;39

ADDC	A,R2		;3A

ADDC	A,R3		;3B

ADDC	A,R4		;3C

ADDC	A,R5		;3D

ADDC	A,R6		;3E

ADDC	A,R7		;3F

ADDC	A,DIRECT	;3580

ADDC	A,@R0		;36

ADDC	A,@R1		;37

ADDC	A,#IMM8		;34F0

AJMP	ABSOL		;A180

ANL	A,R0		;58

ANL	A,R1		;59

ANL	A,R2		;5A

ANL	A,R3		;5B

ANL	A,R4		;5C

ANL	A,R5		;5D

ANL	A,R6		;5E

ANL	A,R7		;5F

ANL	A,DIRECT	;5580

ANL	A,@R0		;56

ANL	A,@R1		;57

ANL	A,#IMM8		;54F0

ANL	DIRECT,A	;5280

ANL	DIRECT,#IMM8	;5380F0

ANL	C,CY		;82D7

ANL	C,2AH.6		;8256

ANL	C,P0.5		;8285

ANL	C,/CY		;B0D7

ANL	C,/PSW.7	;B0D7

ANL	C,/2AH.6	;B056

CJNE	A,DIRECT,$+42H	;B5803F

CJNE	A,#IMM8,$+42H	;B4F03F

CJNE	R0,#IMM8,$+42H	;B8F03F

CJNE	R1,#IMM8,$+42H	;B9F03F

CJNE	R2,#IMM8,$+42H	;BAF03F

CJNE	R3,#IMM8,$+42H	;BBF03F

CJNE	R4,#IMM8,$+42H	;BCF03F

CJNE	R5,#IMM8,$+42H	;BDF03F

CJNE	R6,#IMM8,$+42H	;BEF03F

CJNE	R7,#IMM8,$+42H	;BFF03F

CJNE	@R0,#IMM8,$+42H	;B6F03F

CJNE	@R1,#IMM8,$+42H	;B7F03F

CLR	A		;E4

CLR	C		;C3

CLR	CY		;C2D7

CLR	PSW.7		;C2D7

CLR	2AH.6		;C256

CPL	A		;F4

CPL	C		;B3

CPL	CY		;B2D7

CPL	PSW.7		;B2D7

CPL	2AH.6		;B256

DAA			;D4

DA	A		;D4

LDEC:	DEC	A		;14

DEC	R0		;18

DEC	R1		;19

DEC	R2		;1A

DEC	R3		;1B

DEC	R4		;1C

DEC	R5		;1D

DEC	R6		;1E

DEC	R7		;1F

DEC	DIRECT		;1580

DEC	@R0		;16

DEC	@R1		;17

DIV	AB		;84

DJNZ	R0,$+42H	;D840

DJNZ	R1,$+42H	;D940

DJNZ	R2,$+42H	;DA40

DJNZ	R3,$+42H	;DB40

DJNZ	R4,$+42H	;DC40

DJNZ	R5,$+42H	;DD40

DJNZ	R6,$+42H	;DE40

DJNZ	R7,$+42H	;DF40

DJNZ	DIRECT,$+42H	;D5803F

INC	A		;04

INC	R0		;08

INC	R1		;09

INC	R2		;0A

INC	R3		;0B

INC	R4		;0C

INC	R5		;0D

INC	R6		;0E

INC	R7		;0F

INC	DIRECT		;0580

INC	@R0		;06

INC	@R1		;07

INC	DPTR		;A3

JB	2AH.6,$+42H	;20563F

JB	PSW.7,$+42H	;20D73F

JB	CY,$+42H	;20D73F

JBC	2AH.6,$+42H	;10563F

JBC	PSW.7,$+42H	;10D73F

JBC	CY,$+42H	;10D73F

JC	$+42H		;4040

JMP	@A+DPTR		;73

JNB	2AH.6,$+42H	;30563F

JNB	PSW.7,$+42H	;30D73F

JNB	CY,$+42H	;30D73F

JNC	$+42H		;5040

JNZ	$+42H		;7040

JZ	$+42H		;6040

LCALL	LONG		;121234

LJMP	LONG		;021234

LMOV:	MOV	A,R0		;E8

MOV	A,R1		;E9

MOV	A,R2		;EA

MOV	A,R3		;EB

MOV	A,R4		;EC

MOV	A,R5		;ED

MOV	A,R6		;EE

MOV	A,R7		;EF

MOV	A,DIRECT	;E580

MOV	A,@R0		;E6

MOV	A,@R1		;E7

MOV	A,#IMM8		;74F0

MOV	R0,A		;F8

MOV	R1,A		;F9

MOV	R2,A		;FA

MOV	R3,A		;FB

MOV	R4,A		;FC

MOV	R5,A		;FD

MOV	R6,A		;FE

MOV	R7,A		;FF

MOV	R0,DIRECT	;A880

MOV	R1,DIRECT	;A980

MOV	R2,DIRECT	;AA80

MOV	R3,DIRECT	;AB80

MOV	R4,DIRECT	;AC80

MOV	R5,DIRECT	;AD80

MOV	R6,DIRECT	;AE80

MOV	R7,DIRECT	;AF80

MOV	R0,#IMM8	;78F0

MOV	R1,#IMM8	;79F0

MOV	R2,#IMM8	;7AF0

MOV	R3,#IMM8	;7BF0

MOV	R4,#IMM8	;7CF0

MOV	R5,#IMM8	;7DF0

MOV	R6,#IMM8	;7EF0

MOV	R7,#IMM8	;7FF0

MOV	DIRECT,A	;F580

MOV	DIRECT,R0	;8880

MOV	DIRECT,R1	;8980

MOV	DIRECT,R2	;8A80

MOV	DIRECT,R3	;8B80

MOV	DIRECT,R4	;8C80

MOV	DIRECT,R5	;8D80

MOV	DIRECT,R6	;8E80

MOV	DIRECT,R7	;8F80

MOV	DIRECT,DIRECT	;858080

MOV	DIRECT,@R0	;8680

MOV	DIRECT,@R1	;8780

MOV	DIRECT,#IMM8	;7580F0

MOV	@R0,A		;F6

MOV	@R1,A		;F7

MOV	@R0,DIRECT	;A680

MOV	@R1,DIRECT	;A780

MOV	@R0,#IMM8	;76F0

MOV	@R1,#IMM8	;77F0

MOV	C,2AH.6		;A256

MOV	C,CY		;A2D7

MOV	C,PSW.7		;A2D7

MOV	2AH.6,C		;9256

MOV	CY,C		;92D7

MOV	PSW.7,C		;92D7

MOV	DPTR,#IMM16	;90FC00

MOVC	A,@A+DPTR	;93

MOVC	A,@A+PC		;83

MOVX	A,@R0		;E2

MOVX	A,@R1		;E3

MOVX	A,@DPTR		;E0

MOVX	@R0,A		;F2

MOVX	@R1,A		;F3

MOVX	@DPTR,A		;F0

NOP			;00

MUL	AB		;A4

ORL	A,R0		;48

ORL	A,R1		;49

ORL	A,R2		;4A

ORL	A,R3		;4B

ORL	A,R4		;4C

ORL	A,R5		;4D

ORL	A,R6		;4E

ORL	A,R7		;4F

ORL	A,DIRECT	;4580

ORL	A,@R0		;46

ORL	A,@R1		;47

ORL	A,#IMM8		;44F0

ORL	DIRECT,A	;4280

ORL	DIRECT,#IMM8	;4380F0

ORL	C,2AH.6		;7256

ORL	C,PSW.7		;72D7

ORL	C,CY		;72D7

ORL	C,/2AH.6	;A056

ORL	C,/PSW.7	;A0D7

ORL	C,/CY		;A0D7

POP	DIRECT		;D080

PUSH	DIRECT		;C080

RET			;22

RETI			;32

RL	A		;23

RLC	A		;33

RR	A		;03

RRC	A		;13

SETB	C		;D3

SETB	2AH.6		;D256

SETB	PSW.7		;D2D7

SETB	CY		;D2D7

SJMP	$+42H		;8040

SUBB	A,R0		;98

SUBB	A,R1		;99

SUBB	A,R2		;9A

SUBB	A,R3		;9B

SUBB	A,R4		;9C

SUBB	A,R5		;9D

SUBB	A,R6		;9E

SUBB	A,R7		;9F

SUBB	A,DIRECT	;9580

SUBB	A,@R0		;96

SUBB	A,@R1		;97

SUBB	A,#IMM8		;94F0

SWAP	A		;C4

XCH	A,R0		;C8

XCH	A,R1		;C9

XCH	A,R2		;CA

XCH	A,R3		;CB

XCH	A,R4		;CC

XCH	A,R5		;CD

XCH	A,R6		;CE

XCH	A,R7		;CF

XCH	A,DIRECT	;C580

XCH	A,@R0		;C6

XCH	A,@R1		;C7

XCHD	A,@R0		;D6

XCHD	A,@R1		;D7

XRL	A,R0		;68

XRL	A,R1		;69

XRL	A,R2		;6A

XRL	A,R3		;6B

XRL	A,R4		;6C

XRL	A,R5		;6D

XRL	A,R6		;6E

XRL	A,R7		;6F

XRL	A,DIRECT	;6580

XRL	A,@R0		;66

XRL	A,@R1		;67

XRL	A,#IMM8		;64F0

XRL	DIRECT,A	;6280

XRL	DIRECT,#IMM8	;6380F0

ABSOL:	EQU	0580H

IMM8:	EQU	-16

IMM16:	EQU	-1024

DIRECT:	EQU	80H

LONG:	EQU	1234H

BIT.56:	EQU	56H

ZZZ:

PAGE    0            ;STOP PAGING

TITL    ""           ;STOP TITLE

END                  ;END OF PROGRAM