Date: 2 Mar 91 19:55:25 GMT

From: noao!ncar!elroy.jpl.nasa.gov!usc!zaphod.mps.ohio-state.edu!magnus.ircc.ohio-state.edu!news@arizona.edu (Frank E. Seipel)

Subject: Unsprites -- Some documentation

To: Info-Atari8@naucse.cse.nau.edu

UN-SPRITES: Copy & Paste Graphics

by Jason Leigh

Sprites (Player/Missile Graphics) has always been the easiest method of

animating objects on Atari computers. But what about computers like the

Apple IIs and IBM PCs that do not have Sprites? They achieve animation by

actually moving parts of the graphics screen. Although this is a less

efficient method, it does have capabilities that Sprites does not; that is,

they are not limited to the 8/255 resolution that Sprites has, and they can

have as many colors as a graphics mode will allow. Un-sprites is a machine

language program that will allow you to paste or copy windows to or from any

of the 16 Atari graphics modes.

Setting Un-sprites Up

Type in Listing 1 using BASIC Editor II found elsewhere in this issue, and

save it to disk or cassette. Executing this program will cause it to POKE

the machine-language program directly into memory. For cassette users,

append listing 1 to the end of your program that will use Un-sprites and call

a GOSUB to load in the data. Disk users should save the data at address

$6000 to $6300 using the "save binary" command in DOS.

Two demonstration programs have been included (Listing 2 and 3) to give you

an idea of how Un-sprites can be used for animation. Listing 4 is the BASIC

equivalent of Unsprites; the BASIC program should make it easier for you to

understand the assembly listing.

Using Un-sprites in BASIC

Copying a window

The command in BASIC to copy a window is:

COPY=USR(24842,Screen Start, StartX, StartY, EndX, EndY, Buffer Address,

Graphics Mode)

Screen Start is the address of the start of the screen memory. This can

easily be computed by entering the desired graphics mode and reading memory

locations $58 (88) and $59 (89). Here's an example:

10 GRAPHICS 7

20 SCRNSTART=PEEK(88)+PEEK(89)*256

You ae not limited to just that screen area; you may also specify a totally

different memory address and copy a window from that by assigning the

appropriate values for Screen Start.

StartX, StartY & EndX, EndY

These specify the boundaries of the copy window. Although there is no limit

to the size of the window you can copy, it is advisable to keep the

coordinates within the limits of the declared graphics mode.

+-----------------------------------------------------------+

|                                                           |

|   (StartX,StartY)                                         |

|          *------------------------+                       |

|          |                        |                       |

|          |                        |                       |

|          |                        |                       |

|          |                        |                       |

|          +------------------------*                       |

|                              (EndX,EndY)                  |

|                                                           |

+-----------------------------------------------------------+

Figure 1

Buffer Address

The window you copy must be stored somewhere in memory, and so a storage

buffer is needed. The advantage of having a user-definable buffer is that

several windows can be copied, saved, pasted and repasted as needed by

specifying the same buffer address. A buffer space can be declared at any

address where nothing else occupies the memory. This can safely be done by

dimensioning a relatively large string and using that as a buffer pool for

many windows or just one window. The alternative is to dimension many little

strings and store only one window in each of them.

The short program segment below dimensions a string 8K in size and assigns

zeros to it. BUFFADRS holds the starting address of the string buffer.

10 DIM BUFFER$(8192)

20 BUFFADRS=ADR(BUFFER$)

30 BUFFER$(1)=" ":BUFFER$(8192)=" ":BUFFER$(2)=BUFFER$(1)

Graphics Mode

These correspond exactly to those of the XL and XE Ataris. Graphics modes 0

to 11 are the same for all 8-bit machines, but graphics modes 12 to 15 are

defined as follows:

Graphics Mode 12: ANTIC Mode 4 (four color text mode 40/24)

Graphics Mode 13: ANTIC Mode 5 (four color text mode 40/12)

Graphics Mode 14: ANTIC Mode 12 (two color graphics mode 160/192)

Graphics Mode 15: ANTIC Mode 14 (four color graphics mode 160/192)

Pasting a Window

The command in BASIC to paste a window is:

PASTE=USR(25077,Screen Start, StartX, StartY, Buffer Address)

Screen Start is the address of the screen memory in which you wish to paste

the window.

StartX, StartY specify the position to place the top-left corner of the

window.

Buffer Address is the address of the buffer that holds data from a

previously copied window.

It is assumed that you will past a window in the same graphics mode it was

cut from. You may, however, choose to paste it in a different graphics mode;

the effects can be quite interesting (or disasterous).

Warning: If you accidentally omit a parameter or put them in incorrect

order, you could experience a serious lockup that will result in the loss of

your program.

Explanation

Listing 4 is the BASIC equivalent of Unsprites. It would probably be a good

idea to flip between the BASIC and assembly listings while reading the

following explanation.

Copying

When start and end coordinates are specified, Un-sprites will find the

closest byte that encloses those coordinates (See Figure 2).

DX holds the width of the window and DY, the height. Before the window is

saved, these are used as headers in the buffer, as shown in Figure 3.

The number of bytes user per window can therefore be calculated as: (DX * DY).

After DX and DY have been calculated, the top-left memory (TLMEM) location

of the window is computed. When all this is done, DX and DY will be saved to

the buffer followed by the data in the window.

Pasting

Whereas Copy saved the entire byte that enclosed StartX and StartY, Paste

allows exact alignment. Un-sprites will automatically shift the required

number of bits to place the window at the exact StartX and StartY position.

There are two methods of performing these shifts:

Method 1.

a. Calculate TLMEM

b. Find from the shift table, the number of shifts needed

c. Shift to the right

Byte a i{ shifted the required number of bits to the right. Shift Store 1

acts as a bit bucket that collects all the bits that have shifted out of a.

The data in the shift store is then ORed with shifted byte b. But in order to

shift b, a separate shift store (Shift Store 2) is needed. This method

therefore requires two shift-storage registers that alternate in use, plus it

needs registers to keep track of the address of the next byte. There isn't

(at least not to me) an obvious way to alternate the use of two registers

efficiently, so I came up with an alternative method.

Method 2. (figure 5)

a. Calculate TLMEM

b. Add 1 to TLMEM (TLMEM+1).

c. Find from shift table the number of shifts needed

d. Shift to the left

Instead of shifting to the right, we move to TLMEM+1 and shift the required

number of bits to the left. This is much easier, as only one shift-storage

register is needed; there is no intriguing alternation between two shift

registers, as in method 1.

Calculating the Shift

The code needed to do this is:

Shift_Pointer = STX -(INT(STX/Pixels_Per_Byte)*Pixelx_Per_Byte)

With the shift pointer computed, the number of shifts is then found from

SHFTABLE.

An example of calculating the shift is shown below:

Let Pixelx_Per_Byte (PPB) = 4 and STX = 9;

STX/PPB = 00001001 / 100 = 10 = 2 (all binary #'s);

then 2*4 = 8 and (STX-8) = (9-8) = 1

Look up 1 in SHFTABLE4 and we get 6. That means we need to do six left-

shifts from byte to byte 1. (Figure 6)

Note: Two PPB graphics modes (GTIA modes 9, 10, 11) need four shifts per

pixel, four PPB modes (Graphics Modes 3,5,7,15) need two shifts

per pixel, eight PPB modes (Graphics Modes 4,6,8,14) need one shift and one

PPB modes (the text modes) need no shifts.

That should be enough information for you to tailor the Unsprites assembly

listing to your own needs. If $6000 is not a convenient location for this

program, you could always move the start of the program to another location.

The .OPT LIST command has been included so that it will always display the

new location of COPY and PASTE routines.

Note that for larger windows that program could get rather slow, but if you

are page flipping, the window canbe made to appear instantaneously by

pasting on one screen while displaying another.

There are many applications for windowing: animating frames, editing

graphics screens and manipulating GEM-like windows are just a few. Sprites and

Un-sprites should be the complete pair that will allow you to perform the

graphics manipulation you've always wanted on that Atari. Enjoy.

Saved Window

(StartX, StartY)                         \/

+----------+----------+----------+----------+----------+

/  | Enclosing|          |          |          |          |

|  |   Byte   |          |          |          |          |

|  |          |          |          |          |          |

|  +----------+----------+----------+----------+----------+

|  |          |          |          |          |          |

|  |          |          |          |          |          |

|  |          |          |          |          |          |

DY  +----------+----------+----------+----------+----------+

|  |          |          |          |          |          |

|  |          |          |          |          |          |

|  |          |          |          |          |          |

|  +----------+----------+----------+----------+----------+

|  |          |          |          |          |          |

|  |          |          |          |          |          |

\  |          |          |          |          |          |

+----------+----------+----------+----------+----------+

(EndX, EndY)

\_______________________DX__________________________/

Figure 2.

+----------+----------+----------+         +----------+----------+

|  Start   |          |          |         |          |          |

|    of    |          |          |  * * *  |          |          |

|  BUFFER$ |          |          |         |          |          |

+----------+----------+----------+         +----------+----------+

Dx          Dy      \______________window data______________/

Figure 3.

Sorry, I haven't the patience to key in the other figures; you'll have

to 'figure' them out on your own!