STE Reference Documentation, v1.00 for ST User CoverDisk

Another STE extra from Adam Greenwood

The following list is a complete reference for the STE, including

all  new registers and variables plus original ones which are  of

special interest on the STE.  This list is designed to be used in

conjunction with the 'Inside the STE' series of articles.

=================

1. DMA SOUND CHIP

=================

1.1 SOUND CHIP REGISTERS:

-------------------------

$ff8900   (r/w)     enable register:

00 - disable sound (default state)

01 - enable sound, play once

11 - enable sound, loop forever

$ff8902   (r/w)     frame base address high

$ff8904   (r/w)     frame base addeess middle

$ff8906   (r/w)     frame base address low

$ff8908   (ro)      frame address counter high

$ff890a   (ro)      frame address counter middle

$ff890c   (ro)      frame address counter low

$ff890e   (r/w)     frame end address high

$ff8910   (r/w)     frame end address middle

$ff8912   (r/w)     frame end address low

$ff8920   (r/w)     sound mode control:

bit  7 6 5 4 3 2 1

use  m . . . . r r

m - 0 = stereo mode, 1 = mono mode

rr - 00 = 6.258 kHz frequency

01 = 12.517 kHz

10 = 25.033 kHz

11 = 50.066 kHz

1.2 INTERRUPT CONTROL:

----------------------

MFP Interrupt vectors for sound control:

$134      Timer A vector

$13c      GPI(7) mono monitor detect vector

MC68901 MFP configuration registers for sound:

$fffa03   Active edge

$fffa07   Interrupt enable A

$fffa0f   Interrupt in service A

$fffa13   Interrupt mask A

$fffa19   Timer A control

$fffa1f   Timer A data

Timer A uses bit 5, GPI(7) bit 7

Shift mode register:

$ff8260   Shift mode

For interrupt at start of sample:

Screen mode    Active edge

-----------    -----------

2 (mono)       Positive - or.b #$80 with edge

0,1 (colour)   Negative - and.b #$7f with edge

1.3 VOLUME/TONE CONTROL:

------------------------

Microwire registers:

$ff8922   microwire data register

$ff8924   microwire mask register

Volume/Tone addresses and parameters:

Master Volume: 10 011 DDDDDD

Left Volume:   10 101 xDDDDD

Right Volume:  10 100 xDDDDD

Treble:        10 010 xxDDDD

Bass:          10 001 xxDDDD

Mix:           10 000 xxxxDD

Where x indicates don't care, and D is a data bit.

Volume/Tone data values:

Master Volume:      0 - 40 (0 = -80dB, 40 = 0dB)

Left/Right Volume:  0 - 20 (0 = -80dB, 20 = 0dB)

Treble/Bass:        0 - 12 (0 = -12dB, 6 = 0dB, 12 = +12dB)

Mix:                0 - 3  (0 = -12dB,  1 = Mix PSG, 2 = don't mix

PSG, 3 = reserved)

=====================

2. VIDEO SHIFTER CHIP

=====================

2.1 EXTENDED COLOUR PALETTE

---------------------------

$ff8240 - $ff835e   rgb colour registers

colour word:

bit       11 10  9  8    7  6  5  4    3  2  1  0

meaning   R0 R3 R2 R1   G0 G3 G2 G1   B0 B3 B2 B1

Rx = Red , Gx = Green, Bx = Blue

where x = bit no.

Giving the following colour range:

intensity level  0 1 2 3 4 5 6 7 8 9 a b c d e f

4-bit nibble     0 8 1 9 2 a 3 b 4 c 5 d 6 e 7 f

2.2 SCREEN ADDRESS REGISTERS

----------------------------

$ff8201   screen base address high    (vbasehi)

$ff8203   screen base address middle  (vbasemd)

$ff820d   screen base address low     (vbaselo)

2.3 HORIZONTAL SCROLLING

------------------------

$ff820f   over-length line width      (linewid)

$ff8265   pixel scroll offset         (hscroll)

===============

3. BLITTER CHIP

===============

3.1 BLITMODE TOS CALL

---------------------

Xbios call 64 - Blitmode

flag.w    Status

bit 0 - enable

1-14 - reserved

15 - 0 = clear

1 = get

Blitmode  64

trap #14  (tidy #4)

Returns d0.w:

Bit 0 - set (Blit on)

1 - set (If blitter present)

2-14 - reserved

15 - clear

3.2 BLITTER CHIP REGISTERS

--------------------------

Address   Length    Description

$ff8a00   .w        Half tone RAM 0

$ff8a02   .w        Half tone RAM 1

$ff8a04   .w        Half tone RAM 2

$ff8a06   .w        Half tone RAM 3

$ff8a08   .w        Half tone RAM 4

$ff8a0a   .w        Half tone RAM 5

$ff8a0c   .w        Half tone RAM 6

$ff8a0e   .w        Half tone RAM 7

$ff8a10   .w        Half tone RAM 8

$ff8a12   .w        Half tone RAM 9

$ff8a14   .w        Half tone RAM 10

$ff8a16   .w        Half tone RAM 11

$ff8a18   .w        Half tone RAM 12

$ff8a1a   .w        Half tone RAM 13

$ff8a1c   .w        Half tone RAM 14

$ff8a1e   .w        Half tone RAM 15

$ff8a20   .w        Source x increment

$ff8a22   .w        Source y increment

$ff8a24   .w        Source address high word

$ff8a26   .w        Source address low word

$ff8a28   .w        Endmask 1

$ff8a2a   .w        Endmask 2

$ff8a2c   .w        Endmask 3

$ff8a2e   .w        Destination x increment

$ff8a30   .w        Destination y increment

$ff8a32   .w        Destination address high word

$ff8a34   .w        Destination address low word

$ff8a36   .w        x count

$ff8a38   .w        y count

$ff8a3a   .b        Half tone operation (HOP)

$ff8a3b   .b        Logical operation (OP)

$ff8a3c   .b        bits 0-3 - Line number

bit  5   - Smudge

bit  6   - HOG

bit  7   - Busy

$ff8a3d   .b        bits 0-3 - Skew

bit  6   - NFSR

bit  7   - FXSR

3.3 LOGICAL OPERATIONS

----------------------

OP register

Value     Result

0         all 0 bits

1         source AND destination

2         source AND NOT destination

3         source

4         NOT source AND destination

5         destination

6         source XOR destination

7         source OR destination

8         NOT source AND NOT destination

9         NOT source XOR destination

10        NOT destination

11        source OR NOT destination

12        NOT source

13        NOT source OR destination

14        NOT source OR NOT destination

15        all 1 bits

3.4 HALF TONE RAM OPERATIONS

----------------------------

HOP register

value     operation

0         all 1 bits

1         half tone RAM

2         source

3         source AND half tone RAM

===================

4. CONTROLLER PORTS

===================

4.1 JOYSTICK ADAPTER

--------------------

Pin connections for two joysticks into one STE controller port.

Port A/B  Joystick 0/1  Joystick 2/3

1             1             -

2             2             -

3             3             -

4             4             -

5             -             -

6             6             -

7             7             7

8             -             -

9             8             8

10            -             6

11            -             1

12            -             2

13            -             3

14            -             4

15            -             -

4.2 CONTROLLER PORT REGISTERS

-----------------------------

Port A joysticks are 0 and 2, Port B are 1 and 3

Paddles are 0 for Port A, 1 for Port B

$ff9200   xxxx xxxx xxxx 3120      Fire Buttons

$ff9202   UDLR UDLR UDLR UDLR      Joystick Direction

Joy3 Joy2 Joy1 Joy0

$ff9210   xxxx xxxx nnnn nnnn      Paddle 0 X

$ff9212   xxxx xxxx nnnn nnnn      Paddle 0 Y

$ff9214   xxxx xxxx nnnn nnnn      Paddle 1 X

$ff9216   xxxx xxxx nnnn nnnn      Paddle 1 Y

$ff9220   xxxx xxnn nnnn nnnn      Light Pen/Gun X

$ff9222   xxxx xxnn nnnn nnnn      Light Pen/Gun Y

UDLR for joysticks 0 and 1 are bi-directional

=============

5. COOKIE JAR

=============

5.1 COOKIE POINTER

------------------

Cookie jar pointer address

$5a0  L   p_cookies      Pointer to cookie jar

5.2 COOKIE NAMES AND MEANINGS

-----------------------------

Official Atari Cookies:

Name:     _CPU

Values:   The  number here is the decimal value of the  last  two

digits of the processor present in the machine,  indicating which

CPU of the 68000 family it is:  00, 10, 20, 30. For instance, the

value 30 indicates a 68030 processor.

Name:     _VDO

Values:   The  high word of this cookie contains a number from  0

to 2 which indicates what type of video shifter is fitted:

0 - Standard ST video shifter

1 - STE video shifter

2 - TT Graphic Chip

Name:     _SND

Values:   Here  it  is  the bits which tell us  about  the  sound

hardware.  Bit  0  set indicates the presence of a  Yamaha  sound

chip, bit 1 of the DMA sound chip.

Name:     _MCH

Values:    This cookie also uses the high word,  so that the  low

word can be used for version changes, and the value describes the

overall machine:

0 - Standard ST

1 - STE

2 - Mega ST

3 - TT

Name:     _SWI

Values:   This  cookie is used to indicate the positions  of  the

configuration  switches on Mega STEs and TTs.  At  present  these

switches are unused.

Name:     _FRB

Values:   This longword value will be the address of the fast RAM

buffer,  or 0 if no fast RAM buffers are fitted.  This cookie  is

not found on standard STs or STEs.