# TTP

Please read the Whole document

-----------

- Prepare 28-pin socket:

- cut pins: 1, 20, 22, 27,

- Solder together pins 20 and 22 using piece of wire,

- to pin no. 1 solder the white wire,

- to pin no. 27 solder the red wire.

- Locate the Atari OS 28pins ROM (CO61598)

- Soldier the Prepared Socket onto the ATARI OS ROM.

- Solder the White wire from socket to pin no. 13 PIA (CO14795)

- Solder Red wire from socket to pin no. 12 PIA (CO14795)

- To pin no. 12 and 13 in PIA solder resistors 4K7,  second ends of resistors connect with pin no. 20   in PIA (+5V)

ATARI 65XE 130XE (and similar)

------------------------------

- Find on the board integrated circuit NE555  (has 8 pins). At its pin no. 3 is path. You should cut it at this pin.

- To pin no. 3 NE555 solder blue wire from connector

- To pin no. 34 PIA CO14795 solder green wire from connector

ATARI 800XL

-----------

- Find on board integrated circuit U19 74LS14.

It has 14 pins. At its pin no. 8 is path. You

should cut it at this pin.

- To pin no. 8 U19 74LS14 solder blue wire from

connector

- To pin no. 34 PIA CO14795 solder green wire

from connector

- Find on board integrated circuit MMU. It has

symbol CO61618. At its pin no. 15 is path. You

should cut it at this pin.

- To pin 15 MMU solder grey wire from connector.

- To integrated circuit CO61598, to its pin no. 20

solder orange wire from connector (on this

integrated circuit is soldered socket)

- Find on board integrated circuit with symbol

74138 (74LS138)

- To its pin no. 9 solder yellow wire from

connector

- In back of computer drill hole and mount button.

On the diagram it has symbol P

- To its solder two white wires from connector.

- Brown wire from connector solder to socket

soldered at integrated circuit CO61598

Wire solder to pins 20 and 22, which are soldered

together before.

- Black wire from connector connect with ground

on board. This is a wide path rounded the board.

Or solder to pin 14 soldered socket.

- Red wire from connector connect with pin no. 28

on soldered socket.

- TTP board preserve from short-circuit and put it

on computer's board.

- To socket put eprom with program for TTP

- TTP board connect with connector.

Markers should be next to each other.

- Assemble computer.

- Turn on power.

After pressing TTP button at back of computer

on the screen you will see the menu.

The view of TTP board at side of elements.

REMARKS

-------

- Dismount up part of ATARI

- Remove keyboard

If you pull the belt, the keyboard will

disconnect with socket which is on board.

- Remove board, remove sheet metal

- Solder with using warmer soldering-iron (12V-24V)

(power supply throught transformer which

separate from 220V). Do not use any transformer

soldering-iron!

- Test of running of TTP should be done when board

is not mounted yet and without mounted keyboard.

- When everything is ok you should mount computer.

Proposed way of TTP running

---------------------------

- remove the casing of computer

- solder the 28-pins socket to integrated circuit

CO61598

- turn on the computer, should works correctly.

If not, probably is short-circuit between pins.

- solder the white and the red wire from soldered

socket to integrated circuit PIA CO14795

- solder all the wires from TTP connector to

described before points on Atari board

- cut path between pin no. 15 MMU CO61618 and

pin no. 20, 22 OS Atari CO61598

- cut path between pin no. 3 and the rest

integrated circuits (RST). This concern 65XE or

similar

for 800XL cut path between pin no. 8 U19 74LS14

and the rest integrated circuits.

- in TTP connector join together wires grey and

orange,

also join together wires blue and green,

- turn on the computer, should works correctly.

- remove made before joines (grey-orange and

blue-green),

- to soldered socket put TTP eprom

- connect to connector the TTP board

- look out on short-circuit! Isolate

anything that need it.

- turn on the computer, should work correctly

- turn on TTP (press button), you will see

screen with TTP menu.

REMARK

------

- eprom has sign which desymmetrical it. The sign

should be in the same place where the sign of

integrated circuit CO61598 is.

Different putting eprom to socket may damage

eprom or computer.

- TTP connector should be connect with TTP board

in that way: white signs should be next to each

other. Other way of connect it may damage

computer

- all connections on the board should be done

when the power is turn off.

- solder only with using warmer soldering-iron.

Power throught transformer which separate from

220V (220/12V or 220/24V)

- module is checked and works correctly

- eprom works correctly (was checked)

Good luck!

Zenon/Dial

Translation: K.P.