SAMPLING: THEORY

----------------

By Frankenstein

Just cured from the packer virus, and now this! A sample virus.

I cought it when I looked at the Falcon030 with its 16-bit

sample resolution..

Yes, we're going to talk about audio sampling! Every time you

read sample in this article, you can also read 'audio sample'.

We're going to dig deep into this subject, so it will be very

useful for experienced users.

SAMPLES?

--------

A sample is digital quantity converted from an analogue signal

with help of an ADC. If you don't understand this, please read

the article about sampling hardware.

Playing samples has (normally) nothing to do with the great (or

less great) facilities of your soundchip. By playing samples you

directly send voltages to the speaker.

Let me tell you something about audio speakers first. When a

speaker is in rest (neutral position), there's no noise. It

starts to produce noise if it vibrates. Lets see:

speaker    /

-------   /

/

minimum   <- O -> maximum

(negative)   \    (positive)

\

\

After reading the article about sampling hardware from AlphaSys,

we can conclude that sampling is nothing more than recording the

state of the speaker (no.. really?).

Resolution

----------

An analogue signal can contain an unlimited amount of values,

while a digital signal has only two values, you know: 0 and 1.

Actually the number of available values to represent the

analogue signal depends on the number of bits we use.

Notice that we are talking about the amplitude of the signal

now.

The more bits we use, the more values we have to represent the

amplitude.

Or.. with more values we can divide the amplitude into smaller

steps.

The amount of values used to divide the signal, represented in

bits is called the resolution.

Now I have to tell you that most CD players use 16-bits. CD's

are recorded at 16-bits resolution, so CD players get the most

out of the CD when using the same resolution to play it back!

You might have seen adverts of CD players which have more than

16 bits (e.g. 18 or 20). Although those players still read

16-bits from the CD, it's somehow possible to let it internally

work with more bits.

Don't worry about it right now. Just remember that it's

possible.

To be complete, its said that you can calculate the so called

'signal-noise distance' (a measurement which has everything to

do with the quality of the sound). This can be done by

multiplying the number of bits by 6.

S/N = resolution * 6

Here are some facts:

resolution signal-noise

---------- ------------

1-bit         6 dB (PC)

4-bit        24 dB (ATARI XL/XE)

8-bit        48 dB (ST/STE/AMIGA)

12-bit        72 dB (ST/STE)

16-bit        96 dB (ATARI FALCON)

(dB = decibels)

A very good and expensive record player is able to reach about

60 dB, while there are CD-players which can reach over the 100

dB!

Frequency and Memory

--------------------

There's another thing involved in the great quality of CD

players and hifi samplers. This is the frequency of recording

samples.

The first thing noticed is that the human ear can only hear

frequencies between about 20 and 20kHz. By recording we have to

take advantage of this knowledge by using a filter (a so called

'anti alias' filter), to get rid of the signals we can't hear

anyway.

There are people who still think this makes a hearable

difference and therefore prefer the analogue record player above

any CD-player (sick!).

According to the theory of Nyquist and Shannon we have to use a

play/record sample frequency of 'two times' the maximum

frequency of the signal (which is 20 kHz as mentioned earlier).

In other words; after recording with such a high sample rate, we

can play it back and reconstruct the exact same analogue signal

again <grin>.

So, theoraticly we have to use a '2 times 20' = 40 kHz sample

rate. Now take a look at CD players..

CD-players use a playback frequency of 44.1 kHz. Calculating:

44.100

------ = 22.050 kHz

2

This is a little more than the 20kHz we've mentioned. I've read

that this has something to do with the angle of the filter which

can't be exactly 90 degrees. That's all I know.

(Alphasys: It is correct. For the Freax: When you are using a

filter, you always have a Resistor/Capacitor set. The Complex

notation of this is R+iC, which is the representation of a

Vector. This vector would have an angle of 90 deg. when there is

no Resistor. Too bad that you always need a Resistor to set the

filter to a certain frequency. This means that the filter can

never be 90 deg.)

44.1 kHz you said? This means that it plays 44100 samples per

second! So for one second we need about 86 kB (44100 x 16-bits)

memory?

Yes, but that's only mono! That's 172 kB per second in stereo!!

Hmm.. that's about 10MB per minute. A CD can hold upto 80

minutes, so that would be about 800MB!

Practical use

-------------

Pfew! But do we really need such a high frequency? Well, in most

cases we can record speech at a sample rate between 5 and 10kHz

while reasonable quality music needs about 15kHz. Beware that

every frequency under 40kHz is not perfect.

Remember the rule of Nyquist and Shannon. Low frequencies only

need a sample rate of two times the highest frequency of the

signal. When I'm looking at the build in equalizer of my SONY

stereo amplifier, I notice that the 15kHz bar barely moves when

playing low-noise music like 'Obituary' (for example). The

6.3kHz (and lower) bars are more important in that case. A

frequency of two times 6.3kHz would be OK for this kind of

music. Beware though that we're still talking about a resolution

of 16-bits!

When taking the track 'Stranger Aeons' from 'Entombed', we see

the 15kHz bar bouncing much more. OK, just wait a second. I'll

get some CD's from my younger brother..

Here I've got 'Dangerous' from 'Michael Jackson'. This CD really

moves the 15kHz bar high up in the air. Esspecially the songs

'In the closet' and 'Dangerous'.

If you like to experiment with measurements like this, I'll

recomment you to buy the Atari Falcon030. This computer has a

DSP (digital signal processor) which is capable of sorting out

the frequencies (Fast Fourier Transformation) in REALTIME! With

some software you can create equalizer graphics, voice

recognition, music recognition (!) and much more neat stuff.

Ofcourse you could also buy a NeXT computer which has this same

DSP (but a NeXT is much more expensive).

Let's get back to the subject..

The playback resolution depends on which computer you're using.

The ATARI XL/XE's are capable of 4-bit samples using POKEY or

just 1-bit samples by using the keyclick bit from GTIA.

4-bits resolution also means that we can store two samples in

one byte! (saves us a lot of memory).

In fact, playing samples on the XL/XE is very easy compared to

the normal ST. We have a special option in our Pokey chip for

'volume only' which the ST lacks. Therefore if you want to play

samples on the normal ST, you need a 'sample to frequency' table

for converting the samples (Quarted has made a great table with

which you can reach 12-bit resolution!).

Bluaaarchh! Fortunately the Atari STE/TT and Amiga ARE capable

of playing samples without conversion. On these machines you can

output samples of 8-bit resolution by using DMA! (direct memory

access). This means that you can load a sample into ram, give

the soundchip the start and end addresses of the sample and it

will be played without help of the processor!

This brings us to another point of the subject. How to play

these samples on the ATARI XL/XE? Sorry guys, our Pokey chip has

no DMA. Therefore we have to play samples by using almost every

clock cycle of the 6502 processor.

I've been experimenting with this for a while and was very

surprised to find the maximum sample play rate to be about 68

kHz! This rate isn't practical ofcourse since:

68000 x 4-bits = 32000 bytes

Bogus! 32k gone in one second!

On side two of this issue you'll find the 'worthless' routine

for 68kHz.

Read the next article for the full details about the routines.

Remember my drawing of the speaker? Aren't you curious which

value represents which state of the speaker? Well.. I was. So

after some experiments and reading some documents I could make

this table:

SPEAKER POKEY STE/AMIGA

------- ----- ---------

+7    15         7

+6    14         6

+5    13         5

+4    12         4

+3    11         3

+2    10         2

+1     9         1

0     8         0

-1     7        15

-2     6        14

-3     5        13

-4     4        12

-5     3        11

-6     2        10

-7     1         9

-8     0         8

In this table I only showed the upper four bits of STE/Amiga

samples.

If you take a close look and compare the STE/Amiga values with

the Pokey values, you'll see that only the 'negative' bit has to

be exclusive or'ed to be compatible!

(I figured this out a long time ago when I tried to play samples

from SoundTrackers on my 800XL).

The samples from the STE/Amiga are so called 'signed' samples

(because of using the sign-bit).

OVERSAMPLING

------------

While reading about the interesting subject of sound sampling, I

often saw this strange word: 'oversampling'.

It turned into an obsession to find out what this actually ment.

Oversampling is an extra trick to improve the sound quality.

It's done by calculating the intermediate value of two samples.

Let's give a drawing:

--- time --->

7                            S

6                          So

5                         o

4    S                   S

3     oS

2   o   oSoS

1           oS

0 -S----------o---------o-----

1              S

-2

-3               o      S

-4

-5                S    o

-6                 o

-7                  SoS

S : sample      (data)

o : oversample  (calculated)

This example shows 4-bit resolution samples with singular

oversampling!

To maintain the same frequency you have to play both samples and

oversamples in the same time (not AT the same time!). So, if the

frequency was for example 15kHz, you have to play samples and

oversamples at 30kHz.

How about multiple oversampling? I've been told that if you talk

about multiple oversampling, you just keep on taking

intermediate values between both samples and oversamples, like:

Normal    S.......S.......S.......S

1-time    S...O...S...O...S...O...S

2-times   S.O.O.O.S.O.O.O.S.O.O.O.S

3-times   SOOOOOOOSOOOOOOOSOOOOOOOS

You can see that the frequency to play samples and oversamples

can just be calculated by:

Play freq.= Normal freq. x ( 2^n )

(where 'n' is the times oversampling)

I think this technique is not only handy to improve the sound

quality, but also to adjust te sample frequency without 'not

playing' data.

That's the whole story. Read the next article to find out about

the practical examples on this magazine.

Hope you liked this article. There's at least one who liked it..

that's me!

<END>