PrintXL  1997 THOR-Software, Software documentation

_____________________________________________________________________________

Printer modes:

Mode 0	=	Default mode: Convert EOL to LF,CR

Mode 1	=	One to One mode: no conversions take place

Mode 2	=	Seven Bit Mode: Convert EOL to LF,CR.

Ignore all non-printable characters.

Mode 3  =	Gfx mode: Convert EOL to LF,CR. Print all

remaining characters as graphics in a NEC P6

compatible mode.

Setting the printing mode:

Either by the printer unit: (Pn:)

Unit 1	=	Mode 0

Unit 2	=	Mode 2

Unit 3	=	Mode 3

Unit 4	=	Mode 0

Unit 5	=	Mode 1

Or by providing the printer mode in AUX2 of the open call,

or with a POINT command. The sector number contains the printer mode.

The NOTE command can be used to read the current mode.

Another way is to control the printer mode thru ESC sequences.

BE WARNED! This WON'T work in Mode 1 since all ESC parsing is turned off in

this case. Once you've switched to mode 1, NO ESC sequence can abort this

mode! (ESC is the control character hex $1b=27)

<ESC>*1		Mode 1 (one to one)

<ESC>*2		Mode 0 (Default mode)

<ESC>*3		Mode 2 (7Bit mode)

<ESC>*4		Mode 3 (GFX mode)

<ESC>*5		This one is special and causes a tiny delay. Most useful

after sending a reset sequence <ESC>@ over the port, to

give the printer some time before sending new data.

Additonal XIO commands:

XIO 32		Causes a delay like <ESC>*5

XIO 37		Point, see above

XIO 38		Note, see above.

Additional features:

The printer interface software comes in two versions: PrintXL and PrintXLT,

controlled by the "Turbo" switch in the source code.

The later version is setup to co-operate with the "TurboBasic" language and

is loaded between the DOS and the Turbo-Basic. The longer "PrintXL" release

occupies the math pack area $d800 to $e000 and requires a hardware hack.

This hack allows software control of the MPDisable line thru an unused pin

of the PIA port B $d301 (I hope in a way compatible to the XE series,

although I haven't tried it!). Except that, NO ADDITIONAL MEMORY IS NEEDED,

the handler table is placed in the old printer buffer instead.

Hence, this version should be VERY compatible to almost all software, only

tiny adjustments might be necessary. It provides an additional "requester"

in case the printer runs out of paper, to give the user a chance to reload

the paper without aborting the print process.