WARNING: This document is currently outdated!!!

####  #      ####   #####  #####

#    # #     #    # #      #       The Clarke Assembler

#      #     ######  ####   ####

#    # #     #    #      #      #  (C)1991 Lutz Vieweg

####  ##### #    # #####  #####

------------------    Manual   ------------------------

0 Preface

---------

Please excuse linguistic mistakes in this text. English is not

my native language. If you are not great at english, enjoy the

easy words that are used, if your native language is english,

think of what would be if you had to translate this text from

german...

This manual is written for people who have some experience in

programming assembler on any computer. The HP48sx is not the

right processor to learn assembler.

Of course there are NO (0.00, zero) warranties that there's one

single true word in this text. Your Monitor may blow up while

you read it, your grand-ma may die instantly having taken a spot

on it, I cannot take responsibility on anything that happenes

with this text or with the assembler.

1.0 The Clarke-CPU Architecture

-------------------------------

The Clarke processor is a child of the "Saturn" processor family.

The Saturn processor family is a rather old one. As far as I know

it was first used in HP's 71, later modified and customized for usage

in the HP19B, 28C, 28S, 10B, 20S, 21S and finally 48sx.

1.1 The Saturn-CPU's characteristics

------------------------------------

- CMOS technology (low power consumption)

- 4 bit data-bus (memory organized in nibbles)

- 20 bit adress-bus (able to adress 1 mega-nibble = 512kB unpaged)

- Micro-code based instruction-set

- 4 not-omni-purpose 64-bit data-registers (called a,b,c,d)

- 5 64-bit "scratch"-registers (called r0,r1,r2,r3,r4)

- 2 20-bit adress-registers (called d0,d1)

- one 4-bit "field-pointer" register (called p)

- one 20-bit program counter

- 8-level return-adress stack, 20-bit each level

- one 16-bit status-register (called st)

- one 4-bit hardware-status-register (called hst)

- one 16-bit I/O input-register (called in)

- one 12-bit I/O output-register (called out)

- a carry flag

- operating in either HEX-mode or DEC-mode

1.2 Clarke-specific features

----------------------------

The Clarke processor runs up to 2000000 cycles per second. Actually,

the processor-clock runs at a speed of about 1,96 MHz in the HP48sx.

There has been some new instructions implemented in the processor's

micro-code, compared to the 28s' CPU:

RSI

MOVE.dd     #$xxx,a      (needs more nibbles than MOVE.dd #$xxx,c !)

MOVE.fs|a   a|c,rx

MOVE.fs|a   rx,a|c

EXG.fs|a    a|c,rx

ADD.fs|a    #$x,a|b|c|d

SUB.fs|a    #$x,a|b|c|d

LSR.fs|a    #1,a|b|c|d

BCLR        #$x,a|c

BSET        #$x,a|c

BBC         #$x,a|c,label

BBS         #$x,a|c,label

JMP         (a)

JMP         a|c

move.a      pc,a|c

exg.a       a|c,pc

BUSCD

1.3 Register usage

------------------

The data-registers are the ones that are used to hold values,

do arithmetics on them or move them elsewhere. There is some

kind of hierarchy between the four registers, because it is not

possible to do the same instructions on each of them as it

is at the 680xx-family's data-registers. From "good" to "bad"

this hierarchy is c,a,b,d. There are a lot of adressing modes

available only to the c or a register...

There are no differences in the way the processor supports

each of the five "scratch"-registers. I set the quotation marks

because these registers a very important for machine-language

programmers even if they are supposed to hold "scratch" due to

their name.

The two adress-registers are also equivalent, they are used to

hold the adress of any location you want to access in the

memory. Note that there is no "absolute" adressing possibility

if want to move data around the memory.

The p register is not found similar on other machines. The normal

usage of this register will be clear if you look at the "size

extensions" chapter. But you can use this register also to hold

one nibble of any kind of data. There are some strange adressing

modes using this register. Because of it's meaning to the

move.dd #expr,a.p|c.p instruction it is usually reset to zero

after it has been changed and used.

The program-counter does not need to be discussed here. It just

holds the adress were execution takes place actually.

The stack is much to small!!!! I really don't know how people

can wish to save sillicium by sizing the stack to 8*20 bit.

You'll have to place your own software-handled stack into memory

when you wish to run complex machine-language programs.

The status register consists of 16 bit, which are used by the

operating-system to hold important flags.

The hardware-status-register consists of four bit, having the

following meanings:

Bit 0  XM  Module missing (set by opcode $00, "empty memory")

Bit 1  SB  Sticky bit (used as some sort of carry for shifting operations)

Bit 2  SR  Service request (set if an I/O device needs "service")

Bit 3  MP  Module pulled (really don't know the sense of this bit yet)

The input and output registers are used to transfer data from and

to "devices" that are connected "daisy-chained" to the I/O bus.

I guess HP has decided to keep this chain rather short, at the

HP48sx I know only two devices accessed via I/O registers

(piezo-beeper and keyboard), and they are using memory-register

based I/O in the 48sx as it is done in most computers...

The carry-flag is the only conventional flag that exist in the Clarke

CPU. It is used as on any other processor I know...

2.0 The CLASS Assembler's Mnemonics

-----------------------------------

The mnemonics used by the CLASS-Assembler are not equal to those published

by HP in the IDS-manual. HP's mnemonics were a real horror to every

guy who had programmed machine-language once before. I decided to

use mnemonics that are similar to the ones used by the 680xx-family

Assemblers. This is not the same as the BABEL command-set used by

some former assemblers, but I did not like to use this one because

I think it is rather inconsequently structured. You may define macros

to make BABEL source-codes processable by CLASS.

A typical CLASS source-line looks like this:

label     mnemonic.size  parameter,parameter,parameter  ; comment

where label is a typical assembler-label which can be of any lenght

up to 2^31 bytes. Of course, the label should not be equal to any

register- or other special name.

Mnemonic is just the command that should be translated into machine-code,

with respect to the following size and/or parameters.

The size plays a very important role at the Clarke-processors machine

language. There are a lot of possible sizes available, but they are

of course not available to any opcode/adressing mode combination.

2.1 Size extensions:

--------------------

The great variety of possible size-extensions seems confusing and

often useless. But remember, the Saturn-architecture was developed

to laborate with 64-Bit BCD encoded floating-point-values, and

therefore the size-extensions really make sense.

.1, .2, .3, .... .16

A size extension that consists of a decimal number in the range from

one to sixteen gives just the number of nibbles to be processed by

the opcode. There are only a few commands that can handle one to

sixteen nibbles, but this size extension is also used by some

pseudo-opcodes described later.

This size extension type will be abbreviated '.dd' in the following.

.a

One of the most used size-extension is .a, due to its meaning for the

adress-handling. This extension is available to many commands. It

defines the size to be processed to 20 bits (5 nibbles).

.p

The .p extension tells the command to process the nibble pointed to by

the p register, therefore accessing one nibble only.

.wp

This extension tells the command to access the nibbles 0 to p of the

specified register(s).

.xs

Accesses the exponent sign: Nibble #2 of the registers.

.x

Accesses the whole exponent: Three nibbles,  nibble 0 to nibble 2 of the

registers.

.s

Accesses the sign: Nibble 15 of the registers

.m

Accesses the mantissa: 12 nibbles, nibble 3 to nibble 14 of the registers.

.b

Accesses one byte: 2 nibbles, nibble 0 to nibble 1 of the registers.

These to nibbles are used to hold the exponent (without the exponent

sign).

.w

Acesses one complete register or floating-point value: 16 nibbles,

nibble 0 to nibble 15 of the registers.

If you access the memory by using one of the adress-registers, the

specified size does not affect the location in memory.

MOVE.s   (d0),c

MOVE.xs  c,(d0)

do both access the same adress in memory, but they use different

nibbles of the c register.

In most cases, CLASS will accept the following equivalents:

.2  = .b

.3  = .x

.4  = .as

.5  = .a

.16 = .w

2.2 Size extension graphical summary:

-------------------------------------

Assuming register p to consist the value 9

--------- Nibble of register ------------------

15 14 13 12 11 10 09 08 07 06 05 04 03 02 01 00     Size-extension

.. .. .. .. .. .. .. .. .. .. .. ** ** ** ** **     .a

.. .. .. .. .. .. ** .. .. .. .. .. .. .. .. ..     .p

.. .. .. .. .. .. ** ** ** ** ** ** ** ** ** **     .wp

.. .. .. .. .. .. .. .. .. .. .. .. .. ** .. ..     .xs

.. .. .. .. .. .. .. .. .. .. .. .. .. ** ** **     .x

** .. .. .. .. .. .. .. .. .. .. .. .. .. .. ..     .s

.. ** ** ** ** ** ** ** ** ** ** ** ** .. .. ..     .m

.. .. .. .. .. .. .. .. .. .. .. .. .. .. ** **     .b

** ** ** ** ** ** ** ** ** ** ** ** ** ** ** **     .w

The following entries are only valid for the

MOVE.dd  #xxx,a.p|c.p

command, still assuming p to be 9

.. .. .. .. .. .. ** .. .. .. .. .. .. .. .. ..     .1

.. .. .. .. .. ** ** .. .. .. .. .. .. .. .. ..     .2

.. .. .. .. ** ** ** .. .. .. .. .. .. .. .. ..     .3

.. .. .. ** ** ** ** .. .. .. .. .. .. .. .. ..     .4

.. .. ** ** ** ** ** .. .. .. .. .. .. .. .. ..     .5

.. ** ** ** ** ** ** .. .. .. .. .. .. .. .. ..     .6

** ** ** ** ** ** ** .. .. .. .. .. .. .. .. ..     .7

** ** ** ** ** ** ** .. .. .. .. .. .. .. .. **     .8

** ** ** ** ** ** ** .. .. .. .. .. .. .. ** **     .9

** ** ** ** ** ** ** .. .. .. .. .. .. ** ** **     .10

** ** ** ** ** ** ** .. .. .. .. .. ** ** ** **     .11

** ** ** ** ** ** ** .. .. .. .. ** ** ** ** **     .12

** ** ** ** ** ** ** .. .. .. ** ** ** ** ** **     .13

** ** ** ** ** ** ** .. .. ** ** ** ** ** ** **     .14

** ** ** ** ** ** ** .. ** ** ** ** ** ** ** **     .15

** ** ** ** ** ** ** ** ** ** ** ** ** ** ** **     .16

2.3 Parameters

--------------

The Clarke-processors commands use up to three parameters. The

following types of parameters are supported by CLASS:

label    A string that defines the name of a label, a numerical

or one that is a offset to the object-code's begin. Example:

JMP label

#expr    An expression that is calculated while assembling.

See also chapter 2.4            Example:

BSET #expr,st

a|b|c|d  One of the four data-registers a,b,c and d.

The '|' sign is used as a logical OR in the following

Example:

MOVE a,c

c.dd     The nibble pointed to by dd (decimal digit 0 to 15) in c register

c.p      The nibbles from the one p points to higher ones in c register

a.p      The nibbles from the one p points to higher ones in a register

rx       One of the five scratch-registers r0,r1,r2,r3 and r4

p        The p register

st       The status-register

hst      The harware-status-register

in       The I/O input-register

out      The I/O output-register

2.4 Expression evaluation

-------------------------

Expressions may consist of labels, absolute values, operators

and parentheses. Expressions are evaluated without any kind

of hierarchy, you have to set parentheses if required. This behaviour

may change in future versions of CLASS.

Numerical labels and those who include an offset to the object's

begin are treated the same in the actual version. This may also

change in future.

2.4.1 Operators

---------------

Actually, there are only a few operators supported:

+   Addition

-   Subtraction / negative sign

*   Multiplication

/   Division

|   OR

&   AND

\   XOR

2.4.2 Absolute values

---------------------

Decimal numbers begin with a number from 0 to 9.        1234

Hexadecimal numbers are prefixed with an dollar sign.  $12ab

Binary numbers are prefixed with a per-cent sign.      %01101

ASCII-codes are written between quotation marks.       "xy12"

2.5 Adressing modes

-------------------

The following is a complete (?hope so?) list of the available

adressing modes:

#expr          Absolute value

label          Absolute adress

a|b|c|d        Data-register direct

(a)            Data-register indirect (used in JMP (a) only)

d0|d1          Adress-register direct

(d0)|(d1)      Adress-register indirect

r0|r1|r2|r3|r4 Scratch-register direct

p              P-register direct

st             Status-register direct

hst            Hardware-status-register direct

in             I/O input-register direct

out            I/O output-register direct

c.dd           Data-register c with special nibble-pointer

a|c.p          Data-register from nibble pointed to by p register upwards

2.6 Mnemonic summary

--------------------

Many of the mnemonics listed here are sharing the format

mnemonic.size source,dest

Notice that this is similar to the 680xx family's assembly format,

and the opposite of Intel's one.

The following abbreviations will be used:

fs  Field selctor. One of the extensions .p, .wp, .xs, .x, .s, .m, .b, .w

rx  One of the five scratch-registers r0,r1,r2,r3,r4

Notice that commands that use the .a size-extension need less nibbles

to be coded in most cases.

2.6.1 Data transfer instructions

--------------------------------

MOVE

----

The most used mnemonic at all is the move-command. There are a lot of

adressing modes available, but they are not very similar to the

adressing modes used by other processors. In fact, there are

missing very potent ones like index'ed adressing.

Available:

move.fs|a    b|c    ,a

move.fs|a    a|c    ,b

move.fs|a    a|b|d  ,c

move.fs|a    c      ,d

move.3       c      ,st         ; only 12 of the 16 st-bits are affected!

move.3       st     ,c

move.a|4     a|c    ,d0|d1      ; move.a|4 d0|d1,a|c does not exist!!!

; use exg.a|4 a|c,d0|d1 if required

move.fs|a|dd a|c    ,(d0|d1)    ; .a and .b are needing less nibbles

move.fs|a|dd (d0|d1),a|c        ; .a and .b are needing less nibbles

move.w       a|c    ,rx

move.w       rx     ,a|c

move.fs|a    a|c    ,rx         ; these two need more nibbles than the

move.fs|a    rx     ,a|c        ; previous - they were implemented later

move.4       in     ,a|c        ; this instruction fails if it is executed

; on an odd adress. A bug in the microcode

move.3       c      ,out

move.s       c      ,out        ; out high-nibble affected

move.1       p      ,c.dd

move.1       c.dd   ,p

move.1       #expr  ,p

move.a       #expr  ,d0|d1

move.ao      #expr  ,d0|d1      ; read "RELTAB" description

move.as      #expr  ,d0|d1

move.b       #expr  ,d0|d1

move.dd      #expr  ,a.p|c.p    ; this instruction is used that often, I

; decided to allow 'a' or 'c' as destination

; take care of the contents of p !

move.ao      #expr  ,a.p|c.p    ; read "RELTAB" description

move.a       pc     ,a|c

EXG

---

EXG exchanges the contents of two registers. It makes is no difference

which register is defined first:

exg.w c,r0  =  exg.w r0,c   and so on...

EXG is a very usual instruction on the Clarke processor, it allows to

use the scratch-registers as quick-accessable data storage and

it is the only way to move an adressa out of an adress-register.

Available:

exg.w        c|a    ,rx

exg.fs|a     c|a    ,rx         ; this one needs more nibbles than the

; previous - it were implemented later

exg.a        c|a    ,d0|d1

exg.fs|a     a|b|c  ,a|b|c

exg.fs|a     c      ,d

exg.1        p      ,c.dd

exg.a        a|c    ,pc

PUSH

----

PUSH "pushes" a 20-bit value from register c onto stack.

Available:

push

POP

---

POP transfers the top 20-bit of the stack into the lower 20-bit of

register c.

Available:

pop

2.6.2 Arithmetic instructions

-----------------------------

ADD

---

ADD is just what it is on every micro-processor: It adds two values

and stores the result in a register. There are powerful increment

instructions similar to the ADDQ instruction in the 680xx family,

but I decided to use the same mnemonic, add, for them.

Available:

add.a        #expr  ,d0|d1   ; expr must be >=1 and <=16

add.fs|a     #expr  ,a|b|c|d ; expr must be >=1 and <=16. In fact,

; only the .b, .x, .a, .m and .w extension

; work properly on this instruction.

add.fs|a     a|b|c  ,a|b|c   ; adding one register on itself is allowed!

add.fs|a     c|d    ,d|c     ; It's then just a shift left one bit!

add.a        p+1    ,c       ; This one is always done in hex-mode

; Wonderful to combine with .wp size extension

SUB

---

SUB subtracts the first parameter from the second and stores the

result in the register that has to be specified as the second parameter.

Available:

sub.fs|a     a|b|c  ,a|b|c

sub.fs|a     c|d    ,d|c

sub.a        #expr  ,d0|d1      ; expr must be >=1 and <=16

add.fs|a     #expr  ,a|b|c|d ; expr must be >=1 and <=16. In fact,

; only the .b, .x, .a, .m and .w extension

; work properly on this instruction.

SUBR

----

SUBR is not available on the 680xx-family. It subtracts the second

parameter from the first and stores the result in the register that

has to be specified as the second parameter.

Available:

subr.fs|a    b      ,a

subr.fs|a    c      ,b

subr.fs|a    a      ,c

subr.fs|a    c      ,d

INC

---

INC increases a specified register by one.

Available:

inc.fs|a     a|b|c|d

inc.1        p

inc.a        d0|d1              ; identical to add.a #1,d0|d1

DEC

---

DEC decreases a specified register by one.

Available:

dec.fs|a     a|b|c|d

dec.1        p

dec.a        d0|d1              ; identical to sub.a #1,d0|d1

CLR

---

CLR stores zero into a register.

Available:

clr.3        st           ; only the lower 12 bits are cleared!

clr.fs|a     a|b|c|d

clr.1        #expr  ,hst  ; the bits set in expr are cleared in hst

NEG

---

NEG converts a registers value into it's 2's complement.

Available:

neg.fs|a     a|b|c|d

NOT

---

NOT inverts all the bits in a register's field.

Available:

not.fs|a     a|b|c|d

2.6.3 Binary-arithmetic instructions

------------------------------------

OR

--

OR bitwise "ors" two registers and stores the result in the second one.

Available:

or.fs|a      a|b|c  ,a|b|c

or.fs|a      c|d    ,d|c

AND

---

AND bitwise "ands" two registers and stores the result in the second one.

Available:

and.fs|a     a|b|c  ,a|b|c

and.fs|a     c|d    ,d|c

LSL

---

LSL shifts the bits in a register to the left, setting the rightmost

bit to zero after each shift.

Available:

lsl.fs|a     #4     ,a|b|c|d  ; a one-nibble shift in fact

lsl.fs|a     #1     ,a|b|c|d  ; is identical to add.fs|a a,a|b,b|c,c|d,d

LSR

---

LSR shifts the bits in a register to the right, setting the leftmost

bit to zero after each shift.

Available:

lsr.fs|a     #4     ,a|b|c|d  ; a one-nibble shift in fact

lsr.w        #1     ,a|b|c|d

lsr.fs|a     #1     ,a|b|c|d  ; needs more nibbles than the

; previous one - later implemented

ROL

---

ROL rotates the bits in a register to the left, setting the rightmost

bit to the value shifted out

Available:

rol.w        #4     ,a|b|c|d  ; that's all - sorry

ROR

---

ROR rotates the bits in a register to the right, setting the leftmost

bit to the value shifted out

Available:

ror.w        #4     ,a|b|c|d  ; that's all - sorry

BSET

----

BSET sets a specified bit in a register. It is not possible to use

this instruction on a bit-number higher than 15. Bit numbers always

counts from 0, the LSB of the register. There's no size-extension.

Available:

bset         #expr  ,a|c      ; expr must be >=0 and <=15

bset         #expr  ,st       ; expr must be >=0 and <=15

BCLR

----

BCLR cleares a specified bit in a register. It is not possible to use

this instruction on a bit-number higher than 15. Bit numbers always

counts from 0, the LSB of the register. There's no size-extension.

Available:

bclr         #expr  ,a|c      ; expr must be >=0 and <=15

bclr         #expr  ,st       ; expr must be >=0 and <=15

2.6.4 Conditional branches

--------------------------

The instructions that affect the program counter do not differ much

from those found on other processors. But there are two anomalies:

Every conditional branch can be also a conditional return-from-

subroutine by setting the branch-offset to zero.

Due to the lack of flags, the conditional branches include the

neccesary test operations.

The following tests are supported:

eq  =   equal

ne  !=  not equal

lt  <   lower than

le  <=  lower or equal

gt  >   greater than

gs  >=  greater or equal

There are four other, special tests:

cc      carry clear      The branch is taken if the carry flag is clear

cs      carry set        The branch is taken if the carry flag is set

bc      bit clear        The branch is taken if the specified bit is 0

bs      bit set          The branch is taken if the specified bit is 1

The label that defines the adress to jump to when the branch is taken,

must be within a range of -128 to 127 around the adress the offset is

defined.

Available:

bcc          label

bcs          label

rtcc                           ; return-if-carry-clear ...

rtcs

beq.fs|a     a|b|c|d,0,label   ; branch is taken if register is zero

bne.fs|a     a|b|c|d,0,label   ; branch is taken if register is not zero

beq.fs|a     a|b|c,a|b|c,label ; branch is taken if registers are equal

beq.fs|a     c|d,d|c,label

rteq.fs|a    a|b|c|d,0         ; return if register is zero

rtne.fs|a    a|b|c|d,0

blt.fs|a     a|b|c,a|b|c,label ; branch is taken if first register is lower

blt.fs|a     c|d,d|c,label     ; than second one

ble.fs|a     a|b|c,a|b|c,label

ble.fs|a     c|d,d|c,label

bgt.fs|a     a|b|c,a|b|c,label

bgt.fs|a     c|d,d|c,label

bge.fs|a     a|b|c,a|b|c,label

bge.fs|a     c|d,d|c,label

rtlt.fs|a    a|b|c,a|b|c

rtlt.fs|a    c|d,d|c

rtgt.fs|a    a|b|c,a|b|c

rtgt.fs|a    c|d,d|c

rtle.fs|a    a|b|c,a|b|c

rtle.fs|a    c|d,d|c

rtge.fs|a    a|b|c,a|b|c

rtge.fs|a    c|d,d|c

beq.1        #expr,hst,label ; branch is taken if hst AND expr is zero

rteq.1       #expr,hst

beq.1        #expr,p,label  ; branch is taken if expr is equal to contents

rteq.1       #expr,p        ; of p register

bne.1        #expr,p,label

rtne.1       #expr,p

bbc          #expr,st,label ; branch is taken if specified bit in st is clear

rtbc         #expr,st

bbs          #expr,st,label ; branch is taken if specified bit in st is set

rtbs         #expr,st

bbc          #expr,a|c,label

rtbc         #expr,a|c

bbs          #expr,a|c,label

rtbs         #expr,a|c

2.6.5 Unconditional branches and other pc manipulation

------------------------------------------------------

Please notice: Use relative branches if possible;

use always the "smallest" jump, that reaches

the desired adress.

Available:

bra.3        label   ; jumps relative (range: -2048 to +2047)

bra.4        label   ; jumps relative (range: -32768 to 32767)

bsr.3        label   ; calls subroutine (range: -2048 to +2047)

bsr.4        label   ; calls subroutine (range: -32768 to +32767)

jmp          label   ; jumps to absolute 20-bit adressa

jsr          label   ; calls subroutine at absolute 20-bit adressa

jmp          a|c     ; jumps to the location pointed to by the lower

; 20 bits of the specified register

jmp          (a)     ; jump to the adress that is defined at the

; memory location pointed to by the lower 20 bits

; of the specified register

rtnsxm               ; return and set XM-flag in hst. Used to catch

; "jumps into the desert" (opcode $00)

rtn                  ; simply return from subroutine

rtncc                ; return and clear carry

rtnsc                ; return and set carry

rsi                  ; return from system interrupt. Not fully explored yet

2.6.6 Miscellaneous instructions

--------------------------------

sethex       ; sets the processor into HEX-Mode

setdec       ; sets the processor into DEC-Mode

Not fully explored yet.

I do not know exactly how the mode affects the processors work.

nop4         ; an artifical 4-nibble-nop, in fact a senseless branch

uncnfg       ; unconfigures all chips (?) and transfers the lower

; 20 bits of register c into each chip controller's

; data pointer. Not fully explored yet.

config       ; sends the lower 20-bits of register c to the chip

; which has daisy chain input high and config flag low

; Not fully explored yet

c=id         ; identifies chip: transfers the ID of the chip which has

; daisy chain input high and config flag low to the

; lower 20 bits of register c

; Not fully explored yet

shutdn       ; sends bus-shut-down command and stops CPU-Clock

inton        ; enables some sort of interrupt (from keyboard?)

intoff       ; disables some sort of interrupt (from keyboard?)

reset        ; sends system-bus-reset command, resets chips. Not fully...

buscc        ; sends system-bus command "C". Not fully...

buscd        ; sends system-bus command "D"? Not fully...

sreq?        ; if any chip on bus needs service, SR-bit in hst is set,

; and the "device-identifier" is latched to the lowest

; nibble of register c

3.0 The CLASS Assembler's pseudo-ops

------------------------------------

There are a lot of so called "pseudo-opcodes" supported - instructions

that are not translated into bits instructing the processor to do something.

3.1 Include files

-----------------

Every serious assembler supports include-files. You may type a line

into your source-code, that has the "include" command in the mnemonic

field and a filename as a parameter. This file will be inserted

into the source while assembly. An include-file (which is, of course

a normal source-file also) may include other files. There's no nesting

limit but your stack's size.

Examples:

include   "blub.a"

include   trash.a

3.2 Constants

-------------

Constants are nibbles created by the assembler that are not executable

code.

3.2.1 Nibble-by-nibble constants

--------------------------------

With the "dc", "dcr" and "dcg" pseudo-ops you can place any kind of data

into your object-file. The format is:

dc.dd   value, value, value, ...

dcr.dd  value, value, value, ...

dcg.dd  value, value, value, ...

dd defines the constant's size.

"dc" puts the values from high- to low-nibble into memory, as it is done

by the 680xx family for example.

"dcr" puts the values from low- to high-nibble into memory, this is how

the Clarke-processor does it. Example:

start       move.a #data,d0

move.a (d0),c

add.a  #5,d0

move.a (d0),a

rtn

data        dcr.5  $12345

dc.5   $12345

After calling this routine, register c will hold $12345 while register a

contents $54321.

"dcg" is used to define graphic-data for the HP48sx bit-plane format.

Not only the order of the nibbles is reversed, the bits itself are

also exchanged. You can shift graphic-data pixel-wise by just shifting

the register that holds the data. Notice that the bit-plane of the HP48sx

is organized byte-wise.

3.2.2 Text-Constants

--------------------

With the "text" pseudo-ob you can place ASCII-codes into your object file.

There's another pseudo-ob, "textr", which is used to reverse the nibbles

of each ASCII. Format:

text    "The quick brown fox jumps over the lazy dog"

textr   "The quick brown fox jumps over the lazy dog"

3.3 The EQU instruction

-----------------------

Use the "equ" pseudo-ob to define the value of a label to be created.

Format:

label     equ     expr

References between label-definitions are solved completely. The following

example will cause no trouble:

jim       equ     bob+jeff-7

bob       equ     jeff/2

jeff      equ     6

The following example will cause trouble:

jim       equ     bob+jeff-7

bob       equ     jeff/2

jeff      equ     jim

>ERROR: Unable to solve reference between following labels:

>jim

>bob

>jeff

3.4 Macros

----------

To define a macro, use this syntax:

name         macro     para1,para2,para3,para4

....

....

endmac

Notice: - You can use any string to define a parameter. It's a good idea

to choose a string that never occures in normal source-code.

- The endmac statement must be the only word in the terminating

line.

- You cannot nest macros.

- Macros must be defined BEFORE they are used in the source.

- Use "?cnt?" to place a unique number for each macro in the

resulting source

- The number of possible parameters is limited to 4

Examples:

rpl         macro    value

dcr.5    value

endmac

absadr      macro    lbl,reg

bsr      absadr?cnt?

absadr?cnt? pop

move.a   #lbl-absadr?cnt?,a

add.a    a,c

move.a   c,reg

endmac

; what follows is an example how the macros

; are called

rpl      $02d9d

absadr   data,d0

move.3   #$123,c

move.3   c,(d0)

; just an example ... has no meaning

data        text     "-----------"

The "absadr"-macro would look like this after expanding:

bsr      absadr1

absadr1     pop

move.a   #data-absadr1,a

add.a    a,c

move.a   c,d0

3.5 The relocation-table

------------------------

To write programs that are relocated before execution I implemented a

pseudo-ob called "RELTAB". Use this pseudo to insert a table of the

following format into your code:

$XXXXX   20-Bit-Offset from PGM's base, where to find first adress to reloc

$XXXXX   2. adress...

...      3. 4. 5. ... adresses

...

$00000   A terminating zero-adress to signal list's end

Entries in this table are made the following way:

Whenever CLASS faces a valid instruction with a size-extension that is ".ao",

it calculates the offset from the begin of the program to the adress

where to find the 20-Bit-adress that is specified within the instruction.

This adress is held in a variable until the RELTAB instruction is found.

Then all the adresses collected becomes entries in the relocation table.

The variables are cleared, and the game starts over.

You may have more than one relocation-table in one program. But be sure

to use the right ones for relocation during execution then.

A sample relocation- and re-relocation-routine comes within the CLASS package.

It'll fit your needs on a HP48sx.

Note: A RELTAB command only cares about the adressas that have been specified

in the source above.

Be sure to avoid a 48sx-garbage-collection between relocation and

re-relocation.

If you fail to re-relocate a program after execution, your system

may crash the next time you call the program. Avoid unwanted

interrupts!!!

4.0 Local labels

----------------

The CLASS assembler supports local labels in a very easy way, to explain

how to use them I will simply explain what CLASS does:

Whenever CLASS finds a label that does end with an "." character, it

assumes this label to be global. It stores the name of such a label

in a special variable.

Whenever CLASS encounters a label with an "." as FIRST character, it

takes the name of the last global label from the special variable,

apends the name of the local label (including the point) and adds

another point at the end of the total name.

This is the whole thing. Now look what you can do with it...

start      text   "the example starts here with a global label"

bra    .1        ; a first reference to a local label

text   "just a filler"

later.                      ; a label that has been inserted later.

; it is a global one, but it does not

; affect the last-global-name variable

text   "just a filler"

.1

text   "this would be the destination of the branch"

.jimbob

text   "local label names are normal strings..."

ende                     ; the end of the routine, a global label

dc.5   start.1.  ;access local label from an outer point...

.1                            ; you can use ".1" as label name again...

dc.5   later.    ; a reference to the later inserted label

bra    .1        ; this jump goes to ende.1.

5.0 Invocation

--------------

You have to invoke CLASS from a command-line interpreter. Use the

following format:

CLASS -a SOURCENAME [-o OBJECTNAME] [-i INCLUDE_DIR] [-ml maximal_lines]

[-mm maximal_macros] [-ul] [-rn] [-mr maximal_reloctab_entries]

[-s SYMBOL_FILE]

Upper-case parameters have to be replaced by a string, lower-case ones

have to be replaced by a decimal number.

Parameters in parentheses are optional

-a   Defines the name of the source file

-o   Defines the name of the object file

-i   Defines the name of the include file directory (the actual path

is searched also)

-s   Defines the name of an optinal symbol-file (e.g. for use with CLDIS)

-ml  Defines the maximal number of source-lines

-mm  Defines the maximal number of macro-definitions

-mr  Defines the maximal number of reloction-table-entries (view "RELTAB")

-ul  Enables listing of unused labels at the end of assembly

-rn  Enables nibble-swapping at end of assembly.

6.0 Adress of the author

------------------------

Lutz Vieweg <lkv@mania.robin.de>

Appendices:

-----------

A. HP48sx Processor Performance - explored by L. Vieweg May 1991

----------------------------------------------------------------

A.1.0 The cycle period

----------------------

The HP48sx Clarke-processor clock runs at 1.96 MHz (in my machine).

About 25% of the processors speed is cut off when you enable the

BitPlane DMA. To run your processor at full speed, clear bit 3 at

$00100.

A.2.0 Instruction execution times

---------------------------------

These are the execution times of the Clarke processor's

most used instructions. This list may be completed or revised later.

Notice: If there are fractional cycles given, calculate the total

cycles first, then round the number (7.5 => 8 and so on...).

The abbreviation "siz" means the total number of nibbles

affected by an instruction (.s=1, .b=2, .x=3, .a=5, .w=16, ...)

Refer to the CLASS-assembler's manual for further information.

Add one cycle for each odd adress the pc runs through. This will

cause some nasty effects when programming time-critical routines.

Instruction      Source    Dest          Cycles

-----------------------------------------------------------------

move.dd          #0        ,c.p          2.5+dd*1.5

normal / access $00100-$0013f

move.dd          (d1)|(d0) ,c|a          21.5+dd*1.5 / 21+dd

move.fs          (d1)|(d0) ,c|a          21.5+siz*1.5 / 21+siz

move.b           (d1)|(d0) ,c|a          19          /17

move.a           (d1)|(d0) ,c|a          23          /20

move.a           c|a       ,(d0)|(d1)    19

move.b           c|a       ,(d0)|(d1)    16

move.fs          c|a       ,(d0)|(d1)    20+siz

move.dd          c|a       ,(d0)|(d1)    20+siz

move.a           a         ,c            8

move.fs          a|b|c|d   ,a|b|c|d      4+siz

move.a           a|c       ,d0|d1        9

move.4           a|c       ,d0|d1        8

move.w           a|c       ,rx           20

move.w           rx        ,a|c          20

move.1           p         ,c.dd         8

move.1           c.dd      ,p            8

move.1           #expr     ,p            3

move.a           #expr     ,d0|d1        10

move.as          #expr     ,d0|d1        9

move.b           #expr     ,d0|d1        6

clr.fs           a|b|c|d                 4+siz

clr.a            a|b|c|d                 8

clr.1            #expr     ,hst          4

exg.w            c|a       ,rx           20

exg.a            c|a       ,d0|d1        9

exg.fs           a|b|c|d   ,a|b|c|d      4+siz

exg.a            a|b|c|d   ,a|b|c|d      8

exg.1            p         ,c.dd         8

push             pop                     18  (the two's total)

add.a            #expr     ,d0|d1        8

add.fs           a|b|c|d   ,a|b|c|d      4+siz

add.a            a|b|c|d   ,a|b|c|d      8

add.a            p+1       ,c            9

sub.fs           a|b|c|d   ,a|b|c|d      4+siz

sub.a            a|b|c|d   ,a|b|c|d      8

subr.fs          a|b|c|d   ,a|b|c|d      4+siz

subr.a           a|b|c|d   ,a|b|c|d      8

sub.a            #expr     ,d0|d1        8

inc.fs           a|b|c|d                 4+siz

inc.a            a|b|c|d                 8

dec.fs           a|b|c|d                 4+siz

dec.a            a|b|c|d                 8

inc.1            p                       4

neg.fs           a|b|c|d                 4+siz

neg.a            a|b|c|d                 8

not.fs           a|b|c|d                 4+siz

not.a            a|b|c|d                 8

or.fs            a|b|c|d   ,a|b|c|d      6+siz

or.a             a|b|c|d   ,a|b|c|d      11

and.fs           a|b|c|d   ,a|b|c|d      6+siz

and.a            a|b|c|d   ,a|b|c|d      11

lsl.fs           #4        ,a|b|c|d      5+siz

lsl.a            #4        ,a|b|c|d      9

lsr.fs           #4        ,a|b|c|d      5+siz

lsr.a            #4        ,a|b|c|d      9

lsr.w            #1        ,a|b|c|d      21

rol.w            #4        ,a|b|c|d      22

ror.w            #4        ,a|b|c|d      22

bset             #expr     ,a|c          8

bclr             #expr     ,a|c          8

bsr.3            rtn                     26   (the two's total)

bsr.4            rtn                     29

bra.3                                    14

bra.4                                    17

;          taken/not taken

bcc.2                                    12/4

b??.fs           a|b|c|d   ,a|b|c|d|0    16+siz/8+siz

b??.a            a|b|c|d   ,a|b|c|d|0    21/13

beq.1            #expr     ,hst          16/8

beq.1            #expr     ,p            17/9

bbc|bbs          #expr     ,a|c          20/12

A.3.0 Overall Performance Comment

---------------------------------

The processor's speed isn't very impressive. There's especially

one point of criticism to the developers I wish to say:

Everyone knows it's not possible to run handhelds processor clocks at

a high speed because of the temperature-problems that would cause

massive battery exhaust when semi-conductors get hot. But if so,

why the devil have you created such a cycle-eating micro-code?

There are so many nice processors around that do not need that much

cycles for each intruction (6800, 65xx, 8510 etc., even the Z80 needs

less cycles...), why does your's do????

The Clarke processor is able to move large amounts of data quicker

through memory than comparable processors because of it's 64-bit

registers. It is also fast on doing simple floating-point operations

(I'll explore the decimal-mode later...). But it is very slow

on more complex tasks as array-accessing, fixed-point operations

and parameter-passing to subroutines (the little stack...).